3GPP TSG-RAN WG3 Meeting #125	   				         R3-244245

Maastricht, NL, 19th – 23th August 2024

Agenda Item:		15.2

Source:			China Telecom

Title:		Support for Store and Forward operation in IoT NTN

Document for:	 	Discussion & Decision

- Introduction

The Rel-19 IoT NTN enhancements WI contains the following objective[1]:

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore:

  - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2]

    - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

Note: Strive to minimise UE impact.

Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

In this contribution, we will provide some initial consideration from the perspective of RAN3 to support Store and Forward satellite operation.

- Discussion

Store and Forward as a capability of NTN, “provides a way to enable autonomously network service to UEs without the satellite always being connected to the ground station, which can extend the service availability for the areas without the connectivity to ground station via feeder link or ISL (e.g. at sea, very remote areas lack of ground-station infrastructures). Along with support for discontinuous coverage, support for S&F operation is also key to facilitate cost-effective deployment of IoT NTN services.

S&F satellite operation should be based on the regenerative payload architecture with full eNB on board. In addition, SA2 studied different solutions to support regenerative IoT NTN payload and store and forward operations, which can be categorized into the following options [2]:

- Option 1: eNB on board only.

- Option 2: eNB + Split MME on board

- Option 3: eNB + whole CN on board

For Option 1, the full eNB is on board satellite, all EPC functionalities are deployed on the ground, which is lightweight and easy to implement. The eNB needs to store UE UL data and send UE context, target RAN ID and UE location information to MME when feeder link is available. Simultaneously store DL data and perform paging when reaching the UE's location area.

For Option 2, the eNB and MME-onboard are onboard the satellite. Thus they can provide basic connection management procedures. The MME-onboard executes all the procedures with the UE which does not need interaction with other core network nodes on the ground when the service link is available. The eNB needs to support the transmission of necessary signaling for S&F operations between MME and UE. Besides, may initiate AS pause procedure if NAS is paused, and initiate AS recovery procedure.

For Option 3, whole CN including eNB, MME, SGW, PGW, HSS, E-SMLC, SMSC etc are on board each satellite. Since all CN components are onboard, the UE attach procedure can be directly finished as legacy TN or NTN. The eNB may needs to support the transmission of necessary signaling for S&F operations between MME and UE.

Proposal 1: Regarding how to support regenerative IoT NTN payload and store and forward operations, there’re some potential options that RAN3 needs to discuss:

- Option 1: eNB on board only.

- Option 2: eNB + Split MME on board

- Option 3: eNB + whole CN on board

The details and down-selection of these options need to be coordinated with SA2. At this stage, RAN3 can evaluate the potential impact of the above options and start with the baseline assumption that at least the eNB on board only.

Proposal 2: RAN3 should start with the regenerative payload with at least eNB on board only of S&F operation. Whether and which CN components are on board need to be coordinated with SA2.

Moreover, it is important that the S&F Satellite operation mode can properly support multi-satellite deployment scenarios by allowing a UE to be potentially served by multiple satellites, i.e. not restricting the S&F operation for a UE to a single satellite within the constellation. Multi-satellite support is indeed considered critical to be able to reduce the latencies in data transfer delivery, given that the revisit time of a single satellite may be around 12h or longer for typical orbits. Therefore, we suggest supporting both single satellite and multi-satellite deployment.

Proposal 3: It is proposed to support both single satellite and multi-satellite deployment in S&F mode.

- Conclusions

In this paper, we have the following proposal:

Proposal 1: Regarding how to support regenerative IoT NTN payload and store and forward operations, there’re some potential options that RAN3 needs to discuss:

- Option 1: eNB on board only.

- Option 2: eNB + Split MME on board

- Option 3: eNB + whole CN on board

Proposal 2: RAN3 should start with the regenerative payload with at least eNB on board only of S&F operation. Whether and which CN components are on board need to be coordinated with SA2.

Proposal 3: It is proposed to support both single satellite and multi-satellite deployment in S&F mode.

- References

[1]	RP-241624, Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek Inc.

[2]	TR 23.700-29, Study on integration of satellite components in the 5G architecture; Phase 3.