3GPP TSG-RAN WG3 Meeting #125	   				         R3-244246

Maastricht, NL, 19th – 23th August 2024

Agenda Item:		14.2

Source:			China Telecom

Title:		Further discussion on support MBS broadcast service for NTN

Document for:	 	Discussion & Decision

- Introduction

The following agreement has been made at the end of previous RAN3 meeting:

WA: The mapped cell ID can be used to define MBS broadcast service in NTN.

Other options can be further checked.

In this contribution, we will further discuss the potential enhancement to MBS broadcast service topic.

- Discussion

For how to enhance the delivery area of MBS broadcast services on NGAP, there are the following options:

- Mapped cell ID list in MBS service area

- New intended service area, e.g., smaller granularity for the MBS broadcast service area

- Based on GNSS description

For Option 1, considering that RAN3 has introduced the concept of Mapped cell IDs with smaller granularity in NTN to identify specific geographic regions and has been widely applied in NGAP messages. Similarly, Mapped Cell ID can be used for the MBS broadcast setup procedure. A specific geographical location may be mapped to multiple Mapped Cell ID(s), and such Mapped Cell IDs may be configured to indicate different geographical areas, which may be similar to the coverage of TN cells. This Option will not have any impact on Stage-3.

Proposal 1: Turn WA into agreement: The mapped cell ID can be used to define MBS broadcast service in NTN.

For Option 2 or 3, as the actual MBS broadcast service area in NTN scenarios may be polygonal or irregular shapes, RAN3 can discuss whether additional indicator information needs to be introduced to support finer grained area partitioning (e.g. GNSS description). However, some companies are concerned that options 2 and 3 require enhanced NGAP and CN interfaces, which may affect AF and CN nodes.

On the other hand, RAN2 is considering signaling design for air interface broadcast of geographic information, e.g. via polygons, to better approximate the intended shape of service area. Further investigation may be needed to determine whether gNB can convert cell ID into accurate geographic regions solely based on legacy Cell ID or Mapped Cell ID list.

In our view, Option 2 or 3 may help operators provide accurate E2E broadcasting services, The MBS service area information can be Cell ID list, TAI list, geographical area information or civic address information in CN. Introducing new service areas or GNSS information into RAN and CN interface protocols is not complicated. Since the NTN SI of SA2 did not involve MBS broadcasting services, this can be discussed by RAN3 and the conclusion can be notified to SA2 via LS. At this stage, we are not in a hurry to reach a conclusion and can wait for RAN2 progress to further consider other options.

Proposal 2: Geographical area information (e.g. GNSS description) can be introduced as the MBS service area information in NGAP.

Proposal 3: At this stage, it is proposed for RAN3 to wait RAN2 progress to further consider other options.

- Conclusions

In this paper, we have the following proposal:

Proposal 1: Turn WA into agreement: The mapped cell ID can be used to define MBS broadcast service in NTN.

Proposal 2: Geographical area information (e.g. GNSS description) can be introduced as the MBS service area information in NGAP.

Proposal 3: At this stage, it is proposed for RAN3 to wait RAN2 progress to further consider other options.

- References

[1]	RP-240775, Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT.

- Annex: Text Proposal for 38.300

16.14.5	NG-RAN signalling

The Cell Identity, as defined in TS 38.413 [26] and TS 38.423 [50], used in following cases corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported:

-	The Cell Identity indicated by the gNB to the Core Network as part of the User Location Information;

-	The Cell Identity used for Paging Optimization in NG interface;

-	The Cell Identity used for Area of Interest;

-	The Cell Identity used for PWS.

-	The Cell Identity used for MBS broadcast service.

The Cell Identity included within the target identification of the handover messages allows identifying the correct target cell. The cell identity used in the NG and Xn handover messages, Xn Setup and Xn NG-RAN Node Configuration Update procedures is expected to be Uu Cell ID.

The Cell Identities used in the RAN Paging Area during Xn RAN paging allow the identification of the correct target cells for RAN paging.

NOTE 1:	The Cell Identity used for RAN Paging is assumed to typically represent a Uu Cell ID.

The mapping between Mapped Cell IDs and geographical areas is configured in the RAN and Core Network.

NOTE 2:	A specific geographical location may be mapped to multiple Mapped Cell ID(s), and such Mapped Cell IDs may be configured to indicate differerent geographical areas (e.g. overlapping and/or with different dimensions).

The gNB is responsible for constructing the Mapped Cell ID based on the UE location information received from the UE, if available. The mapping may be pre-configured (e.g., up to operator's policy) or up to implementation.

NOTE 3:	As described in TS 23.501 [3], the User Location Information may enable the AMF to determine whether the UE is allowed to operate at its present location. Special Mapped Cell IDs or TACs may be used to indicate areas outside the serving PLMN's country.

The gNB reports the broadcasted TAC(s) of the selected PLMN to the AMF as part of ULI. In case the gNB knows the UE's location information, the gNB may determine the TAI the UE is currently located in and provide that TAI to the AMF as part of ULI.