3GPP TSG-RAN WG3 Meeting #125	   				         R3-244247

Maastricht, NL, 19th – 23th August 2024

Agenda Item:		14.3

Source:			China Telecom

Title:		Further discussion on support of regenerative payload

Document for:	 	Discussion & Decision

- Introduction

The Rel-19 NTN enhancements WI contains the following objective[1]:

Support of regenerative payload [RAN3, RAN2, RAN4]

- Specify the support of gNB on board in TS 38.300

- Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3]

Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it.

In this contribution, we will continue to discuss the potential impact of supporting NTN regeneration payloads on NGAP.

- Discussion

2.1 NG interface management

In the previous RAN3 meeting, we discussed the following options:

- Option1: NG removal/setup

- Option2: NG suspend/resume

- Option3: Legacy way

In general, all the above solutions are feasible. Although legacy way (SCTP stop) could also be used to remove or setup a new NG connection, but it is unclear how AMF infers the reason for SCTP disconnection, which may cause delay problems. It is recommended that RAN3 discuss possible signaling enhancements to better support full gNB onboard architecture.

We note that some companies argue that the NG connection with the old AMF may be suspended when the satellite leaves and be resumed when the satellite re-visit this area, so they tend to introduce a suspend/resume procedures to achieve fast recovery. However, there is no benefit for the NG suspend/resume after further consideration. First, it does not significantly save NG signaling overhead. Second, AMF is inefficient in managing a large number of NG interfaces, while only a very small number of NG interfaces are “active” at any timing duration. Due to the earth rotation, it may take several hours for the same NGSO satellite/gNB to return to the previous AMF service area. Maintaining these interfaces will result in large storage consumption of AMF and gNB.

Observation 1: NG suspend/resume procedure does not seem to be beneficial.

Based on above analysis, we slightly prefer to introduce a new NG removal procedure to realize the mobility management of on-board gNB and reduce the complexity of gNB and AMF.

Proposal 1: Introduce a NG Removal procedure for regenerative payload.

2.2 Feeder Link Switchover

In NGSO, the high-speed movement of satellites along a predetermined orbit can lead to frequent feeder link switchover. Since we assume that the satellite's movement speed is much faster than UEs, as long as the UE does not leave the current AMF service area, the UE is still served by the same AMF before and after the feeder link switch. The feeder link switchover without AMF change can be considered the main scenario in most cases.

According to the previous discussion in SA2, if the eNB/gNB IP address changes due to soft feeder link switch, it is assumed that the existing procedures can support this case, i.e. gNB can establish a new SCTP association with AMF by using the new TNL address anchored in new NTN-GW. However, in case of hard feeder link switch, a satellite/gNB can only be connected to one NTN-GW at the same time. If 5GC does not know the hard feeder link switch, the 5GC may continue sending the DL NG-C/U packet to the old NTN-GW. The old NTN-GW has no way to deliver it to the gNB or forward it to the new NTN-GW, which will result in packet loss. Although the existing retransmission mechanism can work, it is better not to cause interruption for some services requiring service continuity (such as voice services). It is recommended that gNB inform 5GC to buffer DL data based on actual situation of the UEs and feeder links, so the 5GC can suspend the DL NG-C/U.

Figure 1: Feeder link switchover without AMF change

- The gNB onboard the satellite connects to 5GC via NTN-GW1. In case of hard feeder link switchover, the gNB may informs AMF about the upcoming hard feeder link switchover.

2~4. The AMF informs the corresponding SMF(s) about the upcoming feeder link switchover receive from gNB. Based on feeder link switchover information, the SMF(s) controls the UPF(s) to buffer the DL data of the corresponding PDU sessions.

- The gNB onboard the satellite performs Feeder link switchover to NTN-GW2. After the Feeder link is switched, the gNB is assumed to get a new TNLA.

- The gNB connects to AMF via NTN-GW2 by using the new TNLA.

7~8. The gNB notifies the AMF about the new DL TNLA. The gNB can use existing procedures or define new procedures.

Proposal 2: It is proposed to study if any enhancement is required for hard feeder link switch scenarios to solve the packet loss problem.

2.3 TAI update

The Supported TAI List of onboard gNB is change over time for the NGSO. This requires pre-configuring mapping relationship among Global RAN Node ID, supporting TAI list and valid period. In the context of R17/R18, when RAN updates the TAI list, RAN will inform AMF of the updates. Even though there are frequent interactions between RAN and AMF, the cost of interactions is limited because the interactions are via ground link (e.g. fibre, cable, etc.). However, such interactions will be over feeder link in R19 and may be more frequent, whose bandwidth is scarce. OAM based solution is preferred to exchange the supported TAI list of the gNB.

Proposal 3: The exchange of Supported TAI List of onboard gNB can be based on OAM.

- Conclusions

In this paper, we have the following proposal:

Observation 1: NG suspend/resume procedure does not seem to be beneficial.

Proposal 1: Introduce a NG Removal procedure for regenerative payload.

Proposal 2: It is proposed to study if any enhancement is required for hard feeder link switch scenarios to solve the packet loss problem.

Proposal 3: The exchange of Supported TAI List of onboard gNB can be based on OAM.

- References

[1]	RP-241667, Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT.