3GPP TSG-RAN WG3 Meeting #125	   				         R3-244248

Maastricht, NL, 19th – 23th August 2024

Agenda Item:		16.2

Source:			China Telecom

Title:		(TP for TR 38.769) RAN architecture for Ambient-IoT

Document for:	 	Other

- Introduction

The last RAN3 meeting had some preliminary discussions on the RAN architecture of AIoT, including the preliminary definition of the device entities that provide AIOT services and the system architecture diagram of topology 1. However, there is still no consensus on the system architecture design of topology 2. In this contribution, we will continue to discuss RAN architecture aspects from the perspective of RAN3.

- Discussion

The definition of AIoT RAS (AIoT Radio Access System) is provided in the [1], and we understand that its purpose is to attempt to use a unified architecture to describe AIoT topology 1 and topology 2.

AIoT RAS (AIoT Radio Access System): hosts certain functions for AIoT as part of the functional split between RAN and CN.

For topology 1, the base station that provides AIoT services can be a lightweight gNB(AIoT-enabled gNB) that only provides very simplified functions including common Reader function; For topology 2, AIoT services are jointly provided by UE and base stations, where the base station can be an AIoT enhanced gNB that support both legacy UE and AIoT functions, and the common Reader function is located at the UE. Both AIoT-enabled gNB and UE can be considered as part of AIoT RAN. However, as common Reader function is located in different logical entities, we think that using only AIoT RAS to describe two topologies may not be appropriate, and it is best to use different logical system architecture figures.

In addition, according to the definition of AIoT RAS above, the standard for determining if an AIoT function is in RAS is based on whether it is either part of RAN or CN. In other words, the above definition means that AIoT RAS is equal to AIoT RAN. We would prefer to rename the “AIoT RAS” back to “AIoT RAN”.

The suggested topology architecture could be illustrated in the following figure:

Figure1: Logical System Architecture for AIoT topology 1.

Figure 2: Logical System Architecture for AIoT topology 2.

The above A-IoT architectures should be captured in the TR 38.769. The TP is provided in the annex.

Proposal 1: Capture the above logical system architecture of AIoT topology 1&2 in the TR 38.769.

In addition, there is still no consensus on whether the interface between AIoT RAN and AIoT CN should reuse NG or define a new XX interface in SA2. This may depend on whether a new AIoT NF is defined and whether AMF needs to be involved. It is suggested that RAN3 further discuss whether to reuse NG interface for A-IoT signaling interaction, and coordinate with SA2. We slightly prefer to reuse existing NG interface to support both topology 1 and topology 2, in order to minimize standard impacts. Whether to define a new interface between AIoT NF (similar to NRPPa) needs to wait the progress of SA2.

Proposal 2: It is proposed to enhance the existing NG interface to support both topology 1 and topology 2.

- Conclusions

In this paper, we have the following proposal:

Proposal 1: Capture the above logical system architecture of AIoT topology 1&2 in the TR 38.769.

Proposal 2: It is proposed to enhance the existing NG interface to support both topology 1 and topology 2.

- References

[1]	R3-243962, [TP for TR 38.769] CB:#AIoT1_Architecture (Ericsson, Huawei)

- Text Proposal for TR 38.769

<<<<<<<<<<<<<<<<<< Begin of the changes >>>>>>>>>>>>>>>>>>>

6.4 RAN architecture aspects

Editor’s Note 1: Corresponds to the second RAN3 objective in the SID, to identify RAN architecture aspects, including whether support for split architecture is necessary.

This chapter attempts to identify and describe architectural elements necessary to define a RAN architecture for support of Ambient IoT embedded in the overall 5G system architecture in support of topology 1 and topology 2 (as defined in TR 38.848 [2]).

Editor’s Note 2: What functionalities are hosted by the 5GS for AIoT is TBD.

This chapter also attempts to identify a functional split between RAN and CN.

Figure 6.4-1 depicts the logical system architecture for AIoT topology 1.

Figure 6.4-2 depicts the logical system architecture for AIoT topology 2.

It consists of the following architectural elements:

AIoT device: equipment with characteristics outlined e.g. in TS 22.369 [x] and TR 38.848 [2].

Editor’s Note 3: Further details FFS, if any.

AIoT RAN: hosts certain functions for AIoT as part of the functional split between RAN and CN.

Editor’s Note 4: Further details regarding AIoT functions hosted in the AIoT RAN and the respective functional split to be decided by RAN2, RAN3 and SA2.

AIoT radio: radio interface between AIoT device and Common reader function.

Editor’s Note 5: Further details on AIoT radio to be discussed by RAN1 and RAN2.

AIoT CN: hosts certain functions for AIoT as part of the functional split between RAN and CN

Editor’s Note 6: Further details regarding AIoT functions hosted in the AIoT CN and the respective functional split to be decided by RAN2, RAN3 and SA2.

XX interface: interface between the AIoT RAN and the AIoT CN on which certain AIoT specific functions are performed.

Editor’s Note 7: The functions represented by the XX interfaces are FFS. It is also FFS whether this interface represents a new logical interface or is equal to NG. E.g. for topology 1 it may only represent a single interface instance, e.g. a new interface between AIoT RAN and AIoT CN, for topology 2 it might represent either 2 interface instances, one instance for NG and one instance “XX” for a new interface between AIoT CN and AIoT RAN, or one instance for NG alone.

Common reader function: a function that communicates with the AIoT device by means of AIoT radio. In topology 2, such function is deployed in AIoT-enabled UE device, communicating via Uu interface to its serving gNB.

Editor’s Note 8: Further details on Common reader function is to be discussed by RAN1 and RAN2.

AIoT RAN node function: a function residing in AIoT RAN. It contains e.g. the control of the AIoT radio resources used towards the AIoT device.

Editor’s Note 9: further details are FFS. Note that “control of AIoT radio resources” does not necessarily imply dynamic configuration of resources but could also rely on static assignment of resources by means of OAM. Aspects concerning coordination of the Upper Layer functions (e.g. Inventory, Command) e.g. in case these functions have to be performed over a multitude of instances of the Common Reader Function are FFS.

Figure 6.4-1: Logical System Architecture for AIoT topology 1.

Figure 6.4-2: Logical System Architecture for AIoT topology 2.

<<<<<<<<<<<<<<<<<< End of the changes >>>>>>>>>>>>>>>>>>>