3GPP TSG-RAN WG3 Meeting #125	R3-244249

Maastricht, NL, 19th – 23th Aug, 2024

Title:	Discussion on AI/ML based Coverage and Capacity Optimization

Source:				China Telecom

Agenda item:		11.3

Document for:		Discussion and Decision

1. Introduction

In the previous meeting, we discussed the AI/ML based use case and potential solutions for Coverage and Capacity Optimization (CCO). The reached agreements are as follows:

RAN3#123bis: For AI/ML based CCO, - AI/ML Model Training may be located in the OAM and AI/ML Model Inference may be located in the NG-RAN node (gNB-CU). - AI/ML Model Training and AI/ML Model Inference may be both located in the NG-RAN node (gNB-CU). Solution for non-split architecture: Step 0: gNB predicts the CCO issue. Step 1: gNB generates the future coverage status based on the predicted CCO issue and other information. Step 2: gNB sends the future coverage status to neighbour gNBs. Solution for split-architecture: Step 0: gNB-CU predicts the CCO issue Step 1: gNB-CU sends the predicted CCO issue to gNB-DU. Step 2: gNB-DU generates the future coverage status based on the predicted CCO issue and other local information, whether only local information is used can be further discussed. Step 3: gNB-DU sends the future coverage status to gNB-CU. Step 4: gNB-CU sends the future coverage status to neighbour gNBs. RAN3#124: For non-split architecture, future CCO state can be derived via legacy mechanism or AI/ML techniques which is up to implementation. For split architecture, the standardization of AI/ML based future CCO state inference in the gNB-DU is not supported in R19, how to derive future CCO state is up to implementation in the gNB-DU, e.g., legacy or AI/ML.

But there is still some remaining issue. This document is to discuss the time information and root cause information for AI/ML based CCO.

2. Discussion

The objective of CCO function is to detect and resolve or mitigate CCO issues. As described in the previous meeting, AI/ML based CCO is a proactive approach used to prevent the rise of a CCO issue with the consequent degradation of network (and UE) performance. In AI/ML based CCO, gNB can predict the CCO issue. Based on the predicted CCO issue, gNB can generate the future coverage status for preventing the predicted CCO issue. Therefore, to resolve the CCO issues which are going to rise in future, following information needs to be considered:

-	when and why the CCO issue is going to occur and how to prevent the CCO issue.

-	when the future coverage status is going to be applied to prevent the predicted CCO issue.

Based on the analysis above, we have following observation and proposals:

Observation 1: The time information of predicted CCO issues, root cause of predicted CCO issues and time information of future coverage status are important for preventing the CCO issues which are going to occur in the future.

Proposal 1: When predicting CCO issue, timing information of CCO issue (i.e. when the predicted CCO issue is going to occur) and root cause of CCO issue also need to be predicted.

Proposal 2: When generating future coverage status, timing information of future coverage status (i.e. when the future coverage status is going to be applied) also needs to be generated, i.e., the output of AI/ML-based CCO should include future coverage status and related timing information.

Therefore, based on the agreements reached in the previous meetings and analysis above, we consider the following solutions:

-	for the non-split architecture,

-	Step 0: gNB predicts the CCO issue and related timing information (i.e. when the predicted CCO issue is going to occur) as well as root cause of predicted CCO issue.

-	Step 1: gNB generates the future coverage status and related timing information (i.e. when the future coverage status is going to be applied) based on the predicted CCO issue and related timing information, root cause of predicted CCO issue and other information.

-	Step 2: gNB sends the predicts the CCO issue and related timing information (i.e. when the predicted CCO issue is going to occur), root cause of predicted CCO issue, future coverage status and related timing information to neighbour gNBs.

-	For the split architecture, we consider the following solution:

-	Step 0: gNB-CU predicts the CCO issue and related timing information (i.e. when the predicted CCO issue is going to occur) as well as root cause of predicted CCO issue.

-	Step 1: gNB-CU sends the predicted CCO issue and related timing information (i.e. when the predicted CCO issue is going to occur) as well as root cause of predicted CCO issue to gNB-DU.

-	Step 2: gNB-DU generates the future coverage status and related timing information (i.e. when the future coverage status is going to be applied) based on the predicted CCO issue and related timing information, root cause of predicted CCO issue and other information.

-	Step 3: gNB-DU sends the future coverage status and related timing information to gNB-CU.

-	Step 4: gNB-CU sends the predicts the CCO issue and related timing information (i.e. when the predicted CCO issue is going to occur), root cause of predicted CCO issue, future coverage status and related timing information to neighbour gNBs.

Based on the solution for non-split architecture and split architecture, we have following proposals:

Proposal 3: The predicted CCO issues and related timing information (i.e. when the predicted CCO issue is going to occur), root cause of predicted CCO issue, as well as the future coverage status and related timing information (i.e. when the future coverage status is going to be applied) should be transmitted over Xn interface and F1 interface.

3. Conclusion

We have following observation and proposals:

Observation 1: The time information of predicted CCO issues, root cause of predicted CCO issues and time information of future coverage status are important for preventing the CCO issues which are going to occur in the future.

Proposal 1: When predicting CCO issue, timing information of CCO issue (i.e. when the predicted CCO issue is going to occur) and root cause of CCO issue also need to be predicted.

Proposal 2: When generating future coverage status, timing information of future coverage status (i.e. when the future coverage status is going to be applied) also needs to be generated, i.e., the output of AI/ML-based CCO should include future coverage status and related timing information.

Proposal 3: The predicted CCO issues and related timing information (i.e. when the predicted CCO issue is going to occur), root cause of predicted CCO issue, as well as the future coverage status and related timing information (i.e. when the future coverage status is going to be applied) should be transmitted over Xn interface and F1 interface.