3GPP TSG-RAN WG3 Meeting #125	R3-244250

Maastricht, NL, 19th – 23th Aug, 2024

Title:	(TP to 38.743) Support of AI/ML based Coverage and Capacity Optimization

Source:				China Telecom

Agenda item:		11.3

Document for:		Agreement

1. Introduction

Based on our discussion paper R3-244249, we provide this TP to support AI/ML based Coverage and Capacity Optimization.

2. TP for TR 38.743

START OF CHANGE

## 4.2	AI/ML based Coverage and Capacity Optimization

### 4.2.1	Use case description

Editor Note: Capture the description of use case

The objective of NR Coverage and Capacity Optimization (CCO) function is to detect and resolve or mitigate CCO issues. An NG-RAN node may autonomously adjust within and switch among coverage configurations. When a change is executed, a NG-RAN node may notify its neighbour NG-RAN nodes with the list of cells and SSBs with modified coverage included.

In the legacy CCO solution, a reactive approach is used: when the gNB (gNB-CU in case of CU-DU split architecture) detects a CCO issue which negatively impacts network and UE performance after it has already occurred, the gNB (gNB-DU in case of CU-DU split architecture) attempts to resolve or mitigate it.

With an AI/ML based CCO, a more proactive approach is used to prevent (or limiting at an early stage) the rise of a CCO issue with the consequent degradation of network (and UE) performance.

### 4.2.2	Solutions and standard impacts

Editor Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

#### 4.2.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions can be considered for supporting AI/ML-based CCO:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

4.2.2.a	AI/ML Model Training at OAM and AI/ML Model Inference at NG-RAN

In this solution, NG-RAN makes coverage and capacity optimization decisions using AI/ML model from OAM.

Figure 4.2.2.a-1. Model Training at OAM, Model Inference at NG-RAN

Step 0: NG-RAN node 2 is assumed to have an AI/ML model optionally, which can provide NG-RAN node 1 with input information.

Step 1: NG-RAN node 1 configures the measurement information on the UE side and sends configuration message to UE to perform measurement procedure and reporting.

Step 2: The UE collects the indicated measurement(s), e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3: The UE sends the measurement report message(s) to NG-RAN node 1.

Step 4: NG-RAN node 1 further sends UE measurement reports together with other input data for Model Training to OAM.

Step 5: NG-RAN node 2 (assumed to have an AI/ML model optionally) also sends input data for Model Training to OAM.

Step 6: Model Training at OAM. Required measurements and input data from other NG-RAN nodes are leveraged to train AI/ML models for coverage and capacity optimization.

Step 7: OAM deploys/updates AI/ML model into the NG-RAN node(s). The NG-RAN node can also continue model training based on the received AI/ML model from OAM.

Step 8: NG-RAN node 2 sends the required input data to NG-RAN node 1 for model inference of AI/ML-based coverage and capacity optimization.

Step 9: UE sends the UE measurement report(s) to NG-RAN node 1.

Step 10: Based on local inputs of NG-RAN node 1 and received inputs from NG-RAN node 2, NG-RAN node 1 generates model inference output(s) (e.g., predicted CCO issues, future coverage status etc).

Step 11: NG-RAN node 1 sends Model Performance Feedback to OAM if applicable.

Step 12: NG-RAN node 1 executes coverage and capacity optimization actions according to the model inference output.

Step 13: NG-RAN node 2 provides feedback to OAM.

Step 14: NG-RAN node 1 provides feedback to OAM.

4.2.2.b	AI/ML Model Training and AI/ML Model Inference at NG-RAN

In this solution, NG-RAN is responsible for model training and making coverage and capacity optimization decisions.

Figure 4.2.2.b-1. Model Training and Model Inference at NG-RAN

Step 0: NG-RAN node 2 is assumed to have an AI/ML model optionally, which can provide NG-RAN node 1 with input information.

Step 1: NG-RAN node 1 configures the measurement information on the UE side and sends configuration message to UE to perform measurement procedure and reporting.

Step 2: The UE collects the indicated measurement(s), e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3: The UE sends the measurement report(s) to NG-RAN node 1 including the required measurement result.

Step 4: NG-RAN node 2 sends the required input data to NG-RAN node 1 for model training of AI/ML-based coverage and capacity optimization.

Step 5: NG-RAN node 1 trains AI/ML model for AI/ML-based coverage and capacity optimization based on collected data. NG-RAN node 2 is assumed to have AI/ML model for AI/ML-based coverage and capacity optimization optionally, which can also generate predicted results.

Step 6: NG-RAN node 2 sends the required input data to NG-RAN node 1 for model inference of AI/ML-based coverage and capacity optimization.

Step 7: UE sends the UE measurement report(s) to NG-RAN node 1.

Step 8: Based on local inputs of NG-RAN node 1 and received inputs from NG-RAN node 2, NG-RAN node 1 generates model inference output (e.g., predicted CCO issues, future coverage status, etc).

Step 9: NG-RAN node 1 executes coverage and capacity optimization actions according to the model inference output.

Step 10: NG-RAN node 2 provides feedback to NG-RAN node 1.

4.2.2.c	AI/ML Model Training at OAM and AI/ML Model Inference at CU of NG-RAN

In this solution, NG-RAN CU makes coverage and capacity optimization decisions using AI/ML model from OAM.

Figure 4.2.2.c-1. Model Training at OAM, Model Inference at CU of NG-RAN

Step 0: NG-RAN node 2 CU is assumed to have an AI/ML model optionally, which can provide NG-RAN node 1 CU with input information.

Step 1: NG-RAN node 1 DU configures the measurement information on the UE side and sends configuration message to UE to perform measurement procedure and reporting.

Step 2: The UE collects the indicated measurement(s), e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3: The UE sends the measurement report message(s) to NG-RAN node 1 DU.

Step 4: The NG-RAN node 1 DU sends the measurement report message(s) to NG-RAN node 1 CU.

Stpe 5: NG-RAN node 1 CU further sends UE measurement reports together with other input data for Model Training to OAM.

Step 6: NG-RAN node 2 CU (assumed to have an AI/ML model optionally) also sends input data for Model Training to OAM.

Step 7: Model Training at OAM. Required measurements and input data from other NG-RAN nodes are leveraged to train AI/ML models for coverage and capacity optimization.

Step 8: OAM deploys/updates AI/ML model into the NG-RAN CU. The NG-RAN CU can also continue model training based on the received AI/ML model from OAM.

Step 9: NG-RAN node 2 CU sends the required input data to NG-RAN node 1 CU for model inference of AI/ML-based coverage and capacity optimization.

Step 10: UE sends the UE measurement report(s) to NG-RAN node 1 DU.

Step 11: The NG-RAN node 1 DU sends the measurement report message(s) to NG-RAN node 1 CU.

Step 12: Based on local inputs of NG-RAN node 1 and received inputs from NG-RAN node 2, NG-RAN node 1 CU generates model inference output(s) (e.g., future coverage status etc).

Step 13: NG-RAN node 1 CU sends model inference output to NG-RAN node 1 DU.

Step 14: NG-RAN node 1 CU sends Model Performance Feedback to OAM if applicable.

Step 15: NG-RAN node 1 DU executes coverage and capacity optimization actions according to the model inference output.

Step 16: NG-RAN node 2 CU provides feedback to OAM.

Step 17: NG-RAN node 1 CU provides feedback to OAM.

4.2.2.d	AI/ML Model Training and AI/ML Model Inference at CU of NG-RAN

In this solution, NG-RAN CU is responsible for model training and making coverage and capacity optimization decisions.

Figure 4.2.2.d-1. Model Training and Model Inference at CU of NG-RAN

Step 0: NG-RAN node 2 CU is assumed to have an AI/ML model optionally, which can provide NG-RAN node 1 with input information.

Step 1: NG-RAN node 1 DU configures the measurement information on the UE side and sends configuration message to UE to perform measurement procedure and reporting.

Step 2: The UE collects the indicated measurement(s), e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3: The UE sends the measurement report(s) to NG-RAN node 1 DU including the required measurement result.

Step 4: NG-RAN node 1 DU sends the measurement report(s) to NG-RAN node 1 CU including the required measurement result.

Step 5: NG-RAN node 2 CU sends the required input data to NG-RAN node 1 CU for model training of AI/ML-based coverage and capacity optimization.

Step 6: NG-RAN node 1 CU trains AI/ML model for AI/ML-based coverage and capacity optimization based on collected data. NG-RAN node 2 CU is assumed to have AI/ML model for AI/ML-based coverage and capacity optimization optionally, which can also generate predicted results.

Step 7: NG-RAN node 2 CU sends the required input data to NG-RAN node 1 CU for model inference of AI/ML-based coverage and capacity optimization.

Step 8: UE sends the UE measurement report(s) to NG-RAN node 1 DU.

Step 9: The NG-RAN node 1 DU sends the measurement report message(s) to NG-RAN node 1 CU.

Step 10: Based on local inputs of NG-RAN node 1 and received inputs from NG-RAN node 2, NG-RAN node 1 CU generates model inference output (e.g., predicted CCO issues, future coverage status, etc).

Step 11: NG-RAN node 1 CU sends model inference output to NG-RAN node 1 DU.

Step 12: NG-RAN node 1 DU executes coverage and capacity optimization actions according to the model inference output.

Step 13: NG-RAN node 2 provides feedback to NG-RAN node 1.

4.2.2.e	Input of AI/ML-based Coverage and Capacity Optimization

To predict the optimized network coverage and capacity decisions, NG-RAN may need following information as input data for AI/ML-based coverage and capacity optimization:

From local NG-RAN node:

-	UE mobility/trajectory prediction

-	Current/Predicted CCO issues

-	Current/Future Coverage status

-	Timing information of CCO issues

-	Rout Cause of CCO issues

From the UE:

-	UE location information (e.g., coordinates, serving cell ID, moving velocity)

-	UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc)

From neighbouring NG-RAN nodes:

-	Current/Predicted CCO issues

-	Current/Future Coverage status

-	Timing information of CCO issues

-	Rout Cause of CCO issues

4.2.2.f	Output of AI/ML-based Coverage and Capacity Optimization

AI/ML-based coverage and capacity optimization model can generate following information as output:

-	Predicted CCO issues, root cause and timing information

-	Future Coverage status and timing information

-	Predicted CCO issues of neighboring NG-RAN node(s), root cause and timing information

-	Future Coverage status of neighboring NG-RAN node(s) and timing information

4.2.2.g	Feedback of AI/ML-based Coverage and Capacity Optimization

To optimize the performance of AI/ML-based coverage and capacity optimization model, following feedback can be considered to be collected from NG-RAN nodes:

-	Coverage and capacity status of neighbouring NG-RAN nodes

-	UE performance affected by the coverage and capacity optimization action, including access success rate, access average delay, cell edge call drop rate.

4.2.2.h	Standard Impact

MDT procedure enhancements should be discussed during the normative phase.

Potential Xn interface impact:

-	New signaling procedure or enhanced existing procedure to collect the input data information.

-	Current/Predicted CCO issues as well as related root cause and timing information between neighbouring NG-RAN nodes and source NG-RAN node.

-	Current/Future Coverage status as well as related timing information between neighbouring NG-RAN nodes and source NG-RAN node.

-	New signalling procedure or enhanced existing procedure to retrieve feedback information.

Potential F1 interface impact:

-	New signaling procedure or enhanced existing procedure to exchange information between gNB-CU and gNB-DU

-	Current/Predicted CCO issues as well as related root cause and timing information between gNB-CU and gNB-DU.

-	Current/Future Coverage status as well as related timing information between gNB-CU and gNB-DU.

END OF CHANGE