3GPP TSG-RAN WG3 Meeting #125	R3-244251

Maastricht, NL, 19th – 23th Aug, 2024

Title:	Discussion on Continuous MDT measurement

Source:				China Telecom

Agenda item:		11.4

Document for:		Discussion and Decision

1. Introduction

In the previous meeting, we discussed the scenario of continuous MDT collection targeting the same UE across RRC states. In management-based MDT, a UE in the NG-RAN can be configured with management-based Logged MDT when in RRC_Idle and RRC_Inactive states and with management-based Immediate MDT when in RRC_Connected state. UEs in management-based MDT are not uniquely identified. Therefore, when a UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive or when a UE is handed over between gNBs, the network does not have standardized means to select again the same UE for continuous MDT for subsequent MDT data collection.

In this scenario, data collection continuity can be split into two tasks as below:

-	Problem A (measurement continuity): how to ensure that the same UE collecting MDT measurements during the same RRC state and across different RRC states.

-	Problem B (trace correlation): how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE.

This document is to discuss potential solutions for problem A (measurement continuity).

2. Discussion

To solve the problem A (measurement continuity), it is needed to select again the same UE when the UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive or when the UE is handed over between gNBs. In management-based MDT, UEs are not uniquely identified. Therefore, the continuous MDT measurement indication parameter may need to be considered. Based on the indication information, gNB knows that this UE should be selected for continuous MDT measurement. The questions about the indication information are as follows:

-	Q1: how to generate the continuous MDT measurement indication parameter.

-	Q2: where to store the continuous MDT measurement indication parameter, and how to get the continuous MDT measurement indication parameter when the UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive or when the UE is handed over between gNBs.

For Q1, we propose that the continuous MDT measurement indication parameter can be the MDT configuration parameter provided by OAM in the MDT activation message. For Q2, the following solutions can be considered:

-	Solution A1: The continuous MDT measurement indication parameter is stored at UE. When the UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive and/or when the UE is handed over between gNBs, the UE indicates to the gNB that it should be selected for continuous management-based immediate MDT.

-	Solution A2: The continuous MDT measurement indication parameter is stored at AMF. When the UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive, the AMF indicates to the gNB that the UE should be selected for continuous management-based immediate MDT. When the UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive and the UE is handed over between gNBs, the AMF indicates to the gNB that the UE should be selected for continuous management-based immediate MDT

-	Solution A3: The continuous MDT measurement indication parameter is stored at gNB. When the UE is handed from source gNB to target gNB, the source gNB indicates to the target gNB that the UE should be selected for continuous management-based immediate MDT.

The solution A1 is a UE-based solution with impacts to the UE. The solution A2 and A3 are network-based solution with impacts to AMF and gNB respectively. If we adopt UE-based solution, the legacy UE will not support the continuous MDT feature. So we propose to consider the network-based solutions (i.e, A2 and A3) that have no impact to UE.  The Figure 1 and Figure 2 are the examples of continuous management-based MDT based on solution A2. The Figure 3 is the example of continuous management-based MDT based on solution A3.

Figure 1 The UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive

Figure 2 The UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive when the UE is handed to gNB2 from gNB1

Figure 3 The UE is handed to gNB2 from gNB1

Based on the discussion above, we have following proposals:

Proposal 1: The MDT configuration parameter provided by OAM should include the continuous MDT measurement indication parameter.

Proposal 2: The solution for continuous MDT measurement should not have impacts to UE.

Proposal 3: We propose to consider the solution A2 and solution A3 for continuous MDT measurement.

Apart from the continuous MDT measurement indication parameter, the continuous MDT measurement duration is also important. When the UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive and/or when the UE is handed over between gNBs, the gNB should know whether the UE is configured the continuous MDT measurement indication parameter and whether the validity of continuous MDT measurement indication parameter is expired. If the validity of continuous MDT measurement indication parameter is expired, the gNB will not select the UE for continuous management-based immediate MDT even though the UE is configured the continuous MDT measurement indication parameter. Therefore, we have following proposal:

Proposal 4: The MDT configuration parameter provided by OAM should include the continuous MDT measurement duration. It defines the validity of continuous MDT measurement indication parameter.

3. Conclusion

We have following proposals:

Proposal 1: The MDT configuration parameter provided by OAM should include the continuous MDT measurement indication parameter.

Proposal 2: The solution for continuous MDT measurement should not have impacts to UE.

Proposal 3: We propose to consider the solution A2 and solution A3 for continuous MDT measurement.

Proposal 4: The MDT configuration parameter provided by OAM should include the continuous MDT measurement duration. It defines the validity of continuous MDT measurement indication parameter.