3GPP TSG-RAN WG3 Meeting #125	R3-244252

Maastricht, NL, 19th – 23th Aug, 2024

Title:	(TP to 38.743) Support of Continuous MDT measurement

Source:				China Telecom

Agenda item:		11.4

Document for:		Agreement

1. Introduction

Based on our discussion paper R3-244251, we provide this TP to support continuous MDT measurement.

2. TP for TR 38.743

START OF CHANGE

## 5.4	Continuous MDT collection targeting the same UE across RRC states

Editor Note: Capture the description and its potential standard impacts.

### 5.4.1	Use Case Description

The problem of continuous data collection for management-based MDT can be described as follows: a UE in the NG-RAN can be configured with management-based Logged MDT when in RRC_Idle and RRC_Inactive states and with management-based Immediate MDT when in RRC_Connected state. Differently from signalling-based MDT, in management-based MDT, a UE is not uniquely identified in the MDT activation. Therefore, when a UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive (during which Logged MDT data have been collected) or when a UE is handed over between gNBs, the network does not have standardized means to select again the same UE for continuous MDT for subsequent MDT data collection.

The Data Collection continuity in this scenario can be split into two tasks as below:

-	Problem A (measurement continuity): how to ensure that the same UE collecting MDT measurements during the same RRC state and across different RRC states.

-	Problem B (trace correlation): how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE.

### 5.4.2	Solutions and standard impacts

Solution for measurement continuity:

- When the UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive, the gNB initiate management-based immediate MDT according to the MDT configuration stored in the core network to ensure the ensures MDT measurement continuity.

- When the UE is handed to target gNB from source gNB, the target gNB initiate management-based immediate MDT according to the MDT configuration stored in the source gNB to ensure the ensures MDT measurement continuity.

- When the UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive and the UE is handed to target gNB from source gNB, the target gNB initiate management-based immediate MDT according to the MDT configuration stored in the core network to ensure the ensures MDT measurement continuity.

END OF CHANGE