3GPP TSG RAN WG3#124	R3-244258

Maastricht, NL, 19 – 23 Aug, 2024

Agenda item:	12.3

Source: 	China Telecom

Title: 	(TP to TR 38.799) On Access Control and Mobility for NR Femto

Document for:	Discussion and decision

- Introduction

After several meeting discussing on 5G Femto support, the consensus on 5G Femto Architecture including NG and Xn interfaces was reached, and the descriptions have been captured in TR38.799[1]. There are two remaining issues i.e. the access control for NR Femto and the access to local services from 5G Femto that need to be discussed and drawn some conclusions which could be captured in TR 38.799.

In this paper, we discuss the mobility mechanism between CAG cell of NR Femto and CSG cell.

- Discussion on UE mobility between CAG cell of NR Femto and CSG cell of HeNB

In LTE Femto, Closed Subscriber Group (CSG) identifies a group of subscribers who are permitted/allowed to access one or more CSG cells of the PLMN identified by CSG IDs. A CSG cell is a cell broadcasting one or several CSG IDs. If a UE tries to connect to a CSG cell the MME checks whether the UE is allowed to do so considering the subscription data.

In 5G, a Closed Access Group (CAG) identifies a group of subscribers who are permitted/allowed to access one or more CAG cells associated to the CAG ID. A CAG cell is a cell broadcasting one or several CAG IDs. The CAG concept is like the CSG concept in EPS but was originally introduced in the context of PNI-NPNs to prevent UEs, which are not allowed to access the NPN. The CAG based access control was introduced in 3GPP Release-16. Now the current PNI-NPN mechanism has been agreed to reuse for the initial access control for a NR Femto node.

At last RAN3 meeting, SA2 sent an LS to RAN3 to raise the mobility issue between CAG cell of 5G Femto and 4G CSG cell. Some companies thought the mobility scenarios would be valid as the deployment of 5G Femto may start on top of an existing LTE Femto deployment such as in an Enterprise deployment.

There are two solutions proposed in SA2

- Solution1: The UE partitions CSG-CAG ID and constructs mapped CSG/CAG ID, and reports to the NG-RAN or E-UTRAN (depending on the considered mobility direction) as described in pCR (S2-2405814).

- Solution2: RAN recognizes the target CSG cell (or the target CAG cell) as an open cell during the handover (e.g., via local configuration) and the core network performs access control as described in pCR (S2-2405789).

For Solution1, when UEs move from 4G CSG cell to 5G CAG cell, UE needs to read the CAG ID of the target CAG cell of NR Femto and constructs a mapped CSG ID and send it within 4G measurement report message to the source 4GS. UE also needs to read the CSG ID of the target CSG cell and constructs a mapped CAG ID and sends to the source 5GS a 5G measurement report message including the mapped CAG ID during mobility from CAG cell of NR Femto to 4G CSG cell. It is obvious that he solution 1 has UE impact.

For soluition2, the access control is conducted by AMF/MME. there is no RAN impact since

As the SID states, 'It is expected that there should be no impact to UEs at this late stage of 5G deployment.' Therefore, Solution 2 is more preferable.

Observation: Solution 2 is no RAN impact as the access control of solution 2 is conducted by AMF/MME.

Proposal 1: Solution 2 should be captured in the section related to UE mobility between CAG cells of NR Femtos and CSG cells of HeNBs in TR38.799.

Proposal 2: Discuss, revise and if agreeable capture the TP for TR 38.799.

- Conclusion

Based on the above discussion on access control, we have the following observation and proposals:

Observation: Solution 2 is no RAN impact as the access control of solution 2 is conducted by AMF/MME.

Proposal 1: Solution 2 should be captured in the section related to UE mobility between CAG cells of NR Femtos and CSG cells of HeNBs in TR38.799.

Proposal 2: Discuss, revise and if agreeable capture the TP for TR 38.799.

- Reference

- TR 38.799, Study on additional topological enhancements for NR (Release 19), 3GPP.

- TS 23.501, System architecture for the 5G System (5GS); Stage 2 (Release 18), 3GPP.

- TS 23.548, 5G System Enhancements for Edge Computing; Stage 2 (Release 18), 3GPP.

- S2-2405814,KI#1 Solution for handover from/to CSG cell to/from CAG cell using mapped CAG/CSG ID

- S2-2405789, KI#1, New solution–Management based CAG and CSG correlation to support UE move.

- Text Proposal for TR 38.799

<<<<<<<<<<<<<<<<<<<< START OF CHANGES >>>>>>>>>>>>>>>>>>>>

5.3	Access control

Editor Note: Study how to define the 5G access control mechanism by (re-)using the existing CAG functionality and identify needed enhancements (if any)

The CAG access control and mobility mechanisms specified for PNI-NPN are reused for NR Femto nodes.

Three types of cells served by NR Femto node for UE access control are defined as below:

- Closed cell: a CAG Cell broadcasting one or multiple CAG Identifiers per PLMN and a CAG-only indication.

- Hybrid cell: a CAG Cell broadcasting one or multiple CAG Identifiers per PLMN.

- Open cell: a cell operated as a normal cell which does not broadcast any NPN Identifiers.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

5.X	UE Mobility between CAG cells and CSG cells

For support of UE mobility between CAG cells of NR Femto and CSG cells of HeNB, the correlation between CAG information and CSG information (including corresponding Home eNB ID) should be configured locally in the AMF that the 5G NR Femto cell is connected to.

During the UE HO procedure from a CAG cell of 5GS to a CSG cell of EPS, the AMF provides to the MME in the Forward Relocation Request message with CSG related information, based on CAG information in subscription data and the configured correlation between the Target ID contained in N2 Handover Required signalling (i.e. the Global Home eNB ID) and the corresponding CSG information.

During the UE HO procedure from a CSG cell of EPS to a CAG cell of 5GS, the AMF performs normal HO procedure based on existing access control function.

The detailed HO procedures between CAG cells and CSG cells are described in TS 23.501[xx]

<<<<<<<<<<<<<<<<<<<< END OF CHANGES >>>>>>>>>>>>>>>>>>>>