3GPP TSG RAN WG3#124	R3-244259

Maastricht, NL, 19 – 23 Aug, 2024

Agenda item:	12.3

Source: 	China Telecom

Title: 	(TP to TR 38.799) On local service access for NR Femto

Document for:	Discussion and decision

- Introduction

After several meeting discussing 5G Femto support, the consensus on 5G Femto Architectures including NG and Xn interfaces was reached, and the descriptions have been captured in TR38.799[1]. There are two remaining issues i.e. the access control for NR Femto and the access to local services from 5G Femto that need to be discussed and drawn some conclusions which should be captured in TR 38.799.

In this paper, we discuss the local service access for NR Femto.

- Discussion on local service access

In TS 23.501[2], the local service access mechanism is described in the section 5.13 ‘Support for Edge Computing’. The three local service access models -- Distributed Anchor Point, Session Breakout and Multiple PDU Sessions -- are specified to enable Edge Computing in TS 23.548[3], as follows:

- For Distributed Anchor Point mode, UE can have a PDU Session with PSA UPF in a local site, i.e. close to the UE location.

- For Session Breakout, UE can have a PDU Session with a PSA UPF in a central site (C-PSA UPF) and one or more PSA UPF in the local site (L-PSA UPF). The application traffic is selectively diverted to the L-PSA UPF using UL Classifier or multi-homing Branching Point mechanisms.

- For Multiple PDU Sessions: UE can have both some PDU Sessions with PSA UPFs in local sites and some PDU Sessions with PSA UPFs in the central sites.

Three above modes are illustrated in Figure 1.

For local service access from UE side, the UE receives DNS settings during PDU Session Establishment, then the UE may do a discovery to get the IP address of a suitable Edge Application Server (e.g. the closest one), so that the traffic with the IP address as the destination IP can be locally routed to the Edge Application Server.

Figure 1. 5GC Connectivity Models for Edge Computing

For local service access from the network side, the SMF determines the DNS server address for the PDU Session based on local configuration and EAS Deployment Information and provides its information to UE. The UE can get the IP address of a suitable Edge Application Server (e.g. the closest one) using the DNS. The SMF selects a UPF close to the UE and indicates the gNB which is serving the UE to establish a PDU session with this local UPF, then UPF forwards the user traffic to the DN via a N6 interface according to the provided traffic steering rules to the UPF.

Figure 2. 5GC providing local service access for NG-RAN

Proposal 1: Existing NR system supports the local distributed UPF deployment (physically located close to or at RAN), there is no need to introduce further enhancements for local service access.

- How to select the local UPF to be collocated with the NR Femto?

It is assumed that a large number of NR Femto will be deployed in the network with each NR Femto providing 5G coverage in a limited area. Therefore, the local UPFs cannot be collocated one by one with each NR Femto.

To select the appropriate UPF that is closest to the NR Femto serving the UE, SMF must know the location of each NR Femto and the local UPF.

In current NR system, NG-RAN supports its location reporting for the services that require accurate cell identification (e.g. emergency services, lawful intercept, charging) or for the UE mobility event notification service subscribed to the AMF by other NFs. The AMF may request the NG-RAN location reporting with event reporting type (e.g. UE location or UE presence in Area of Interest), reporting mode and its related parameters (e.g. number of reporting).

For 5G femto, the similar mechanism for Femto location reporting can be used. After AMF knows the location of NR Femto, the SMF can select the nearest suitable UPF to the femto node for the UE’s local service.

Figure 3. 5GC providing local service access for NR Femto

Proposal 2: Reusing the existing NG-RAN reporting mechanism for NR Femto location reporting, 5GC can select an appropriate local UPF for local service access according to NR Femto location.

Proposal 3: Discuss, revise and if agreeable capture the TP for TR 38.799.

- Conclusion

Based on the above discussion on local service access, we have the following proposals:

Proposal 1: Existing NR system supports the local distributed UPF deployment (physically located close to or at RAN), there is no need to introduce further enhancements for local service access.

Proposal 2: Reusing the existing NG-RAN reporting mechanism for NR Femto location reporting, 5GC can select an appropriate local UPF for the local service access according to NR Femto location.

Proposal 3: Discuss, revise and if agreeable capture the TP for TR 38.799.

- Reference

- TR 38.799, Study on additional topological enhancements for NR (Release 19), 3GPP.

- TS 23.501, System architecture for the 5G System (5GS); Stage 2 (Release 18), 3GPP.

- TS 23.548, 5G System Enhancements for Edge Computing; Stage 2 (Release 18), 3GPP.

- Text Proposal for TR 38.799

<<<<<<<<<<<<<<<<<<<< START OF CHANGES >>>>>>>>>>>>>>>>>>>>

References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP RP-240319, " Study on additional topological enhancements for NR".

[xx]	3GPP TS 23.501, "System Architecture for the 5G System; Stage 2".

[yy]	3GPP TS 23.501: "5G System Enhancements for Edge Computing; Stage 2".

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

5.4	Local services access

Editor Note: Clarify the access to local services from the 5G Femto via collocated local UPF and identify issues, if any

The access to local services from NG-RAN is supported with the functions outlined in section 5.6.2, 5.6.5, 5.6.7, 5.13, and 6.3.3 of TS 23.501 [xx], and further described in TS 23.548 [yy]. The existing mechanism for accessing local services can be reused for NR Femto.

The architecture of 5GS providing local services access for NR Femto is shown in Fig. xx.

Figure xx. 5GS providing local service access for NR Femto

When a UE initiates an application service request via a NR Femto, the SMF identifies an Edge Application Servers (EAS) in local Data Network (DN) that can provide the requested service. The SMF then selects a local UPF with a local breakout interface (N6) to the local DN that is closest to the NR Femto serving the UE, based on NR Femto reporting location. The SMF indicates the NR Femto to establish a PDU session with the selected local UPF. Once this session is established, the UE can access the local service from the EAS via the local UPF. Detailed information on accessing local services from NR Femto is specified in TS 23.501[xx].

<<<<<<<<<<<<<<<<<<<< END OF CHANGES >>>>>>>>>>>>>>>>>>>>