3GPP TSG-RAN WG3 #125                                                 R3-244264

Maastricht, NL, 19th – 23rd Aug, 2024

Agenda Item:	10.2

Source:	CATT

Title:	Discussion on MRO for R18 mobility mechanisms.

Document for:	Discussion

Introduction

In last RAN3 meeting, the following agreements are captured:

MRO for LTM:

RAN3 to prioritize MCG LTM over SCG LTM.

RAN3 takes RAN2#125-bis agreement on MRO for LTM scenarios as baseline for further study.

For failures due to wrong selection of candidate LTM cell, CU is in charge of root cause analysis and performs optimization.

For failures due to inappropriate cell switch triggering (e.g. wrong cell selection at cell switch, wrong cell switch timing, …) source DU performs optimization.

RAN3 to discuss enhancements related to the information of RACH-less access and RACH-based access from UE related to MRO for LTM.

MRO for CHO with Candidate SCG(s):

Do not distinguish between fast MCG recovery/no-fast MCG recovery for now (to simplify use cases).

Concurrent error cases (MCG+SCG) is FFS. The definition of these error cases needs to be further clarified.

RAN3 will start with the failure scenarios with UEs configured with CHO with candidate SCGs.

Whether to include failure and near failure scenarios related with configuration of CHO with candidate SCG(s) and CHO only is FFS.

MRO for S-CPAC:

CPAC failure scenarios and detection mechanism captured in stage2 used as baseline.

Based on above agreements, here we provide some analysis on MRO for R18 mobility mechanisms.

Discussion

- 2.1 MRO for LTM

LTM failure case and stage2 description

In last RAN3 meeting, RAN3 make the following agreement on LTM scenarios.

RAN3 takes RAN2#125-bis agreement on MRO for LTM scenarios as baseline for further study.

RAN2 agreed LTM scenarios are listed below:

For LTM MRO, RAN2 considers the following three connection failure cases:

- -	Too late LTM

- -	Too early LTM

- -	LTM to wrong cell

For too late LTM, the following sub-cases are considered but we may down prioritize later (not limiting):

- -	Case 1a: the UE detects RLF in source cell after receiving LTM candidate configurations and performs reestablishment procedure.

- -	Case 1b: the UE detects RLF in source cell after receiving LTM candidate configurations, selects an LTM candidate cell, detects HOF with the selected LTM cell.

- -	Case 1c: the UE detects RLF in source cell after receiving LTM candidate configurations, and successfully completes LTM execution with the selected LTM candidate cell.

For too early LTM, the following sub-cases are considered but we may down prioritize later (not limiting):

- -	Case 2a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure with the source cell.

- -	Case 2b: the UE detects HOF/RLF in the LTM target cell, selects the source cell which is also an LTM candidate cell, detects HOF with the source cell, and performs reestablishment procedure.

- -	Case 2c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected source cell which is also an LTM candidate cell.

LTM to wrong cell, the following sub-cases are considered but we may down prioritize later (not limiting):

- -	Case 3a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure in a cell other than the source cell and the target cell.

- -	Case 3b: the UE detects HOF/RLF in the LTM target cell, selects an LTM candidate cell which is different from the source or target one, detects HOF with the selected LTM candidate cell, and performs reestablishment procedure.

- -	Case 3c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected LTM candidate cell which is different from the source or target one.

Since LTM is based on CHO, CHO failure case which is captured in [1] in R17 may be reviewed first in order to compare the difference between CHO and LTM and decide whether MRO for CHO could be reuse for LTM if possible.

The following table illustrates the relationship between CHO failure case and LTM failure case defined in RAN2 above.

| Failure type | LTM failure case | Corresponding CHO failure case in [1] |
| --- | --- | --- |
| Too late failure type | 1a | Too late CHO Case 1 |
|  | 1b | too late CHO Case 2 |
|  | 1c |  |
| Too early failure type | 2a | too early CHO Case 1 and Case 2 |
|  | 2b | Null |
|  | 2c |  |
| To wrong cell failure type | 3a | CHO to wrong cell Case 1 |
|  | 3b | CHO to wrong cell Case 3 |
|  | 3c | CHO to wrong cell Case 2 |

We can see that for each LTM failure case defined by RAN2 there is a corresponding CHO failure type captured in R17. The main difference is for too early failure type. The reason is after failure, CHO can only perform RRC reestablishment to source cell while LTM may perform Recovery procedure to source cell because source cell may be included in candidate cell list in LTM. So, for too early failure type only the procedure of UE going back to source cell is different. Others are all the same.

Observation 1: For each LTM failure case agreed by RAN2, there is a corresponding CHO failure type which is captured in [1] in R17 except case 2b/2c of too early LTM.

According to above analysis, since LTM failure case is covered by CHO failure case, we could reuse CHO failure type detection mechanisms for LTM.

Proposal 1: It is proposed to reuse CHO failure type detection mechanisms for LTM with necessary update.

For too late and to wrong cell failure type, there is no obvious difference between CHO failure type and LTM failure type according to above table. So, only LTM scenario is captured in stage2 description.

For too early failure type, the description on LTM recovery to source cell is added.

Therefore, the updated detection mechanisms are as below:

-	Intra-system Too Late Handover: there is no recent handover for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if CHO/LTM is configured but the CHO/LTM execution is not initiated for the UE prior to the connection failure, e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt).

-	Intra-system Too Early Handover: there is a recent handover for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the successful re-connect cell/LTM recovery cell is the cell that served the UE at the last handover initialisation or fall back to the source cell configuration in case of DAPS HO.

-	Intra-system Handover to Wrong Cell: there is a recent handover for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/ the cell UE attempts to re-connect/the cell UE attempts CHO/LTM recovery is neither the cell that served the UE at the last handover initialisation nor the cell that served the UE where the RLF happened or the cell that the handover was initialized toward.

Proposal 2: It is proposed for RAN3 to discuss the above stage2 description.

Reviewing the CHO failure cases defined in R17, because of complexity, actually some CHO failure cases are not supported. For example, the recovery cell ID in RLF report is recorded as the following text in TS38.331.

1>	if the selected cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig and the condExecutionCondPSCell is not configured for the corresponding condReconfigId in the MCG VarConditionalReconfig:

2>	if the UE supports RLF-Report for conditional handover, set the choCellId in the VarRLF-Report to the global cell identity, if available, otherwise to the physical cell identity and carrier frequency of the selected cell;

2>	apply the stored condRRCReconfig associated to the selected cell and perform actions as specified in 5.3.5.3;

We can see that choCellId IE in RLF Report is recorded upon selecting a recovery NR cell but the result of CHO recovery procedure is not recorded in RLF Report. So, CHO to wrong cell Case 2 and case 3 in [1] cannot be distinguished by RLF Report in R17.

Observation 2: In R17 MRO for CHO, network cannot distinguish CHO recovery success and CHO recovery failure case by RLF report.

Another example is the case: RLF occurs shortly after CHO recovery success, a new RLF Report would be generated after successful CHO recovery procedure. This new RLF Report would overwrite the previous one which triggered by CHO handover failure, i.e. the information in the first RLF Report is lost. So, actually CHO to wrong cell Case 4 in [1] is not supported in R17.

Observation 3: In R17 MRO for CHO, the issue of two consecutive RLF report generated in the procedure of failure occurs shortly after CHO recovery success was not fully addressed because of complexity.

Since LTM failure type is similar to CHO failure type, we could take R17 CHO as base line. But considering CHO failure type is not fully supported currently, LTM failure case beyond CHO should not be pursued.

Proposal 3: It is proposed to take MRO for CHO as the base line for MRO for LTM, i.e. LTM failure case does not pursue to be supported if corresponding CHO failure case is not supported.

RACH-less and RACH-based access

In last RAN3 meeting, enhancement on RACH information was discussed and the following agreement was achieved.

RAN3 to discuss enhancements related to the information of RACH-less access and RACH-based access from UE related to MRO for LTM.

First, fall back mechanism from RACH-less to RACH-based access is not supported currently. The following text describes UE behavior when receiving LTM Cell Switch Command captured in TS38.321.

- 5.18.35 LTM Cell Switch Command The network may instruct the UE to perform LTM cell switch procedure by sending the LTM Cell Switch Command MAC CE described in clause 6.1.3.75. The MAC entity shall: 1> if the MAC entity receives an LTM Cell Switch Command MAC CE on a Serving Cell: 2> indicate to upper layers that the LTM cell switch procedure is triggered and the Target Configuration ID included in the LTM Cell Switch Command MAC CE; 2> if the MAC reset operation as specified in clause 5.12 is performed, as requested by upper layers: 3> if Timing Advance Command value (hexa-decimal) is not set as FFF: 4> process the received Timing Advance Command (see clause 5.2); 4> consider the RACH-less LTM cell switch to be ongoing; 4> if the MAC entity is associated with SCG: 5> indicate to upper layers to skip the Random Access procedure for this LTM cell switch. 3> else if the Timing Advance measurement is configured as specified in TS 38.331 [5] and the UE has successfully measured the Timing Advance for the indicated LTM target: 4> process the measured Timing Advance (see clause 5.2); 4> consider the RACH-less LTM cell switch to be ongoing. 4> if the MAC entity is associated with SCG: 5> indicate to upper layers to skip the Random Access procedure for this LTM cell switch. 3> consider the SSB associated to the TCI state indicated by TCI state ID field as the one used for configured uplink grant selection for the initial uplink transmission towards the candidate cell for RACH-less LTM cell switch (as in clause 5.8.2); 3> indicate to lower layers the information regarding the TCI state information included in the LTM Cell Switch Command MAC CE.

We can see that UE would perform RACH less LTM cell switch when Timing Advance is configured. Then UE performs RACH-less access until RACH failure occurs, i.e. t304 expires. There is not fallback mechanism in above procedure.

Observation 4: Fall back mechanism from RACH-less to RACH-based access is not supported according to TS38.321.

If network receives ra-InformationCommon in RLF report, network would perform RACH based optimization. However, a lack of ra-InformationCommon does not mean RACH-less happens because ra-InformationCommon is an optional IE. For safety, it would be better to introduce RACH-less indication in RLF report.

Proposal 4: It is propose to report RACH-less indication in RLF Report for optimization.

Split architecture

For split architecture, the following agreements have been achieved.

For failures due to wrong selection of candidate LTM cell, CU is in charge of root cause analysis and performs optimization.

For failures due to inappropriate cell switch triggering (e.g. wrong cell selection at cell switch, wrong cell switch timing, …) source DU performs optimization.

And there is a FFS as below:

For failures due to inappropriate cell switch triggering (e.g. wrong cell selection at cell switch, wrong cell switch timing, …) FFS if CU or DU performs root cause analysis.

Nowadays RAN2 has agreed to include L1 measurement result in RLF Report, but whether RRC could use the L1 measurement result is still unknown because legacy RRC only perform analysis on RRM measurement result. If RRC could perform the L1 measurement analysis, it propose to follow legacy principle that CU perform analyse. On the contrary, if RRC cannot analyse L1 measurement result, maybe only DU can perform the analysis on inappropriate cell switch triggering. So, RAN3 may ask RAN2 whether RRC could perform analysis on L1 measurement.

Proposal 5: It is proposed to ask RAN2 whether RRC can analysis L1 measurement and then decide whether CU perform analysis on inappropriate cell switch trigger based on L1L2 measurement.

Other issues

The goal of LTM is to enable a serving cell change via L1/L2 signaling, in order to reduce the latency, overhead and interruption time. UE may perform frequent and repeated handover within candidate cell list. Is there Ping-Pong issue during LTM triggered frequent and repeated handover needs consideration.

According to the LTM scenario, frequent and repeated handover may be normal. Moreover, it is hard for RRC to record every PCell information in legacy UHI because L1L2 make trigger handover very fast and RRC cannot catch up with L1L2.

Proposal 6: It is proposed for RAN3 to discuss how to tell frequent LTM handover from ping-pong issue and whether/how to update UHI during LTM.

During the following LTM triggered handover, more than one times of failure or near failure maybe occur. Since the LTM triggered handover may be frequent and quick, network may be not able to fetch SHR or/and RLF Report before the next failure or near failure occurs which may cause overwriting issue. RAN3 may discuss whether it is needed to record multiple failures or near failures information during LTM triggered handover. LS may be sent to ask RAN2 to record multiple failures or near failures information in UE, if needed.

Proposal 7: It is proposed for RAN3 to discuss whether it is needed to record multiple failures or near failures information during LTM triggered handover. If needed, LS to RAN2 may be sent.

- 2.2 MRO for CHO with candidate SCGs

In last RAN3 meeting, the failure case described in R3-243279 is noted as below:

Taking the cases described in 2.1 as the starting point to provide a separate Tdoc to maintain the cases worked in R19 in next meeting.

There are 9 cases listed in R3-243279, we simply summarize them as below

Phase1: UE is configured with CHO with candidate SCGs, but execution conditions are not satisfied. (case1/2/3/9)

Phase2: CHO with candidate SCGs execution conditions are satisfied, but CHO with candidate SCGs execution failure. (case4/7)

Phase3: CHO with candidate SCGs execution conditions are satisfied, but MCG/SCG failure occurs shortly after successful handover. (case5/6/8)

During each phase, there may be SCG or/and MCG failure occurs.

After MCG failure occurs, UE may preform RRC recovery procedure, RRC reestablishment procedure, or RRC re-connected procedure which is the same as R17 MRO for CHO, i.e. refer to CHO for the detailed failure case.

After SCG failure occurs, UE may send SCGFailureInformation message to network which is the same as R18 MRO for CPAC, i.e. refer to CPAC for the detailed failure case.

We can reuse R17 MRO for CHO and R18 MRO for CPAC as baseline, in this way, for CHO with candidate SCGs, the following procedure after SCG or MCG failure can be used.

For both MCG and SCG failure case occurs during CHO with candidate SCGs, it may be not needed to support as discussed in P10.

So, we make the following proposal.

Proposal 8: It is proposed to dividing the failure cases into three phases and the following procedures after SCG/MCG failures are similar for each phase. So, the CHO with candidate SCGs failure cases can be descripted in following way:

Phase 1: UE is configured with CHO with candidate SCGs, but execution conditions are not satisfied.

Phase 2: execution conditions are satisfied. CHO with candidate SCGs from PCell 1/PSCell A to PCell 2/ PSCell B execution failure.

Phase 3: execution conditions are satisfied. UE performs successful CHO with candidate SCGs from PCell 1/PSCell A to PCell 2/ PSCell B.

The following failure or procedure may occur for each phase:

- SCG failure occurs, UE may send SCGFailureInformation message to network which is same as R18 MRO for CPAC use case.

- MCG failure occurs, UE may preform RRC recovery procedure, RRC reestablishment procedure, or RRC re-connected procedure which is same as R17 MRO for CHO use case.

In case of only MCG or SCG failure during CHO with candidate SCGs, according to the reselecting PCell or PSCell, network may detect CHO with candidate SCGs too late/too early/to wrong cell failure type.

For example, UE is configured with CHO with candidate SCGs, but execution conditions are not satisfied, MCG failure occurs. If UE re-connects to a PCell different from the previous source PCell, it would a too late CHO with candidate SCGs for PCell. But the cause of this too late failure may be too strict SCG execution condition, i.e. either PCell or PSCell execution conditions may cause too late failure type which may have no relationship with MCG/SCG failure.

Observation 5：MCG failure occurs during CHO with candidate SCGs，either PCell or PSCell execution conditions may be optimized.

Similar example: SCG failure occurs when UE is configured with CHO with candidate SCGs, but execution conditions are not satisfied. The cause of this too late failure may be too strict MCG execution condition.

Observation 6：SCG failure occurs during CHO with candidate SCGs，either PCell or PSCell execution conditions may be the optimized.

For too early CHO with candidate SCGs and CHO with candidate SCGs Execution to wrong PSCell, for the same reason as too late failure type, either PCell or PSCell execution conditions may be optimized no matter MCG or SCG failure occurs.

Proposal 9: For only MCG or SCG failure during CHO with candidate SCGs, either PCell or PSCell configuration may be optimize no matter MCG or SCG failure occurs, i.e. MCG or SCG failure may cause optimization on PCell or/and PSCell configuration.

In case of both MCG and SCG failure during CHO with candidate SCGs, it may be first SCG failure and then MCG failure, or first MCG failure and then SCG failure. Anyway, the first MCG/SCG failure could trigger network to perform MRO on CHO with candidate SCGs based on the measurement result.

For the second MCG/SCG failure, it may have been optimized by the first failure triggered MRO, or if not, the second failure could be optimized by other CHO with candidate SCGs, i.e. solving all failure in one CHO with candidate SCGs is not perused due to complexity.

Observation 7：For both MCG and SCG failure during CHO with candidate SCGs, the first failure report (MCG failure report or SCG failure report) could trigger network to perform MRO on CHO with candidate SCGs.

According to above analysis, it is not needed to support MRO for both MCG and SCG failure during CHO with candidate SCGs because the MRO based on the first MCG/SCG failure is enough to optimize CHO with candidate SCGs.

Proposal 10: It is proposed to not support MRO for both MCG and SCG failure during CHO with candidate SCGs.

If SCG failure occurs, network could receive SCGFailureInformation message, network could trigger MRO on CHO with candidate SCGs based on UE context.

If MCG failure occurs first, network could receive RLF report at the time UE context may be removed. For example, to solve too late failure type which leads to MCG failure, both SCG execution condition and MCG execution condition needs to be optimized in order to be satisfied more easily. So, SCG related information (SCG execution condition/PSCell candidate cells) should also be saved by network or by UE.

Proposal 11: It is proposed for RAN3 to discuss how to obtain SCG related information when network receives RLF Report.

CHO with candidate SCGs should be not configured for some scenarios which are different from R17 CHO and R18 CPAC.

There is an example illustrated the scenario in figure1 and figure 2:

Figure 1: not suitable to configure CHO with candidate SCG scenario

Figure 2: suitable to configure CHO with candidate SCG scenario

CHO with candidate SCG(s) is defined as a PCell change with PSCell addition/change that is executed by the UE when the execution conditions for both candidate PCell and the associated candidate PSCell are met.

For the scenario in figure 1, the UE should first perform handover from PCell 1 to PCell 2 and then perform PSCell change from PSCell A to PSCell B. Even CHO with candidate SCG is configured, both PCell and PSCell execution conditions could not be satisfied at the same time because of no common coverage for PCell and PSCell. So, it is needed to optimize whether configure CHO with candidate SCG or not rather than CHO with candidate SCG configuration itself.

For the scenario in figure 2, CHO with candidate SCG could be used because both PCell and PSCell execution may be satisfied when UE within the common coverage area. If the configuration is un-optimized, there may be too early/too late/to wrong cell failure type occurs.

Observation 8: Only for figure 2 scenario, i.e. with common coverage area for PCell and PSCell, CHO with candidate SCG could be used. While in figure 1 scenario, CHO with candidate SCG should not be configured.

Proposal 12: It is proposed to first detect the scenario, i.e. whether common coverage exists for both PCell and PSCell.

- If not exist, CHO with candidate SCG should not be configured.

- If exist, MRO for CHO with candidate SCG configuration would be performed.

According to above understanding, before deciding the detection mechanism on CHO with candidate SCGs failure type, the text on scenario validation may be captured first in stage2 description. The following text may be captured:

CHO with candidate SCG(s) is defined as a PCell change with PSCell addition/change that is executed by the UE when the execution conditions for both candidate PCell and the associated candidate PSCell are met. If no common handover coverage for PCell and PSCell, CHO with candidate SCG(s) should not be configured.

Proposal 13: It is proposed to capture the text on common coverage detection for CHO with candidate SCGs in stage2 description.

In the figure 1 scenario, CHO with candidate SCG may not be triggered. So, legacy handover/PSCell change, CHO/CPC only, or CHO with target SCG should be used instead. If only configure CHO with candidate SCG, RLF may occur because CHO with candidate SCG would not be executed.

Therefore, since we may not able to aware about the scenario ahead of the time, legacy handover/PSCell change, CHO/CPC only, or CHO with target SCG may be configured as a backup together with CHO with candidate SCG in order to avoid RLF.

Proposal 14: It is proposed to support MRO for a case that UE is configured with legacy handover/PSCell change, CHO/CPC only or CHO with target SCG together with CHO with candidate SCGs.

For the SCG failure after successful CHO with candidate SCGs from PCell 1/PSCell A to PCell 2/ PSCell B, due to PCell 2/PSCell B taking as last serving node, Pell 2 may receive failure report from UE or third node, i.e. SCGFailureInformation message. After MRO analysis, PCell 1/PSCell A may be informed to perform optimization. Xn interface message may be enhanced to provide SCGFailureInformation message for the same UE from PCell 2 to PCell 1/PSCell A.

For example，SCGFailureInformation message is received by MN (PCell 2) shortly after a successful CHO with candidate SCGs from PCell 1/PSCell A to PCell 2/ PSCell B. If PCell1/PSCell A as the initiating node should be optimized, a handover report message（or other messages）should be sent from PCell2 to PCell1 which includes SCGFailureInformation message.

Proposal 15: It is proposed to enhance XN interface message to transfer SCGFailureInformation message for the same UE between source MN and target MN.

- 2.3 MRO for Subsequent CPAC

A Subsequent Conditional PSCell Addition or Change (subsequent CPAC) is defined as a conditional PSCell addition or change procedure that is executed after a PSCell addition, a PSCell change, a PCell change or an SCG release based on pre-configured subsequent CPAC configuration of candidate PSCell(s) without reconfiguration and re-initiation of CPC/CPA. So, RAN3 agree to take CPAC as baseline in last RAN3 meeting as below:

CPAC failure scenarios and detection mechanism captured in stage2 used as baseline.

The main difference between S-CPAC and CPAC is UE can perform continuous CPAC without RRC reconfiguration each time.

Another difference is the initiating node may be not the source SN and target SN of current PSCell change. For example, SN1 as source SN initiate subsequent CPAC, UE perform PSCell change from SN1 to SN2, and then PSCell change from SN2 to SN3. If SCG failure occurs in SN3, SN2 and SN3 as current source SN and target SN may be involved to perform MRO. SN1 as the initiating node which provide the candidate cell list may be also optimized.

Therefore, we think the above two differences could impact stage 2 detection mechanism.

Proposal 16: The main differences between S-CPAC and CPAC which could impact stage 2 detection mechanism may include:

- UE can perform continuous CPAC without RRC reconfiguration each time;

- The initiating node which may be not source SN and target SN should be involved to perform MRO;

For legacy MRO for CPAC, SCGfailureInformation message is used to indicate SCG failure information from UE to NG-RAN. As for subsequent CPAC, SCGfailureInformation message can be reused to indicate SCG failure during subsequent CPAC. Some necessary enhancement for SCGfailureInformation message may be needed.

Proposal 17: It is proposed to reuse SCGfailureInformation message to indicate SCG failure for MRO for subsequent CPAC.

Here we discuss the forwarding mechanism which is also based on CPAC. But there are some differences due to the following principles applied to subsequent CPAC captured in TS37.340.

-	For MN initiated subsequent CPAC, the MN initially triggers the candidate cell preparation of subsequent CPAC procedure and generates the execution conditions for the initial execution of subsequent CPAC (e.g. CPA or CPC).

-	For SN initiated subsequent CPAC, the source SN initially triggers the candidate cell preparation of subsequent CPAC procedure and generates the execution conditions for the initial execution of subsequent CPAC.

-	For both MN and SN initiated inter-SN subsequent CPAC, the candidate SN generates the execution conditions for the following execution of subsequent CPAC when the candidate SN prepares the candidate SCG configuration(s) for candidate PSCell(s). For SN initiated intra-SN subsequent CPAC, the source SN generates the execution conditions for the following execution of subsequent CPAC when the source SN prepares the candidate SCG configuration(s) for candidate PSCell(s).

For optimization on execution conditions, for the initial execution of S-CPAC, MN or SN as the initiate node generate the execution conditions.

For the following execution of subsequent S-CPAC, in case of no intra-SN PSCell change, candidate SN generates the execution conditions. Here the candidate SN refers to the source SN of PSCell change.

In case of intra-SN PSCell change, the source SN (or last serving SN) generates the execution conditions.

Therefore, execution conditions are always generated by the source SN of the latest PSCell change or MN for the initial execution of S-CPAC. When MN receives SCGfailureInformation message and decide it is not MN to optimize execution conditions, SCGfailureInformation message should be provided to the source SN (or last serving SN) to optimize execution conditions.

Proposal 18: It is proposed for MN to perform initial analysis when receiving SCGfailureInformation message from air interface. If needed, MN forwards SCGfailureInformation message to source SN to optimize execution conditions. The source SN is related to the latest PSCell change. In case of intra-SN PSCell change, source SN refer to last serving SN.

For optimization on candidate cell selection, PSCell candidate list is first selected by initiating node, and then selected by target SN. So, SCGfailureInformation message should be provided to initiating node and target SN to perform MRO on PSCell candidate selection.

Proposal 19: It is proposed for MN forwarding SCGfailureInformation message to initiating node and target SN to optimize PSCell candidate cell list.

According to above understanding, we update the stage 2 forwarding mechanism in TP.

Proposal 20: It is proposed for RAN3 to discuss the stage 2 text.

Network could save UE context in MN which may include the initiating node information. But MN may only know the initiating SN node ID and cannot know the PSCell ID in the initiating node. For the initiating node, it may be not selected as candidate cell and cannot save UE context when UE is not connected to this node. So, it is hard for network to save PSCell information in initiating node in case of SN initiating S-CPAC case.

As we discussed before, the initiating node and PSCell may be required to perform MRO, therefore, SCGfailureInformation may be enhanced to include PSCell information in initiating node.

Proposal 21: It is proposed to include PSCell information in initiating node in SCGfailureInformation message.

There is an issue raised in RAN3 meeting as below:

Support UHI and SCG UE History Information transmission during S-CPAC procedure to avoid the ping-pong or unnecessary PSCell change

To solve the issue on ping-pong or unnecessary PSCell change, there are mainly two steps:

The first step is how to record UHI and detect ping-pong occurrence.

The second step is to optimize configuration to solve the issue.

For the first step, UHI could be updated by MN during S-CPAC procedure. The related text in TS37.340 is as below:

The UE starts evaluating the execution conditions. If the execution condition of one candidate PSCell is satisfied, the UE applies RRCReconfiguration* message corresponding to the selected candidate PSCell, and sends an MN RRCReconfigurationComplete* message, including an RRCReconfigurationComplete** message for the selected candidate PSCell, and information enabling the MN to identify the SN of the selected candidate PSCell.

Since MN is aware of the each PSCell selected during S-CPAC procedure according to above text, MN could correlate PCell and PSCell, i.e. update the correlated UHI which includes PCell and PSCell information.

After the analysis of UHI, MN can detect ping-pong issue occurs during S-CPAC procedure. To solve the issue, i.e. the second step, there may be the following solutions:

Solution 1: MN sends UHI to every candidate SN each time UHI is updated.

Solution 2: MN sends UHI to every candidate SN when ping-pong issue is detected by MN.

Solution 3: MN sends UHI to the candidate SN which ping-pong issue is related to when detecting ping-pong issue by MN.

Solution 4: it is not needed to solve ping-pong issue immediately, i.e. the next time configuring S-CPAC, ping-pong issue will be solved according to the previous collected UHI.

Considering above solutions, we think not every candidate SN should be informed because they may be not involved in ping-pong issue, and it is also not needed to inform SN each time UHI is updated because ping-pong issue may not exist. So, solution 1 and 2 is not preferred.

For solution 3, after detecting ping-pong issue, MN informs the related SN about the updated UHI. The candidate cell add/release may be optimized at once which may lead to many interactions between every candidate cells because the change on each candidate cell should be informed to every other candidate cells in S-CPAC procedure.

For solution 4, considering the complexity to perform the modification of S-CPAC configuration, UHI can be used for the next S-CPAC, i.e. not perform optimization for current S-CPAC. Moreover, most of the times MRO is performed based on the statistics, not one time failure. Therefore, it is not needed to perform MRO in a hurry or each time Ping-Pong issue occurs. Comparing with solution 3, solution 4 also has no specification impact. So, solution 4 is preferred.

Proposal 22: It is proposed to take solution 4 to solve the ping-pong issue for the follow reason:

- Modification of S-CPAC configuration during S-CPAC procedure is complex.

- Performing MRO immediately is not required.

- No specification impact.

Conclusions

MRO for LTM:

Observation 1: For each LTM failure case agreed by RAN2, there is a corresponding CHO failure type which is captured in [1] in R17 except case 2b/2c of too early LTM.

Proposal 1: It is proposed to reuse CHO failure type detection mechanisms for LTM with necessary update.

Proposal 2: It is proposed for RAN3 to discuss the above stage2 description.

Observation 2: In R17 MRO for CHO, network cannot distinguish CHO recovery success and CHO recovery failure case by RLF report.

Observation 3: In R17 MRO for CHO, the issue of two consecutive RLF report generated in the procedure of failure occurs shortly after CHO recovery success was not fully addressed because of complexity.

Proposal 3: It is proposed to take MRO for CHO as the base line for MRO for LTM, i.e. LTM failure case does not pursue to be supported if corresponding CHO failure case is not supported.

Observation 4: Fall back mechanism from RACH-less to RACH-based access is not supported according to TS38.321.

Proposal 4: It is propose to report RACH-less indication in RLF Report for optimization.

Proposal 5: It is proposed to ask RAN2 whether RRC can analysis L1 measurement and then decide whether CU perform analysis on inappropriate cell switch trigger based on L1L2 measurement.

Proposal 6: It is proposed for RAN3 to discuss how to tell frequent LTM handover from ping-pong issue and whether/how to update UHI during LTM.

Proposal 7: It is proposed for RAN3 to discuss whether it is needed to record multiple failures or near failures information during LTM triggered handover. If needed, LS to RAN2 may be sent.

MRO for CHO with candidate SCGs:

Proposal 8: It is proposed to dividing the failure cases into three phases and the following procedures after SCG/MCG failures are similar for each phase. So, the CHO with candidate SCGs failure cases can be descripted in following way:

Phase 1: UE is configured with CHO with candidate SCGs, but execution conditions are not satisfied.

Phase 2: execution conditions are satisfied. CHO with candidate SCGs from PCell 1/PSCell A to PCell 2/ PSCell B execution failure.

Phase 3: execution conditions are satisfied. UE performs successful CHO with candidate SCGs from PCell 1/PSCell A to PCell 2/ PSCell B.

The following failure or procedure may occur for each phase:

- SCG failure occurs, UE may send SCGFailureInformation message to network which is same as R18 MRO for CPAC use case.

- MCG failure occurs, UE may preform RRC recovery procedure, RRC reestablishment procedure, or RRC re-connected procedure which is same as R17 MRO for CHO use case.

Observation 5：MCG failure occurs during CHO with candidate SCGs，either PCell or PSCell execution conditions may be optimized.

Observation 6：SCG failure occurs during CHO with candidate SCGs，either PCell or PSCell execution conditions may be the optimized.

Proposal 9: For only MCG or SCG failure during CHO with candidate SCGs, either PCell or PSCell configuration may be optimize no matter MCG or SCG failure occurs, i.e. MCG or SCG failure may cause optimization on PCell or/and PSCell configuration.

Observation 7：For both MCG and SCG failure during CHO with candidate SCGs, the first failure report (MCG failure report or SCG failure report) could trigger network to perform MRO on CHO with candidate SCGs.

Proposal 10: It is proposed to not support MRO for both MCG and SCG failure during CHO with candidate SCGs.

Proposal 11: It is proposed for RAN3 to discuss how to obtain SCG related information when network receives RLF Report.

Observation 8: Only for figure 2 scenario, i.e. with common coverage area for PCell and PSCell, CHO with candidate SCG could be used. While in figure 1 scenario, CHO with candidate SCG should not be configured.

Proposal 12: It is proposed to first detect the scenario, i.e. whether common coverage exists for both PCell and PSCell.

- If not exist, CHO with candidate SCG should not be configured.

- If exist, MRO for CHO with candidate SCG configuration would be performed.

Proposal 13: It is proposed to capture the text on common coverage detection for CHO with candidate SCGs in stage2 description.

Proposal 14: It is proposed to support MRO for a case that UE is configured with legacy handover/PSCell change, CHO/CPC only or CHO with target SCG together with CHO with candidate SCGs.

Proposal 15: It is proposed to enhance XN interface message to transfer SCGFailureInformation message for the same UE between source MN and target MN.

MRO for Subsequent CPAC:

Proposal 16: The main differences between S-CPAC and CPAC which could impact stage 2 detection mechanism may include:

- UE can perform continuous CPAC without RRC reconfiguration each time;

- The initiating node which may be not source SN and target SN should be involved to perform MRO;

Proposal 17: It is proposed to reuse SCGfailureInformation message to indicate SCG failure for MRO for subsequent CPAC.

Proposal 18: It is proposed for MN to perform initial analysis when receiving SCGfailureInformation message from air interface. If needed, MN forwards SCGfailureInformation message to source SN to optimize execution conditions. The source SN is related to the latest PSCell change. In case of intra-SN PSCell change, source SN refer to last serving SN.

Proposal 19: It is proposed for MN forwarding SCGfailureInformation message to initiating node and target SN to optimize PSCell candidate cell list.

Proposal 20: It is proposed for RAN3 to discuss the stage 2 text.

Proposal 21: It is proposed to include PSCell information in initiating node in SCGfailureInformation message.

Proposal 22: It is proposed to take solution 4 to solve the ping-pong issue for the follow reason:

- Modification of S-CPAC configuration during S-CPAC procedure is complex.

- Performing MRO immediately is not required.

- No specification impact.

Reference

[1] R3-214432 “Update of Way forward on Scenarios for SON enhancements for CHO and DAPS HO”RAN3#113

TP for TS 38.300

- 15.5.2.2.2	Connection failure due to intra-system mobility

<<<<<<<<<<<<<<<<<< Begin of the changes >>>>>>>>>>>>>>>>>>>>>>>

The detailed detection mechanisms for too late handover, too early handover and handover to wrong cell are carried out through the following in the NG-RAN node that served the UE before the reported connection failure:

-	Intra-system Too Late Handover: there is no recent handover for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if CHO/LTM is configured but the CHO/LTM execution is not initiated for the UE prior to the connection failure, e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt).

-	Intra-system Too Early Handover: there is a recent handover for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the successful re-connect cell/LTM recovery cell is the cell that served the UE at the last handover initialisation or fall back to the source cell configuration in case of DAPS HO.

-	Intra-system Handover to Wrong Cell: there is a recent handover for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/ the cell UE attempts to re-connect/the cell UE attempts CHO/LTM recovery is neither the cell that served the UE at the last handover initialisation nor the cell that served the UE where the RLF happened or the cell that the handover was initialized toward.

<<<<<<<<<<<<<<<<<< End of the changes >>>>>>>>>>>>>>>>>>>>>>>

TP for TS 37.340

- 10.18.x	CHO with candidate SCG(s).

CHO with candidate SCG(s) is defined as a PCell change with PSCell addition/change that is executed by the UE when the execution conditions for both candidate PCell and the associated candidate PSCell are met. If no common handover coverage for PCell and PSCell, CHO with candidate SCG(s) should not be configured.

- 10.18.y	Subsequent CPAC.

The MN performs the initial analysis when SCGFailureInformation is received from the UE. If needed, MN forwards SCGFailureInformation message to source SN to optimize execution conditions. The source SN is related to the latest PSCell change when MN receives SCGfailureInformation message. In case of intra-SN PSCell change, source SN refer to last serving SN.

For MN initiated S-CPAC, if the suitable PSCell is one of the candidate PSCells provided by the MN at CPAC preparation, but not one of the candidate PSCells selected by the candidate or target SN, MN sends the SCG Failure Information Report message to the candidate or target SN, which perform the final MRO related optimization. Otherwise, the MN performs the final MRO related optimization.

For SN initiated S-CPAC, the MN sends the SCG Failure Information Report message to the SN which initiating S-CPAC, and the initiating SN performs root cause analysis. If the suitable PSCell is one of the candidate PSCells provided by the initiating SN, but not one of the candidate PSCells selected by the candidate or target SN, the initiating SN indicates to MN that the root cause of the SCG failure may have occurred in the other nodes. MN then sends the SCG Failure Information Report message to the candidate or target SN. Otherwise, the initiating SN performs the final MRO related optimization.