3GPP TSG-RAN WG3 #125                                                 R3-244267

Maastricht, Netherland, 19th – 23rd Aug, 2024

Agenda Item:	10.4

Source:	CATT

Title:	Discussion on SON enhancement for MR-DC SCG failure

Document for:	Discussion

Introduction

Reply LS on support of MRO for MR-DC SCG failure from RAN2 is received as in [1], here we provide some analysis on MRO enhancement for MR-DC SCG failure.

Discussion

  - MRO for MR-DC SCG failure

In Reply LS on support of MRO for MR-DC SCG failure [1], RAN2 provide the supported MR-DC case as following:

RAN2 thanks RAN3 for the LS, and RAN2 agrees to only support EN-DC case on MRO for MR-DC SCG failure. RAN2 understands that whether also supporting NGEN-DC has no additional RAN2 impact hence RAN3 can decide. If time allows in Rel-19, RAN2 can consider NE-DC scenario

RAN2 has decided to support EN-DC case at first. From RAN3 perspective, there is no additional technical issue to support NGEN-DC. The same specification impact can be copied to 38.423. RAN3 can support NGEN-DC case. For NE-DC case, RAN3 may wait for RAN2 result.

Proposal 1: In R19, it is proposed to prioritize EN-DC case and NGEN-DC case for SCG failure.

In R17 NR-DC, we introduced SCG Failure Information Report and SCG Failure Transfer procedure to ask SN whether there is intra-SN PSCell change occurs. In this way, MN can obtain overall information and decide which RAN node shall be optimized.

On MRO for EN-DC and NGEN-DC, in case of SCG failure, the SCGFailureInformationNR message is sent to MN. SCG Failure Information Report and SCG Failure Transfer procedure could be reused to send information between MN and SN.

Proposal 2: It is proposed to reuse SCG Failure Information Report and SCG Failure Transfer procedure in EN-DC case and NGEN-DC case.

For EN-DC and NGEN-DC case, the SCGFailureInformationNR message is defined in TS36.331 as below:

SCGFailureInformationNR message

-- ASN1START

SCGFailureInformationNR-r15 ::=		SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

scgFailureInformationNR-r15			SCGFailureInformationNR-r15-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

SCGFailureInformationNR-r15-IEs ::=	SEQUENCE {

failureReportSCG-NR-r15				FailureReportSCG-NR-r15				OPTIONAL,

nonCriticalExtension					SCGFailureInformationNR-v1590-IEs	OPTIONAL

}

SCGFailureInformationNR-v1590-IEs ::=	SEQUENCE {

lateNonCriticalExtension					OCTET STRING					OPTIONAL,

nonCriticalExtension						SEQUENCE {}					OPTIONAL

}

FailureReportSCG-NR-r15 ::=		SEQUENCE {

failureType-r15						ENUMERATED {

t310-Expiry, randomAccessProblem,

rlc-MaxNumRetx,

synchReconfigFailureSCG, scg-reconfigFailure,

srb3-IntegrityFailure, dummy},

measResultFreqListNR-r15				MeasResultFreqListFailNR-r15		OPTIONAL,

measResultSCG-r15						OCTET STRING						OPTIONAL,

...,

[[	locationInfo-r16				LocationInfo-r10						OPTIONAL,

logMeasResultListBT-r16			LogMeasResultListBT-r15					OPTIONAL,

logMeasResultListWLAN-r16		LogMeasResultListWLAN-r15				OPTIONAL,

failureType-v1610				ENUMERATED {t312-Expiry, scg-lbtFailure,

beamFailureRecoveryFailure, bh-RLF-r16,

beamFailure-r17,

spare3, spare2, spare1}	OPTIONAL

]]

}

measResultSCG Includes the NR MeasResultSCG-Failure IE as specified in TS 38.331 [82]. The field contains available results of measurements on NR frequencies the UE is configured to measure by the NR RRCConfiguration message.

The field measResultSCG is encoded in NR format hence LTE MN cannot decode and use it to select the next suitable PSCell. Although measResultFreqListNR-r15 can be decoded by MN, we notice it is so limited because it includes only SSB measurement result, i.e. cell measurement based on CSI-RS is not supported. In our understanding, SSB RRM measurement result is a little coarse for mobility decision and CSI-RS can provide more precise RRM measurement result which is more important for improving handover successful rate. Without CSI-RS measurement result, MN cannot select the next suitable PSCell and then perform MRO analysis.

Observation 1: For EN-DC and NGEN-DC case, MN cannot decode measResultSCG in SCGFailureInformationNR message while measResultFreqListNR-r15 only supports SSB RRM measurement which cannot provide precise measurement result for MN to select the next suitable PSCell.

We propose last serving SN takes the responsibility to select the next suitable PSCell based on measResultSCG in SCGFailureInformationNR message and send it to MN in existing SCG Failure Transfer message if last serving SN think there is other RAN nodes needs optimization.

Proposal 3: Last serving SN selects the next suitable PSCell based on the measResultSCG in SCGFailureInformationNR message.

In R17 NR-DC case, the following is captured in TS37.340.

The node responsible for the last PSCell change (the source SN, the last serving SN or the MN) performs the final root cause analysis.

For MN initiate PScell change, MN should perform final root cause analysis in R17 NR-DC case. While for EN-DC and NGEN-DC case, it may be not reasonable to enhance LTE MN to perform the final root cause analysis for NR SCG failure case.

If SN performs final root cause analysis, MN should provide previous PSCell ID, failed PSCell ID, and timeSCGFailure explicitly to SN in XN interface in order for SN to detect failure type.

If MN performs final root cause analysis, last serving SN should provide the next suitable PSCell to MN.

Either solution is feasible. We slightly prefer NR SN to LTE MN due to less impact on LTE.

Proposal 4: It is proposed for RAN3 to discuss which node perform final root cause analysis in case of MN initiated PSCell Change, LTE MN or NR SN? We slightly prefer NR SN due to less impact on LTE.

Conclusions

Proposal 1: In R19, it is proposed to prioritize EN-DC case and NGEN-DC case for SCG failure.

Proposal 2: It is proposed to reuse SCG Failure Information Report and SCG Failure Transfer procedure in EN-DC case and NGEN-DC case.

Observation 1: For EN-DC and NGEN-DC case, MN cannot decode measResultSCG in SCGFailureInformationNR message while measResultFreqListNR-r15 only supports SSB RRM measurement which cannot provide precise measurement result for MN to select the next suitable PSCell.

Proposal 3: Last serving SN selects the next suitable PSCell based on the measResultSCG in SCGFailureInformationNR message.

Proposal 4: It is proposed for RAN3 to discuss which node perform final root cause analysis in case of MN initiated PSCell Change, LTE MN or NR SN? We slightly prefer NR SN due to less impact on LTE.

Reference

[1] R3-244011 “Reply LS on support of MRO for MR-DC SCG failure” 3GPP TSG RAN WG3 Meeting #125