3GPP TSG-RAN WG3 Meeting #125	R3-244269

Maastricht, The Netherlands, August 19th – 23rd 2024

Agenda Item:	12.2

Source:	Ericsson

Title:	(pCR for TR 38.799): Functional Aspects of WAB-Nodes

Document for:	Agreement

# Introduction

In this paper we discuss the functional aspects of WAB-nodes.

A pCR for TR 38.799 is provided in the Annex.

# Discussion

We discuss the following functional aspects of WAB-nodes:

- NG connection management.

- Mobility handling.

- WAB authorization handling.

In addition, certain cleanups of the TR 38.799 are proposed.

## NG connection management

The following FFS was captured in the RAN3#124 meeting notes:

FFS whether NG connection(s) can be suspended.

We think that the support for NG connection suspension for WAB-nodes should be considered. In some scenarios, where the trajectory of the vessel with the WAB-node is cyclic, instead of removing the NG connection upon leaving the area of a certain AMF set, it may be more meaningful to suspend the NG connection, to avoid having to set it up and tear it down repeatedly. Another situation in which the NG suspension may be useful is when the WAB-node is temporarily service-unauthorized (e.g., during the night or when the aircraft carrying the node is on the ground). Then, once the service-authorization status changes to “authorized”, setting up the NG connectivity from scratch could be avoided.

Proposal 1: The NG connection(s) of WAB-node can be suspended.

## Mobility handling

Most of the open issues related to WAB-node mobility are captured in TR 38.799 in the form of Editor’s Notes (EN).

One EN states the following:

Editor’s NOTE: Whether the TAC can be changed without the change of UE’s AMF needs to be confirmed.

With respect to the above, we note that the existing NGAP signalling enables the RAN node to inform the AMF about any updates of the list of supported TACs, which means that the EN can be removed.

Proposal 2-1: Remove the following Editor’s NOTE from TR 38.799: “Whether the TAC can be changed without the change of UE’s AMF needs to be confirmed.”.

The main mobility-related issue, however, is captured in the below EN, for the scenario of WAB-gNB mobility with AMF change:

Editor’s NOTE: FFS whether a new logical WAB-gNB needs to be established for the NG connections with the new AMF(s).

So, the essential question is whether it is necessary for a WAB-node to instantiate a second logical WAB-gNB upon WAB-gNB mobility with AMF change.

In our view, the approach with two logical WAB-gNBs seems cleaner than the single logical WAB-gNB approach. For example, if a WAB-gNB undergoing AMF change keeps its TAC under the new AMF, an intra-gNB handover in conjunction with NG-based handover would need to be executed for each UE, as defined in TS 23.502. Herein, since the source and target gNB are the same, the Target ID formulated by the WAB-gNB, and inserted into the HANDOVER REQUIRED NGAP message, would contain the gNB ID and the TAC identical to the gNB ID and the TAC of the WAB-gNB that sent the message. Based on this information, the source AMF should find the target AMF, but it is unclear how this can be done since the source AMF will see the gNB ID and the TAC pertaining to the WAB-gNB that sent the HANDOVER REQUIRED message. To avoid this issue, additional handling logic at the source AMF is needed. Note that the intra-gNB NG-based handover is supported for NTN, but, in this case, the PLMN ID and the TAC indicated in the Target ID IE are different than the PLMN ID and the TAC pertaining to the source cell. In case of two logical WAB-gNBs, at least the gNB ID in the Target ID IE would be different than the ID of the WAB-gNB that sent the HANDOVER REQUIRED message.

Proposal 2-2: Upon mobility with AMF change, a WAB-node can instantiate a second logical WAB-gNB to establish NG connections with the new AMF(s).

## WAB authorization

RAN3 discussions so far assume that the service authorization status of a WAB-gNB can change. For example, a WAB-gNB can be service-authorized to operate only in certain areas or at certain times of day. The conditions under which a WAB-gNB is allowed to serve UEs are managed by service authorization policies.

Clause 4.3.3 of TR 38.799, which deals with WAB-node configuration, allows for a WAB-node to be provided with location-based configurations in advance. Then, the node is responsible for activation of these configuration parameters based on its current location. In our understanding, the same approach should apply for service authorization policies as well. Namely, the OAM could provide service authorization policies to the WAB-gNB, and the WAB-gNB could enforce these policies based on the conditions stipulated therein. Alternatively, the service authorization policy could be enforced by the OAM, i.e., the OAM could configure the WAB-gNB only when the WAB-gNB is authorized.

Proposal 3-1: The OAM can provide the WAB-gNB with the policies for controlling its service authorization status, or the OAM can enforce the policies by configuring the WAB-gNB only when the WAB-gNB is authorized.

At the RAN3#124 meeting it was agreed to support roaming scenarios for the WAB-MT, whereas the WAB-gNB always remains connected to the same PLMN. For WAB-gNB service authorization, this implies that service authorization of a WAB-gNB should be managed by the OAM in the network where the WAB-gNB is connected, even if the WAB-MT is connected to another PLMN.

Proposal 3-2: If the WAB-MT and WAB-gNB are connected to different PLMNs, service authorization of the WAB-gNB is managed by the PLMN in which the WAB-gNB is connected.

Based on the above discussion, a pCR for TR 38.799 is provided in the Annex.

Proposal 4: Agree the pCR for TR 38.799 in the Annex.

# Conclusion

This paper discusses the functional aspects of WAB-node. The following is proposed:

NG connection management

Proposal 1: The NG connection(s) of WAB-node can be suspended.

Mobility handling

Proposal 2-1: Remove the following Editor’s NOTE from TR 38.799: “Whether the TAC can be changed without the change of UE’s AMF needs to be confirmed.”.

Proposal 2-2: Upon mobility with AMF change, a WAB-node can instantiate a second logical WAB-gNB to establish NG connections with the new AMF(s).

WAB authorization

Proposal 3-1: The OAM can provide the WAB-gNB with the policies for controlling its service authorization status, or the OAM can enforce the policies by configuring the WAB-gNB only when the WAB-gNB is authorized.

Proposal 3-2: If the WAB-MT and WAB-gNB are connected to different PLMNs, service authorization of the WAB-gNB is managed by the PLMN in which the WAB-gNB is connected.

The pCR

Proposal 4: Agree the pCR for TR 38.799 in the Annex.

# Annex: pCR for TR 38.799

-------------------------------------------Start of changes-------------------------------------------

## 4.2	WAB Architecture

Figure 4.2-1 shows an example of WAB architecture for 5GS when the WAB-gNB’s NG traffic is transported via PDU session backhaul.

Figure 4.2-1: The WAB architecture example for 5GS when the WAB-gNB traffic is transported via PDU session backhaul

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

## 4.3	Operational aspects

### 4.3.1	WAB-node integration procedure

Figure 4.3.1-1 WAB-node integration procedure

Phase 1: WAB-MT setup. The WAB-MT of a WAB-node connects to the network in the same way as a UE by performing RRC connection setup procedure with the BH-RAN-node. The WAB-MT then performs, authorization and authentication with the BH-5GC. After the WAB-MT is authorized, the WAB-MT can establish one or more PDU sessions for backhauling.

Phase 2: WAB-gNB setup. This phase includes the following 3 sub-phases:

Phase 2-1: WAB-gNB initialization. In this phase, the WAB-gNB is configured by the OAM (e.g., with the information of AMF(s) to serve the UE) and service-authorized by the SeGW or by the OAM.

Phase 2-2: NG connection setup. The WAB-gNB establishes NG connection(s) toward the AMF(s). This step may follow legacy procedures. After the NG is set up, the WAB-gNB can start serving UE(s).

Phase 2-3: Xn connection setup. If needed, the WAB-gNB may establish Xn connection(s) towards the BH-RAN-node and/or other NG-RAN node(s).

### 4.3.2	WAB authorization

WAB authorization includes the authorization of the WAB-MT and the service authorization of the WAB-gNB. The authorization of the WAB-MT is different from the service authorization/configuration/activation of the WAB-gNB.

Authorization of the WAB-MT provides the WAB-MT with the right to support backhauling the traffic of the co-located WAB-gNB’s via BH PDU session(s).

Authorization of the WAB-gNB provides the service authorization, i.e., the right to serve UEs. The service authorization of the WAB-gNB is performed by e.g., OAM/SeGW using legacy procedures. The service authorization can be, e.g., steered by location-based policies, which can be provided in advance to the node by the OAM/SeGW. Alternatively, the OAM can enforce the policies by configuring the WAB-gNB only when the WAB-gNB is authorized.

The WAB-gNB’s service authorization status may change. In case the WAB-gNB’s service authorization status changes from “authorized” to “not authorized”, the UEs served by the WAB-gNB can either be handed over to other RAN nodes or they can be released, after which the NG and Xn connection(s) of the WAB-gNB can be removed or suspended.

In all scenarios, including roaming scenarios, service authorization of the WAB-gNB is managed by the network in which the WAB-gNB is connected.

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

### 4.3.4	Mobility handling

The following scenarios for WAB-node mobility are supported:

- The UE´s AMF/UPF remains unchanged as the WAB-gNB moves.

- The UE´s AMF/UPF changes as the WAB-gNB moves.

- The BH-UPF/AMF remains unchanged as the WAB-MT moves inside a PLMN.

- The BH-UPF/AMF changes as the WAB-MT moves inside a PLMN.

RAN3 assumes that the WAB-gNB does not change PLMN while the WAB-node moves across the network.

#### 4.3.4.1	WAB-MT mobility

The WAB-MT reuses legacy mobility handover procedures for the UE as it moves throughout the BH-RAN.

##### 4.3.4.1.1 The BH-UPF/BH-AMF remains unchanged as the WAB-MT moves inside a PLMN.

In case the WAB-MT’s PSA UPF does not change during these mobility procedures, the IP addresses allocated for the WAB-MT’s PDU sessions do not change. Therefore, the NG and Xn connections of the WAB-gNB carried over the BH PDU session(s) remain unaffected.

##### 4.3.4.1.2 BH-UPF/BH-AMF changes as the WAB-MT moves inside a PLMN.

When the WAB-MT’s PSA UPF needs to be changed due to WAB-MT’s mobility, new BH PDU session(s) need to be established reusing existing mechanisms defined in TS 23.501 [x]/TS 23.502 [y]. In this case, the (outer) IP addresses used by the WAB-gNB for the transport of NG and Xn connections of the WAB-gNB will change.

##### 4.3.4.1.3 Roaming of the WAB-MT

During WAB-node movement, the WAB-MT may connect to a PLMN different than its HPLMN.

#### 4.3.4.2	WAB-gNB mobility

During WAB-node movement, the establishment and the removal of the WAB-gNB’s NG and/or Xn connections may be needed. The WAB-gNB’s NG connections may also be suspended.

Establishment of Xn connections of the WAB-gNB with BH-RAN nodes, as well as with surrounding RAN nodes, is supported, and it can follow legacy procedures.

##### 4.3.4.2.1 WAB-gNB mobility without change of UE’s AMF(s)

During WAB-node movement, radio configuration parameters of the WAB-gNB may be changed (e.g., cell ID, PCI and/or TAC) without the change of the UE’s AMF(s). This change of radio configuration parameters may require the UE handling by means of, e.g., intra-gNB handover and/or Mobility Registration Update as defined in TS 23.502 [y].

##### 4.3.4.2.2 WAB-gNB mobility with change of UE’s AMF(s)

Due to WAB-node movement, the change of UE’s AMF(s) may be needed, based on, e.g., WAB-node’s current location and/or additional criteria. The NG connection handling and WAB-gNB configuration update may affect the served UEs.

- The WAB-gNB may obtain the configuration parameters needed to establish the connection to the UE’s new AMF(s).

- A new logical WAB-gNB is instantiated, and it establishes NG connection(s) towards one or more new AMF(s).

- The new logical WAB-gNB may activate one or more new cells, with new cell configuration parameters related to the WAB-gNB’s current location. The new cells may broadcast the radio parameters configured for the new AMF(s), e.g., TAC, etc. The old cell(s) remain(s) active.

- The UEs are handled as follows:

- A UE in RRC_CONNECTED state is handed over between an old cell served by the old logical WAB-gNB and a new cell served by the new logical WAB-gNB via NG-based handover with AMF relocation, as defined in TS 23.502 [y].

- When all UEs in RRC_CONNECTED state have been handed over, the old cell(s) are removed from service. A UE in RRC_IDLE or RRC_INACTIVE state camping on the old cell(s) reselects a new cell, and legacy procedure (e.g., Mobility Registration Update procedure as defined in TS 23.502 [y]) is performed.

- The NG connection(s) between the old logical WAB-gNB and the old AMF(s) are removed and the old logical WAB-gNB is removed from service.

### 4.3.5 Resource multiplexing

In scenarios where WAB-node’s access link and backhaul link mutually interfere, resource coordination may be needed to facilitate the resource multiplexing of the WAB-node’s access links and backhaul link. For this purpose, the resource coordination mechanism introduced for IAB can be considered as the starting point. For resource coordination between the access link and backhaul link, the BH-gNB needs to discover co-location of the WAB-MT and the WAB-gNB.

-------------------------------------------End of changes-------------------------------------------