3GPP TSG-RAN WG3 Meeting #125	R3-244270

Maastricht, The Netherlands, August 19th – 23rd 2024

Agenda Item:	12.2

Source:	Ericsson

Title:	(pCR for TR 38.799): Handling of Reliability and Latency for WAB

Document for:	Agreement

# Introduction

In this paper we discuss the handling of reliability and latency for WAB-nodes.

A pCR for TR 38.799 is provided in the Annex.

# Discussion

We discuss the following issues:

- Handling of backhaul link degradation.

- Packet delay budget (PDB) handling.

## Handling of backhaul link degradation

The traffic to and from the UEs served by the WAB-gNB will traverse two wireless links:

- The backhaul (BH) link between the BH-gNB and the WAB-MT.

- The access link between the WAB-gNB and the UE.

The BH link, like any other wireless link, is prone to degradation, failures, and congestion. If the BH link is in bad conditions (poor coverage, congestion etc.), all PDU sessions of the WAB-MT carrying the traffic to and from the WAB-gNB may be affected. In the worst case, the NG connection(s) of the WAB-gNB may fail, resulting in the WAB-gNB and its served UEs being cut-off from the network.

Observation 1-1: When BH link degradation occurs, avoiding NG connection failure is of highest priority.

Several issues need to be considered.

First, we notice that, when it comes to communication in the WAB architecture, there exist multiple control loops. These loops are overlapping, but may be managed by different, possibly unrelated, entities. For example, one loop, which encompasses the UE PDU sessions established between the UEs and the WAB-5GC, is controlled by the UE’s 5GC. The access link for delivering this traffic to/from the UE is controlled by the WAB-gNB. On the other hand, the UE PDU sessions are carried inside PDU sessions of the WAB-MT (managed by the BH-5GC), over the radio resources on the BH link (managed by the BH-gNB). The WAB-MT’s PDU sessions are managed by the BH-5GC.

Observation 1-2: In the WAB architecture, there are multiple overlapping control loops:

- Radio resources on the BH link are controlled by the BH-gNB.

- Radio resources on the access link are controlled by the WAB-gNB.

- PDU sessions of the WAB-MT are controlled by the BH-5GC.

- UE data traffic flow is controlled by the UE’s 5GC.

Second, the existing tools defined for PDU session management (e.g., on the NG interface) are not suitable for dealing with BH link degradation because:

- The WAB architecture has not yet been specified, meaning that the existing tools against link degradation have not been tailored to the WAB architecture. For example, the design of existing tools did not consider the scenario where WAB-MT’s PDU sessions are used to tunnel the PDU sessions between the WAB-gNB and the UE’s 5GC.

- The NG protocol (including NG-C and NG-U) is designed for wired networks. Meanwhile, in WAB architecture the NG traffic is carried over the wireless BH link.

The existing user plane failure handling can only deal with the GTP-U failure by using Error Indication defined for the GTP-U protocol in TS 29.281. For UEs served by a WAB-node, this indication is sent to the UE’s 5GC. Nevertheless, this indication is intended for different types of protocol failures and session context unavailability, rather than for indicating link degradation. A similar conclusion holds for the NGAP Error Indication procedure. This means that, as of today, the NG signalling cannot be used to indicate that the cause of problems is wireless BH link degradation. Also, a procedure to recover from poor link conditions is not specified. Hence, we conclude that the existing tools cannot support handling of BH link degradation in the WAB architecture.

Observation 1-3: The NG protocol is designed for wired networks, while the NG connection between the WAB-gNB and WAB-5GC is carried over non-stationary wireless BH.

Third, when the BH wireless link deteriorates, the BH-gNB and the WAB-MT are almost immediately aware of it. Meanwhile, the WAB-gNB and the UE’s 5GC are likely unaware of whether the problem occurred on the BH link or in the wired network between the BH-gNB and the UE’s 5GC. They may observe that there is an issue, but they cannot infer whether they can do something about it.

As observed earlier, in this case, it is of utmost importance to prevent the failure of the WAB-gNB NG-C connection. Unfortunately, the BH-gNB may not be able to determine which PDU session/radio resources on the BH link are used for carrying the NG-C traffic and may not be able to prevent the failure of NG connection in the best way. On the other hand, the WAB-gNB and the UE’s 5GC may help the BH-gNB in dealing with BH link degradation and avoiding NG-C connection failure. For example:

- The WAB-gNB and the UE’s AMF(s) may prioritize the sending of NG control plane traffic and the SCTP traffic (e.g., heartbeat messages and pings), to prevent NG connection failure.

- The WAB-gNB may reduce the amount of traffic of the NG and/or Uu interfaces, by e.g., prioritizing some traffic and disconnecting low-priority traffic, so that the remaining available bandwidth is used for control plane traffic.

Observation 1-4: The BH-gNB is able to determine degradation of wireless BH, but it may not be able to determine which PDU session/radio resources on the BH link are used for carrying the NG-C traffic, and it may not be able to prevent the failure of WAB-gNB NG-C connection.

Observation 1-5: The WAB-gNB and UE’s AMF can take actions that can contribute to avoiding NG-C connection failure.

Based on the above, we conclude that, to be able to counteract BH link degradation and to avoid NG-C connection failure, the access and the BH network should be able coordinate for mitigating BH link degradation, so that appropriate action can be taken.

Proposal 1: The access and the BH network can coordinate for mitigating BH link degradation.

## PDB handling

Fig. 1: WAB architecture

In legacy scenarios involving “normal” gNBs (i.e., non-WAB-gNBs), there is no wireless BH – in this case, there is only one wireless hop on the way to the UE, i.e., the Uu interface between the gNB and the UE. In that case, the total end-to-end (i.e., WAB-UPF-to-UE) PDB pertaining to a 5QI consists of the RAN part (5G-AN PDB from TS 23.501) and the CN part (CN PDB from TS 23.501).

With respect to the PDB, in legacy scenarios, during the UE PDU session and QoS setup, the UE’s 5GC should provide to the NG-RAN the CN PDB, either via NGAP signalling, or a static value can be preconfigured/hard-coded at the NG-RAN node. The access network (AN, i.e., the NG-RAN) uses it to calculate the 5G-AN PDB and considers this value in its scheduling decisions. In other words, the PDB of the RAN (5G-AN PDB from TS 23.501) equals the PDB indicated in the configured 5QI minus the CN PDB (either signalled or predefined):

5G-AN PDB = (Total end-to-end PDB) – CN PDB

According to the WAB architecture, shown in Fig. 1, the traffic to/from the UEs served by the WAB-gNB will traverse two wireless links:

- The first link between the BH-gNB and the WAB-MT, i.e., the BH link.

- The second link between the WAB-gNB and the UE, i.e., the access link.

Hence, in the WAB architecture, compared to the legacy scenario, there exists an additional wireless hop between the BH-gNB and the WAB-MT – the packet towards a UE traverses the UE’s 5GC, the BH-5GC, the BH-gNB, the wireless BH link to the WAB-MT, the WAB-gNB and, finally, the wireless access link to the UE. Given that the WAB-gNB and the WAB-MT may be served by two different core networks, the time that packets of UE PDU sessions spend travelling via the BH network may be significant.

Observation 2-1: The time that packets of UE PDU sessions spend travelling via the BH network may be significant.

Applying today’s tools to the WAB architecture, when the 5QI/QoS is configured for the PDU sessions of the UEs served by the WAB-gNB, the existence of wireless BH, and the QoS of the WAB-MT’s PDU sessions (that carry the PDU sessions of UEs served by the WAB-gNB), are not considered. Meanwhile, the BH-5GC, that sets the 5QI/QoS for WAB-MT’s PDU sessions, is unaware of the QoS configured for UE PDU sessions.

Observation 2-2: The BH-5GC that serves the WAB-MT and sets the 5QI/QoS for WAB-MT’s PDU sessions, is unaware of the QoS configured by the UE’s 5GC for UE PDU sessions, and vice versa.

This leads to inefficient resource planning in the BH network. For services with high performance requirement, such as XR, the WAB-gNB must compensate for the delay incurred at the BH part of the network, to ensure the end-to-end service quality.

This means that, in WAB scenarios, the end-to-end PDB consists of the following:

- The PDB of the UE’s 5GC.

- The PDB of the BH-5GC.

- The PDB of the BH-gNB.

- The time used to transmit from BH-gNB to WAB-MT/WAB-gNB.

- The PDB of the WAB-gNB.

Consequently, the “CN PDB”, from WAB-gNB point of view, i.e., the PDB for the path between the WAB-UPF and the WAB-gNB consists of the following:

- The PDB of the UE’s 5GC.

- The PDB of the BH-5GC.

- The PDB of the BH-gNB.

- The time used to transmit from BH-gNB to WAB-MT/WAB-gNB.

Hence, in case of WAB architecture, additional factors with respect to legacy architecture need to be considered. Namely, the BH-5GC PDB and the BH-gNB PDB, i.e., the time between the time the data comes out from WAB-UPF to the time when it reaches the WAB-gNB, need to be subtracted from the end-to-end PDB when calculating the 5G-AN PDB of the WAB-gNB (herein referred to as the WAB-gNB PDB). Otherwise, the WAB-gNB PDB will not be correctly calculated and the data for the PDU session may not be correctly scheduled.

In the simplest case, the WAB-gNB, should calculate the PDB of the WAB-gNB (the WAB counterpart of the legacy 5G-AN PDB) according to the following formula:

WAB-gNB PDB = (Total end-to-end PDB) – (UE’s 5GC PDB) – (BH PDU Session/QoS PDB)

Proposal 2: Calculation of the WAB-gNB PDB (i.e., the WAB counterpart of the 5G-AN PDB specified in TS 23.501) considers the PDB of the BH network.

Based on the above discussion, a pCR for TR 38.799 is provided in the Annex.

Proposal 3: Agree the pCR for TR 38.799 in the Annex.

# Conclusion

In this paper we discuss handling of reliability and packet delay budget for WAB-nodes. The following is observed and proposed:

Handling of BH link degradation

Observation 1-1: When BH link degradation occurs, avoiding NG connection failure is of highest priority.

Observation 1-2: In the WAB architecture, there are multiple overlapping control loops:

- Radio resources on the BH link are controlled by the BH-gNB.

- Radio resources on the access link are controlled by the WAB-gNB.

- PDU sessions of the WAB-MT are controlled by the BH-5GC.

- UE data traffic flow is controlled by the UE’s 5GC.

Observation 1-3: The NG protocol is designed for wired networks, while the NG connection between the WAB-gNB and WAB-5GC is carried over non-stationary wireless BH.

Observation 1-4: The BH-gNB is able to determine degradation of wireless BH, but it may not be able to determine which PDU session/radio resources on the BH link are used for carrying the NG-C traffic, and it may not be able to prevent the failure of WAB-gNB NG-C connection.

Observation 1-5: The WAB-gNB and UE’s AMF can take actions that can contribute to avoiding NG-C connection failure.

Proposal 1: The access and the BH network can coordinate for mitigating BH link degradation.

PDB handling

Observation 2-1: The time that packets of UE PDU sessions spend travelling via the BH network may be significant.

Observation 2-2: The BH-5GC that serves the WAB-MT and sets the 5QI/QoS for WAB-MT’s PDU sessions, is unaware of the QoS configured by the UE’s 5GC for UE PDU sessions, and vice versa.

Proposal 2: Calculation of the WAB-gNB PDB (i.e., the WAB counterpart of the 5G-AN PDB specified in TS 23.501) considers the PDB of the BH network.

The pCR

Proposal 3: Agree the pCR for TR 38.799 in the Annex.

# Annex: pCR for TR 38.799

-------------------------------------------Start of changes-------------------------------------------

## 4.4	Other

### 4.4.1	Handling of backhaul link degradation

In the WAB architecture, the PDU sessions of the UEs are carried inside PDU sessions of the WAB-MT over the radio resources of the BH link managed by the BH-gNB.

The access and the backhaul network can coordinate for mitigating BH link degradation.

### 4.4.1	Handling of packet delay budget

In WAB architecture, the end-to-end PDB for UE traffic consists of:

- The PDB of the UE’s 5GC.

- The PDB of the BH-5GC.

- The PDB of the BH-gNB.

- The time used to transmit from BH-gNB to WAB-MT/WAB-gNB.

- The PDB of the WAB-gNB.

When setting the 5QI/QoS for WAB-MT’s PDU sessions, the BH-5GC should be aware of the QoS configured for the UE’s PDU sessions. When setting the WAB-gNB PDB (i.e., the WAB counterpart of the 5G-AN PDB specified in TS 23.501 [x]), the WAB-gNB should be aware of the BH PDU session PDB.

-------------------------------------------End of changes-------------------------------------------