3GPP TSG-RAN WG3 Meeting #125	R3-244271

Maastricht, The Netherlands, August 19th – 23rd 2024

Agenda Item:	12.2

Source:	Ericsson

Title:	Reply to SA2 Questions Regarding WAB-MT Access Control and Additional ULI

Document for:	Agreement

# Introduction

In this paper we discuss the reply to an LS from SA2 received in R3-244019.

# Discussion

The LS from SA2 contains two issues, as discussed below.

## Issue 1: WAB-MT access control

With respect to Issue 1, the LS from SA2 states:

SA2 has discussed the access control (to restrict MWAB-UE accessing MWAB-gNB cell) for below two cases:

a)	MWAB-UE accessing any MWAB-gNB in IDLE and CONNECTED mode(documented as one candidate solution in clause 6.2.3.6 of TS 23.700-06); and

b)	MWAB-UE accessing MWAB-gNB belonging to same MWAB.

SA2 assume that those scenarios will be discussed by RAN WGs, and ask for RAN feedback.

Clauses 6.2.3.6-9 of TR 23.700-6 propose the framework for preventing a WAB-MT from connecting to a WAB-gNB:

According to TR 23.700-6, upon NGAP establishment, the WAB-gNB indicates to the AMF its WAB capability. Based on this information, the AMF may reject registration requests from WAB-MTs that try to access the WAB-gNB, or it may accept such registration request without authorizing the WAB-MT to act as a WAB-MT (i.e., the WAB-MT can only act as a normal UE).

Further, according to TR 23.700-6, upon NG-based HO of an WAB-MT, the AMF indicates to the target NG-RAN node (which may be a gNB or a WAB-gNB) that the WAB-MT is being handed over. In case the target NG-RAN node is a WAB-gNB, it will reject the HO request for the WAB-MT. According to the TR, upon Xn-based HO of an WAB-MT, the source gNB indicates to the target NG-RAN node that the handover is for an WAB-MT. If the target is a WAB-gNB, it will reject the HO. If the target is a regular gNB, it will accept the HO, for which it may not even need to understand the “WAB-MT” indication.

In our view, the above CN-based framework is not needed, since RAN3 is already discussing two RAN-based solutions for preventing the WAB-MTs to connect to a WAB-gNB:

- Option 1: Introduction of a “WAB-gNB cell” indication in the SIB broadcast by WAB-gNBs.

- Option 2: Introduction of a “WAB-MTs allowed to connect” indication in the SIB broadcast by “regular” gNBs.

Moreover, we see at least the following issues with the SA2 solution:

- According to clauses 6.2.3.7/8 of the TR 23.700-6, the WAB-MT needs to provide a NAS-level WAB-MT indication, but such an approach is not agreed yet in SA2 since the WAB-MT “authorization” is based on slice (for BH PDU sessions) info related to WAB-MT.

- In addition, clause 6.2.3.9 requests that, upon Xn-based HO, the source BH-gNB sends a WAB-MT indication to the target BH-gNB, but it is not clear how the source BH-gNB determines that the UE is a WAB-MT.

Proposal 1-1: Inform SA2 that RAN3 is working on a RAN-based solution for preventing a WAB-MT to connect to a WAB-gNB’s cell, and that a CN-based solution is not necessary.

First, we notice that Issue 1 b), i.e., preventing the WAB-MT from connecting to its co-located WAB-gNB, can be solved by implementation.

Proposal 1-2: Inform SA2 that Issue 1 b), i.e., preventing the WAB-MT from connecting to its co-located WAB-gNB, can be solved by implementation.

## Issue 2: Additional ULI format

With respect to issue 2, the LS from SA2 states:

SA2 has discussed and concluded that Additional ULI format will be determined by RAN WG and SA2 will align based on RAN feedback. Please refer to related conclusion in clause 8.5 of TR 23.700-06.

The issue at hand is how to formulate the ULI that is to be reported to the UE’s 5GC in case the WAB-gNB and WAB-MT connect to different PLMNs. In that respect, clause 8.5 of TR 23.700-6 considers 3 alternatives for additional ULI:

>>>>>>>>>>>>>Start of excerpt from TR 23.700-6<<<<<<<<<<<<<<<<

## 8.5	KI#5: Support of location services for UEs when MWAB(s) is involved Conclusion

The following aspects are taken as way forward during normative work:

-	Existing principles (e.g. the two options for LMF to retrieve the MWAB location either via NRPPa procedure or via GMLC) are re-used for UE location service when MWAB is involved as described in sol#11.

-	The MWAB-gNB provides to the AMF an Additional ULI. The Additional ULI is:

-	TAC/Cell ID of the cell serving the MWAB-UE if the PLMN serving the MWAB-UE and the PLMN broadcasted by the MWAB-gNB are the same PLMN.

-	If the PLMN serving the MWAB-UE and the PLMN broadcasted by the MWAB-gNB are not the same PLMN, which below option used for the Additional ULI will be determined in normative phase.

-	option 1: mapped TAC/Cell ID based on geo-location of the MWAB based on input from OAM.

-	option 2: geo-location of the MWAB.

-	option 3: TAC/Cell ID of the cell serving the MWAB-UE in other PLMN.

-	The AMF may provide the received Additional ULI to LMF in LCS request.

NOTE:	If option 2 is selected in normative phase, support of passing Additional ULI to LMF needs to be reviewed.

>>>>>>>>>>>>>End of excerpt from TR 23.700-6<<<<<<<<<<<<<<<<

In Option 1, the TAC and cell ID pertain to the WAB-gNB’s cell serving the UE. This TAC and cell ID may change, depending on the node’s location. The WAB-gNB is configured by the OAM with the mapping between the geo-location and the TAC/cell ID. Based on its location, the WAB-gNB determines the TAC/cell ID that it is supposed to use. The additional ULI format defined for mIAB (consisting of cell ID and TAC) can be reused to communicate the additional ULI to the AMF. However, it should be noted that, for mIAB, the TAC and cell ID pertain to the cell where the mIAB-MT is connected, while, in the WAB case, they pertain to the WAB-gNB’s cell serving the UE. Note that a similar principle for additional ULI has been adopted for NTN.

With respect to Option 2, we notice that specifying geo-location information of WAB-gNB as the additional ULI may be a substantial deviation from the current framework (which defines additional ULI as the cell ID/TAC), because, as of today, the geo-location information is not used by the LMFs. Note that, in addition to the AMF, the ULI should be provided to other entities in the CN as well, such as the SMF, charging system etc., meaning that the CN other nodes in addition to the LMF would be impacted. Therefore, this option should not be considered by RAN3.

Further, although Option 3 is identical to the additional ULI format currently used for UEs served by the mIAB-nodes, it is unclear how the UE’s 5GC should interpret the TAC/Cell ID from another PLMN. This format was indeed specified for mIAB, but the RAN WG work only considered intra-PLMN scenarios for mIAB, as opposed to WAB, where the WAB-MT and WAB-gNB may connect to different PLMNs.

Based on the above, we prefer Option 1, given that the additional ULI format from mIAB can be reused, and the impact incurred is limited to the mapping provided to the WAB-gNB by the OAM. We also note that Option 1 is line with the general principle of OAM providing location-dependent configurations to the WAB-gNB.

Proposal 2: Additional ULI for WAB consists of WAB-gNB cell’s TAC and Cell ID, which are determined by the WAB-gNB based on WAB-node’s geo-location.

# Conclusion

This paper discusses the reply LS on SA2 questions regarding FS_VMR_Ph2. The following is proposed:

Proposal 1-1: Inform SA2 that RAN3 is working on a RAN-based solution for preventing a WAB-MT to connect to a WAB-gNB’s cell, and that a CN-based solution is not necessary.

Proposal 1-2: Inform SA2 that Issue 1 b), i.e., preventing the WAB-MT from connecting to its co-located WAB-gNB, can be solved by implementation.

Proposal 2: Additional ULI for WAB consists of WAB-gNB cell’s TAC and Cell ID, which are determined by the WAB-gNB based on WAB-node’s geo-location.