3GPP TSG-RAN3 Meeting #125	R3-244272

# Maastrict, Netherland, 19th – 23rd August 2024

Agenda Item:	13.2

Source:	CATT

Title:	Discussion for Inter-CU LTM

Document for:	Decision

- Introduction

In this paper, we will analyse the potential issues in inter-CU LTM.

2. Discussion

2.1 Candidate Cell Preparation

Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM.

- Single vs. Parallel messages

In R18 LTM, the target cell is decided by CU and indicates to candidate DU one by one, by reusing the SpCell ID IE in UE CONTEXT SETUP REQUEST message, and adding an LTM indicator IE for identify the request. For R19, we prefer to follow the similar LTM designation. Source gNB chooses the potential candidate cell, and sending the request to candidate gNBs per candidate cell. For this parallel request, companies may concern about the signaling load is too high, from our understanding, candidate cell preparation always limited by the maximum of candidate cells and it’s still limited to 8 at defined in RAN2, the signaling load is still acceptable.

Considering the existence IEs in HANDOVER REQUEST message, the Target Cell Global ID IE can be reused for indicate LTM candidate cell.

Proposal 1: Source gNB requests candidate cell in a parallel way, e.g., requesting candidate cell one by one.

Proposal 2: Reuse the Target Cell Global ID IE to request LTM candidate cell.

- LTM modification

Different with F1 interface, there is no modification procedure in previous XnAP. So how to execute the second round interaction among source CU and candidate CU has to be discussed, possible way may reuse HO request message or introduce a new class one message.

We think both options are workable and we slightly prefer to introduce a new class 1 message. And is selelcted this way, seems we not need to introduce an explicated indicator in the LTM like F1AP as already have some LTM-dedicated IE to indicating this request is aiming for LTM, e.g., Request for RACH configuration

Proposal 3: Introduce a new class 1 message for second round interaction among source CU and candidate CU for inter-CU LTM.

Proposal 4: If introduced a new message for LTM modification, no need to introduce an explicated indicator in HO request message.

- Other IEs

Similar with intra-CU LTM, reference configuration, CSI-resource configuration, early sync information request and LTM configuration Mapping list should all be introduced in the HO request message for LTM candidate cell request.

For reference configuration, we think the current mechanism is sufficient. The purpose of introducing reference configuration/delta configuration is trying to transmission less information. However, multiple configurations is a kind of information to be transferred. At the worst situation, 8 candidate cells maybe allocated in 8 CUs, in that case, seems 8 reference configuration need to be Synchronization among candidate CUs.

Proposal 5: Only support one reference configuration at inter-CU LTM

For the CSI-Resource configuration, the source gNB-CU is responsible to collecting the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement on candidate cells. And source CU can include it in the first round or second round interactions as soon as CU generate the common CSI resource configuration.

Proposal 6: Source gNB-CU is responsible to collecting the configurations and generates the common CSI resource configuration for L1 measurement.

For the LTM configuration ID Mapping List, source CU also need generate it and inform other candidate CUs. Source CU can use the HO request message at the first round interaction and/or use new class 1 message in second round interaction.

Proposal 7: source CU needs generate the LTM configuration Mapping List and informs other candidate CUs.

For the early sync information request, for avoiding early sync confliction, candidate DU shall provide RACH configuration in per source DU in Rel-18. Considering the same logic, in inter-CU case, besides DU ID, CU ID should also be transferred in early sync information request.

Proposal 8: introduce gNB-CU ID and gNB-DU ID in HO request message for early sync information request.

2.2 LTM execution

- TA transfer

In last meeting, we discussed how to transfer TA information. As RAN2 still define 8 candidate cells in inter-CU LTM, seems the RACH resource insufficient is not heavier than intra-DU LTM. So we suggest to introducing a new class 2 message for TA transfer, including TA value along with the used RACH resource.

Proposal 9: Introduce a new class 2 message for transfer TA value along with the used RACH resource, e.g., TA information transfer.

- Synchronization among candidate cells

To support subsequent inter-CU LTM, at least the below information should below information should be synchronize among candidate cells.

- TCI state information list of all candidate cells

- Configuration ID list of all candidate cells

- Early sync information list of all candidate cell

- The data forwarding information should also be synchronized.

This procedure can be done by the second round interaction between source CU and candidate CUs, also can be done the after UE switch to the new DU. Whatever, this information synchronize should be finished no later than the UE access to the new serving cells.

Proposal 10: Synchronization among candidate cells can be triggered at the preparation phase to all candidate CUs, or triggered after UE execute LTM to the new serving DUs only.

2.3 LTM completion

- Path switch

The purpose of the Path Switch Request procedure is to establish a UE associated signalling connection to the 5GC. And usually, it should be finished as soon as UE access to the target gNB. In LTM, when UE access to the candidate DU, the candidate DU shall send a access success message to the CU among F1AP, which means UE had access to the target gNB. It is propoese target cell triggers the PATH SWITCH REQUEST to the CN once target CU received the access success message.

Proposal 11: start path switch after target CU received access success.

- EARLY STATUS transfer

Early data forwarding can be triggered before the Source gNB triggers a MAC CE Command to the UE to change cells, timing is left to implementation.

Last meeting, we agreed the timing of early data forwarding is left to implement. Accordingly, EARLY STATUS TRANSFER procedure also should supported to transfer the COUNT of the first downlink SDU that the source NG-RAN node forwards to the target NG-RAN node or the COUNT for discarding of already forwarded downlink SDUs for respective DRB.

Proposal 12: Support EARLY STATUS transfer for inter-CU LTM.

- Candidate cell cancel

A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Details are FFS.

For the candidate gNB triggered LTM candidate cells cancel, we support the cancellation of it owns prepared candidate cells. For the signalling designation, in CHO, we have the similar mechanism and introduce a CONDITIONAL HANDOVER CANCEL message for candidate gNB release its own candidate cells. This message can be reused in LTM for the same purpose.

Proposal 13: Reuse conditional handover cancel message for candidate cell triggered cancellation

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals:

Proposal 1: Source gNB requests candidate cell in a parallel way, e.g., requesting candidate cell one by one.

Proposal 2: Reuse the Target Cell Global ID IE to request LTM candidate cell.

Proposal 3: Introduce a new class 1 message for second round interaction among source CU and candidate CU for inter-CU LTM.

Proposal 4: If introduced a new message for LTM modification, no need to introduce an explicated indicator in HO request message.

Proposal 5: Only support one reference configuration at inter-CU LTM

Proposal 6: Source gNB-CU is responsible to collecting the configurations and generates the common CSI resource configuration for L1 measurement.

Proposal 7: Source CU needs generate the LTM configuration Mapping List and informs other candidate CUs.

Proposal 8: Introduce gNB-CU ID and gNB-DU ID in HO request message for early sync information request.

Proposal 9: Introduce a new class 2 message for transfer TA value along with the used RACH resource, e.g., TA information transfer.

Proposal 10: Synchronization among candidate cells can be triggered at the preparation phase to all candidate CUs, or triggered after UE execute LTM to the new serving DUs only.

Proposal 11: Start path switch after target CU received access success.

Proposal 12: Support EARLY STATUS transfer for inter-CU LTM.

Proposal 13: Reuse conditional handover cancel message for candidate cell triggered cancellation

4. Reference

[1]	RP-240299 Revised WID: NR mobility enhancements Phase 4

[2]	RAN3_124_agenda_EOM