3GPP TSG-WG3 Meeting # 125	R3-244273

# Maastricht, NL, 19th – 23rd Aug 2024

Agenda Item:	13.2

Source:	CATT

Title:	Common impact in RAN3 for security key update

Document for:	Decision

- Introduction

Security key issue had been discussed in RAN2 and SA3 in previous meeting, and still pend on SA3’s further conclusion [1][2]. In this paper, we analyse the solution proposed by RAN2, and considering the common impact of those solutions from RAN3 perspective.

2. Discussion

As for security key, in legacy procedure, the source gNB should send the { KgNB, NCC} pair to the target gNB. The KgNB will be used in UE access to the target gNB, and the NCC will be used in target gNB for deduce new key for next HO.

SA3’s will start further discussion based on RAN2 potential solutions, and mainly have four kinds of solutions list as below:

| Option | Interface behavior |
| --- | --- |
| Option 1a: NCC value to use is included in MAC CE cell switch command upon inter-CU LTM execution | After cell switch, new source gNB should update the previous {KgNB, NCC} to other candidate cell/target cell. |
| Option 1b:UE is preconfigured with a NCC value list, and index of NCC is included in MAC CE cell switch command |  |
| Option 2a/b: Follow SCPAC key update mechanism, i.e., UE is preconfigured with a NCC list per CU, and UE choses one to use upon inter-CU LTM execution | After cell switch, new source gNB should update the {KgNB, NCC} to other candidate cell/target cell. New source gNB need interaction with AMF to get NH. |
| Option 3a/b: Initial or list of NCC is preconfigured to UE and UE determines the NCC to use by itself upon inter-CU LTM execution | After cell switch, new source gNB should update the previous {KgNB, NCC} to other candidate cell/target cell. |
| Option 4: After every inter-CU LTM cell switch execution, the UE is provided via RRC signalling with the NCC value to be used by the UE for key derivation at the next inter-CU LTM cell switch. | After cell switch, new source gNB should update the previous {KgNB, NCC} to other candidate cell/target cell. |

Based on the above analysis, one common part for RAN3 is the interaction between the new source gNB-CU and other candidate gNB-CUs to update the key. Also, there are two occasions:

- New source cell update the key for other candidate cells once it turn to source cell, by this way, new gNB needs to send updated key to all the candidate gNBs.

- New source cell only update the key for the target cell before sending cell switch command, by this way, new source gNB can only update key for the target gNB;

Proposal 1: For subsequent LTM, new source gNB need to update the {KgNB, NCC} to other candidate cells/target cell and discuss the two occasions, message still FFS:

- New source gNB update the key for other candidate gNBs once it turn to source cell.

- New source gNB only update the key for the target gNB before sending cell switch command.

3. Conclusion

Proposal 1: For subsequent LTM, new source gNB need to update the {KgNB, NCC} to other candidate cells/target cell and discuss the two occasions, message still FFS:

- New source gNB update the key for other candidate gNBs once it turn to source cell.

- New source gNB only update the key for the target gNB before sending cell switch command.

4. Reference

[1]	R3-243114 LS on security handling for inter-CU LTM in non-DC cases

[2]	S3-241773 LS on security handling for inter-CU LTM in non-DC cases