3GPP TSG-WG3 Meeting # 125	R3-244275

# Maastricht, NL, 19th – 23rd Aug 2024

Agenda Item:	9.2

Source:	CATT

Title:	Discussion on supporting Tag ID Pointer for LTM

Document for:	Decision

- Introduction

In RAN2#126 meeting, RAN2 decided to support LTM candidate cell configured with inter-cell mTRP with 2TA. And given the below assumption:

In case of early RACH across DUs with intra-cell mTRP with 2TA, R2 assumes the target DU should forward the TAG information (e.g. tag-Id-ptr) to the source DU.

Based on above, In RAN3#124 meeting, we introduce the Tag ID Pointer IE in the CU-DU/DU-CU TA INFORMATION TRANSFER message to indicate the TAG for MIMO two TAs.

In this paper, we will discuss the feasibility of transferring Tag ID Pointer IE from RAN3 prespective.

- Discussion

The tag-Id-ptr defined in RAN2 is per UE metric, which means, for the same TRP, for different UE, the candidate cell may assign different tag-Id-ptr.

tag-Id-ptr It indicates the TAG that is associated with this TCI state, value n0 means the TCI state associate with the TAG indicated by tag-Id, value n1 means this TCI state associated with the TAG indicated by tag2-Id. The tag-Id-ptr refers to the TAG of the serving cell where the TCI state is applied.

For example, for the UE1, tag-Id-ptr of TCI states in TRP1 may assign n0 and associate with the TAG indicate by tag-id, tag-Id-ptr of TCI states in TRP2 is n1 and associate with the TAG indicate by tag2-Id. But for UE2, tag-Id-ptr of TCI state in TRP 1 is n1 while tag-Id-ptr of TCI state in TRP2 is n0.

If candidate cell assign the UE as above, candidate cell may have trouble in transferring the tag-Id-pt. When the candidate cell receives the early sync from TRP1, due to can’t identify the UE, candidate cell can not know to providing which tag-Id-ptr in the TA transfer message, e.g., if UE1 access, the tag-Id-ptr should be n0, while if UE2 access, the tag-Id-ptr should be n1.

Observation 1: For the same TRP, candidate cell may assign different tag-Id-ptr for different UE.

It is worth mentioning that this kind of assignment is normal from network part, especially for serving cells. Assuming there are two TRPs in the serving cell, UE1 is access in TRP1, thus, for UE1, the TRP1 is default as n0; UE2 is access in TRP2, thus, for UE2, the TRP2 is default as n0. When LTM triggered and the serving cell turns to be a candidate cell, the tag-Id-ptr assigned for UE1 and UE2 are different if without any limitation.

2.1 Alt 1: candidate DU identify the tag-id-ptr

If depend on candidate DU to identify the tag-Id-ptr, it is required for the same TRP, candidate cell shall always assign the same Tag-Id-ptr for different UE, e.g., for all the UEs, the tag-Id-ptr of TRP1 is n0, and TRP2 is n1. By this way, the candidate cell only need to know the receiving TRP, and then can decide the Tag-Id-ptr.

Proposal 1: If go for Alt1, for same TRP, candidate DU shall ensure to configure same tag-Id-ptr for each UE at candidate cell preparation phase.

Normative impacts:

Add a note to restrict DU allocated same tag-Id-ptr for different UE.

Proposal 2: The normative impact for Alt 1:

Adding a note in stage 2 to restrict for the same TRP, DU shall allocate same tag-Id-ptr for different UE.

2.2 Alt 2: source DU deduce the tag-Id-ptr

If depend on source DU to deduce the tag-Id-ptr, it may deduce it with the help of associated SSB index. A possible way list as below

Source DU trigger early sync to a UE by PDCCH including SSB index. According to the used SSB index, source CU filtrating TCI states associate to this SSB index. As the TCI state associated with the trp-id ptr IE, the source DU can deduce the trp-id-ptr implicitly. The deduction procedure has shown as below.

Fig.1 deduce trp-id-ptr from RACH configuration

It can be seen, if the TCI state only reference to the SSB on its own TRP(Opt1), the deduction procedure is workable. However, the TCI state can also reference to SSB on other TRP(Opt2), if that happened, source DU can not deduce the tag-id-ptr according to the TCI state ID.As shown in fig.2, source DU filtrate TCI States associated to SSB1 and found TCI state 1-3, while TCI state 1-2 is belong to TRP 1 and tag-id-ptr should be n0, and TCI state is belong to TRP2 and tag-id-ptr should be n1.

Fig.2 TCI state may associate with SSB belong to other TRP

Observation 2: It is possible for a TCI state to associate with SSB which belongs to other TRP. And if that happened, source DU can’t deduce tag-id-ptr from the TCI state.

Proposal 3: If go for Alt2, TCI state should always associate with SSB which belong to its own TRP. Check with RAN1 and RAN2.

Normative impacts:

- Ask RAN1 and RAN2 for further confirm and clarification.

- If confirmed source DU can deduce tag-id-ptr, remove the Tag ID Pointer IE in the CU-DU/DU-CU TA INFORMATION TRANSFER message in RAN3.

Proposal 4: The normative impact for Alt 2:

- Ask RAN1 and RAN2 for further confirm and clarification about deduce tag-Id-ptr by source DU.

- If confirmed source DU can deduce tag-Id-ptr, removing the Tag ID Pointer IE in the CU-DU/DU-CU TA INFORMATION TRANSFER message in RAN3.

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals:

Observation 1: For the same TRP, candidate cell may assign different tag-Id-ptr for different UE.

Proposal 1: If go for Alt1, for same TRP, candidate DU shall ensure to configure same tag-Id-ptr for each UE at candidate cell preparation phase.

Proposal 2: The normative impact for Alt 1:

Adding a note in stage 2 to restrict for the same TRP, DU shall allocate same tag-Id-ptr for different UE.

Observation 2: It is possible for a TCI state to associate with SSB which belongs to other TRP. And if that happened, source DU can’t deduce tag-id-ptr from the TCI state.

Proposal 3: If go for Alt2, TCI state should always associate with SSB which belong to its own TRP. Check with RAN1 and RAN2.

Proposal 4: The normative impact for Alt 2:

- Ask RAN1 and RAN2 for further confirm and clarification about deduce tag-Id-ptr by source DU.

- If confirmed source DU can deduce tag-Id-ptr, removing the Tag ID Pointer IE in the CU-DU/DU-CU TA INFORMATION TRANSFER message in RAN3.

4. Reference

[1]	RAN2_126_Meeting_Report

[2]	R3-243689, Discussion on Remaining issues for LTM, Samsung

Annex for Alt1:

CR for TS 38.401

#### 8.2.1.5	Inter-gNB-DU LTM

This procedure is used for the case when the UE moves from one gNB-DU to another gNB-DU within the same gNB-CU during NR operation for LTM. Figure 8.2.1.5-1 shows the inter-gNB-DU LTM procedure for intra-NR.

Figure 8.2.1.5-1: Inter-gNB-DU LTM

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB-DU containing measurements of neighbouring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2.	The gNB-CU determines to initiate LTM configuration.

3.	The gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU(s) for each candidate cell, containing one candidate cell ID and the CSI resource configuration for subsequent LTM. The gNB-CU may provide the LTM configuration ID mapping list to the candidate gNB-DU(s). The gNB-CU may request PRACH resources from the candidate gNB-DU(s). The gNB-CU may request the candidate gNB-DU(s) to provide the lower layer configuration for the purpose of generating the reference configuration or provide the lower layer reference configuration to the candidate gNB-DU(s).

4.	If the candidate gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configurations for the accepted target candidate cell.

NOTE 1:	The CU-initiated UE Context Modification procedure may be initiated for preparing candidate cells in the source gNB-DU as specified in step 3 and 4 in 8.2.1.4 Intra-gNB-DU LTM.

5.	The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU including the information related to early sync and the LTM configuration ID mapping list for the accepted target candidate cell(s). The gNB-CU may send the updated CSI resource configuration to the source gNB-DU.6.	The source gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which includes an updated lower layer configuration, e.g., containing the updated CSI report configuration of the source cell.

7.	The gNB-CU may send a UE CONTEXT MODIFICATION REQUEST message to the candidate gNB-DU(s) containing the information for subsequent LTM or for updating the configurations of candidate cells. The gNB-CU may also provide the lower layer part of the reference configuration to the candidate gNB-DU(s).

8.	The candidate gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message including the updated lower layer configuration (e.g., the updated CSI report configuration).

NOTE 2: Step 7 may also be triggered after step 19, or after step 22 by implementation for subsequent LTM.

9.	The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration.

10.	The source gNB-DU forwards the received RRCReconfiguration message to the UE.

11.	The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

12.	The source gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

13.	Early synchronization to the candidate cell(s) may be performed as specified in TS 38.300 [2].

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Start of change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

14.The candidate gNB-DU sends a DU-CU TA INFORMATION TRANSFER message to the gNB-CU, which includes the TA value, the associated PRACH resource information, tag ID pointer information if two TAs supported, and etc.

Note 3: The candidate DU shall configure same Tag ID Pointer for each UE for the same TRP at candidate cell preparation phase.

15.	The gNB-CU forwards the TA value, and the associated PRACH resource information to the source gNB-DU in the CU-DU TA INFORMATION TRANSFER message.

16.	The UE sends the L1 measurement result to the source gNB-DU.

17.	The source gNB-DU decides to execute LTM to a target cell.

18.	The source gNB-DU sends the Cell Switch Command to the UE.

19.	The source gNB-DU sends the DU-CU CELL SWITCH NOTIFICATION message to the gNB-CU to indicate the initiation of the Cell Switch Command to the UE including the target cell ID, the TCI state ID(s), tag ID pointer information if two TAs supported, and the TA values for subsequent LTM.

20.	The gNB-CU forwards the target cell ID, the TCI state ID(s), and the TA values for subsequent LTM to the target gNB-DU in the CU-DU CELL SWITCH NOTIFICATION message.

21.	The target gNB-DU detects the UE access as specified in TS 38.300 [2].

22.	The target gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target cell ID.

23.	The UE sends an RRCReconfigurationComplete message to the target gNB-DU.

24.	The target gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

25.	The gNB-CU may send the UE CONTEXT RELEASE COMMAND message to the source gNB-DU to release the resources of prepared cells.

26.	The source gNB-DU responds with a UE CONTEXT RELEASE COMPLETE message.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

Annex for Alt2:

LS to RAN1&RAN2

3GPP TSG-RAN3 Meeting #125	R3-24xxxx

# Maastrict, Netherland, 19th – 23rd August 2024

Title:	[DRAFT] LS on tag-Id-ptr transfer in inter-DU LTM

Release:	Release 18

Work Item:	NR_Mob_enh2-Core

Source:	CATT (will be RAN3)

To:	RAN2, RAN1

Cc:

Contact Person:

#### Name:	Jiaying Sun

Tel. Number:

E-mail Address:	sunjiaying at cat dot cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1	Overall description

RAN2 has discussed LTM candidate cell configured with inter-cell mTRP with 2TA. And in case of early RACH across DUs with intra-cell mTRP with 2TA, RAN2 assumes the target gNB-DU should forward the TAG information (e.g. tag-Id-ptr) to the source gNB-DU.

For this issue, RAN3 wants to confirm the following questions and understanding:

Q1: As tag-Id-ptr is configured per UE, RAN3 understands for the same TRP, candidate cell can configure different tag-Id-ptr for different UE, is this correct understanding?

Q2: If the answer of Q1 is yes, from RAN3 perspective, target gNB-DU can not deduce the tag-Id-ptr as candidate gNB-DU can not identify UE during early RACH. RAN3 would like to ask RAN1 and RAN2 whether source gNB-DU can deduce the tag-Id-ptr (TRP information) by the early sync information provided by candidate cells before trigger early sync to the UE. Whether existing any other constrictions for source gNB-DU to realized deduction?

2	Actions

Action to RAN2 and RAN1:

RAN3 respectfully requests RAN2 and RAN1 to answer above questions.

3	Dates of next TSG-RAN WG3 meetings

TSG-RAN WG3 Meeting#125-bis		14 - 18 Oct 2024		 China, HeFei

TSG-RAN WG3 Meeting#126			18 - 22 Nov 2024		Orlando, US