3GPP TSG-WG3 Meeting # 125	R3-244277

# Maastricht, NL, 19th – 23th Aug 2024

Agenda Item:	15.2

Source:	CATT

Title:	Discussion about UE attachment in Store & Forward

Document for:	Decision

- Introduction

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3][1]

In this paper, we provide our initial discussion about S&F scenario, and start with the attachment procedure.

- Discussion

S&F operation is designed for delay-tolerant, non-real-time IoT NTN services in the area where can be visited by satellite but without gateway infrastructure. The most typical case is intelligent sensor allocated in the mid-sea or the remote area. Usually, the common requirement for such UEs is uploading measurement results to the network regularly for monitoring.

In RAN2 previous discussion, they assumed the S&F satellite operation is common for NB-IoT and eMTC, and it applied to both CP and UP solution. To align among RAN WG and to save RAN3 time, we suggest following RAN2’s assumption as a baseline, and further considering RAN3 impact.

Proposal 1：Follow RAN2 assumption in RAN3, S&F satellite operation is common for NB-IoT and eMTC, and applied to both CP and UP solution.

2.1 UE pre-attach in legacy TN/NTN network

Consider the service characteristic, an effective method is pre-register the UE before dropping them to remote area. Pre-attachment is benefit especially for Control Plane CIoT. With pre-attachment, UE can trigger the INITIAL UE MESSAGE message to the satellite regardless the implementation on the satellite, and finish the measurement results reporting.

Proposal 2: For S&F scenario, UE can be pre-registered by TN/NTN network by implementation.

  - UE attach in S&F operation

2.2.1 eNB only

For the eNB only case, SA2 exclude the eNB only on-board case. However, from the perspective of implementation, eNB only on-board is the easiest way to realized. eNB only on board can further divide to single satellite serving UE and multiple satellite serving UE.

In single satellite case, it is assumed that there is only one satellite serving the UE for S&F operation, this way can support delay-tolerant communication service with minimum impact on both RAN and CN.

For the UE attachment procedure, eNB also need to inform the UE attachment is not finished due to the S&F, and both UE and satellite need to solve the intermediate information until the attachment is finished.

After attachment, the UE context suspend and resume procedure may be used for S&F scenario. When the satellite leaves the CN, the eNB can send UE CONTEXT SUSPEND REQUEST to the MME, and data related to the S1AP association, UE Context and bearer context, necessary to resume the connection is kept in the eNodeB, UE and the MME.

In multiple satellite case, the security issue and UE identify issue should be further study.

Observation 1：eNB only on-board is the easiest way for implementation.

Proposal 3: Evaluate the feasibility of supporting eNB only on-board case, considering single satellite as start point.

2.2.2 Split MMEs

To simply the scenario, we assume that eNB1 and MME1-onboard allocated on satellite 1, eNB2 and MME2-onboard allocated on satellite2, while all the MME-grounds and HSS can interaction with each other. Based on the conclusions given in TR23.700[2], the procedure can be summarized as below, and we list potential open issues to be discussed in RAN3.

First round interaction:

Step 0: eNB shall inform UE the satellite serviced in S&F operation.

Step 1-3: The UE sends attach request to eNB when service link is available, and the eNB/MME1 on-board stores it.

Step 4-5:MME1 on-board reject UE attach request, and sends Attach Reject message to the UE, including new indicator, wait timer, and optionally including the list of satellite IDs which indicate satellite UE can re-attempt in the future.

- eNB inform UE satellite serviced in S&F operation

The trigger for the eNB to broadcast support of S&F operation is based on the decision of RAN WGs.

For this FFS left by SA2, it is RAN2 scope to discuss how to inform UE the satellite in S&F operation, and RAN2 had already reach the below agreement, e.g., eNB broadcast support S&F operation as part of system information.

S&F indication can be provided by SIB (FFS on the details).

RAN2 assumes that no NAS indication is needed

For how the trigger eNB broadcasting S&F operation, in this scenario, eNB and MME on-board allocated on the same satellite, both eNB and MME should have the knowledge of whether the feeder link is available or not. Thus, there is no need to transfer the S&F operation between the MME on-board and the eNB. The eNB support S&F can start broadcast S&F operation as soon as the feeder link is not available by implementation.

Proposal 4: For the split MME scenario, as MME and eNB are co-allocated, no need signalling enhancement between MME and eNB for transfer the S&F operation.

- S&F indicator and wait timer

S&F indicator is used for informing UE this attachment cannot be complete because of the S&F operation, while the wait timer is used for indicating the time UE should wait before re-attempt. Both the S&F indicator and wait timer are generating by MME and sending to UE, as the two IE will only be used in UE, it is reasonable to include both in NAS PDU in DOWNLINK NAS TRANSPORT message from MME to eNB, and sending it to UE directly.

Proposal 5: Including S&F indicator and wait timer in NAS PDU in DOWNLINK NAS TRANSPORT message from MME to eNB.

- List of Satellite IDs

List of satellite IDs is used for indicating over which UE may re-attempt the attach procedure after the wait timer expires. Different with S&F indicator and wait timer, this information is also useful in eNB side. Usually, UE will deduce the re-attach opportunity by the list of satellite from MME and the ephemeris information from eNB. However, in current mechanism, the number of ephemeris in the system information is limited. Without knowing the satellite list configured by the MME, the eNB doesn’t know which ephemeris need to broadcast for the UE.

Proposal 6: MME transfer the satellite ID list to the eNB explicitly for assisting ephemeris broadcast. Detail message FFS.

Second round interaction:

Step 6-7: When the feeder link is available, MME1 on-board forward the stored attach request to the MME1 ground and HSS. MME fetches the authentication vector and other details from HSS following current Authentication and security procedures.

Step 8-10: HSS sends feedback to a suitable satellite, e.g., satellite 2. And the satellite 2 stores the relative information and waiting for service link available.

Step11-13: UE re-attempt attach according to the wait timer and the satellite ID list. As satellite already obtained the Authentication and Security information, the authentication and security between UE and CN can be finished. During the re-attempt, the MME may also provide a list of Satellite IDs and a wait-timer, which is similar with step 4,5. The MME may also indicate to UE the estimated delivery time in NAS message.

Step 14-16: Satellite 2 wait for the feeder link available, and send the final update request to the HSS. The HSS consider the UE as register.

Estimated delivery time

The estimated delivery time is the estimated time to send data from the UE to Gateway, and only will be used in UE side. This IE can be included in NAS PDU in DOWNLINK NAS TRANSPORT message from MME to eNB.

Proposal 7: Estimated delivery time can be included in NAS PDU in DOWNLINK NAS TRANSPORT message from MME to eNB.

2.2.3 Whole CN

For the full CN on-board the satellite, SA2 list the following normative impacts.

- Store and forward is only supported by EPS. - Optionally the MME provides the UE with a S&F monitoring list of satellites IDs, during attach/TAU. The UE uses the satellites in the S&F monitoring list for MO/MT data/signalling with the CN. The S&F monitoring list can be determination by the CN. How network determines the S&F monitoring list is outside the scope of 3GPP in this release of specification. NOTE 8: The S&F monitoring list may assist the UE in retrieving MT data. - The UE needs to be aware that a satellite supports S&F mode. NOTE 9: How the UE is aware that a satellite supports S&F mode of operation depends on RAN. - A UE may be rejected if the satellite cannot support the UE at this time. The attach reject may provide a timer for the time the UE should wait before reattempting and S&F monitoring list which the UE can attempt attach again.

At initial attach, UE can trigger attach procedure as required, e.g, MO data transfer. As full CN and eNB already on-board, the initial attach procedure is similar with legacy IoT NTN. After UE attach, the MME may provide the monitoring list to UE. After data transfer, UE shall detach with that satellite.

UE stores the monitoring list, once the satellite in the monitoring list appeared, UE shall trigger an attachment for retrieving MT date. Besides, UE also could wait an indication form application which indicate MT data is arrived and UE should trigger attach, which is out of RAN3 scope.

If the UE be rejected due to satellite cannot support the UE at this time, the attach reject may provide a timer for the time the UE should wait before re-attempting and S&F monitoring list which the UE can attempt attach again.

For the S&F monitoring list of satellites IDs and timer for the time the UE should wait before reattempting, both of them can follow the similar way with split MME. And as for how let UE aware that satellite supports S&F mode, RAN2’s conclusion still workable, and no need for any signalling enhancement for transfer the S&F mode between CN and eNB as they are co-allocation.

Proposal 8: For the whole CN scenario, as MME and eNB are co-allocated, no need signalling enhancement between MME and eNB for transfer the S&F operation.

Proposal 9: For the whole CN scenario, including wait timer in NAS PDU in DOWNLINK NAS TRANSPORT message from MME to eNB.

Proposal 10: For the whole CN scenario, MME transfer the S&F monitoring list of satellites IDs to the eNB explicitly for assisting ephemeris broadcast. Detail message FFS.

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals:

Proposal 1：Follow RAN2 assumption in RAN3, S&F satellite operation is common for NB-IoT and eMTC, and applied to both CP and UP solution.

Proposal 2: For S&F scenario, UE can be pre-registered by TN/NTN network by implementation.

Observation 1：eNB only on-board is the easiest way for implementation.

Proposal 3: Evaluate the feasibility of supporting eNB only on-board case, considering single satellite as start point.

Proposal 4: For the split MME scenario, as MME and eNB are co-allocated, no need signalling enhancement between MME and eNB for transfer the S&F operation.

Proposal 5: Including S&F indicator and wait timer in NAS PDU in DOWNLINK NAS TRANSPORT message from MME to eNB.

Proposal 6: MME transfer the satellite ID list to the eNB explicitly for assisting ephemeris broadcast. Detail message FFS.

Proposal 7: Estimated delivery time can be included in NAS PDU in DOWNLINK NAS TRANSPORT message from MME to eNB.

Proposal 8: For the whole CN scenario, as MME and eNB are co-allocated, no need signalling enhancement between MME and eNB for transfer the S&F operation.

Proposal 9: For the whole CN scenario, including wait timer in NAS PDU in DOWNLINK NAS TRANSPORT message from MME to eNB.

Proposal 10: For the whole CN scenario, MME transfer the S&F monitoring list of satellites IDs to the eNB explicitly for assisting ephemeris broadcast. Detail message FFS.

4. Reference

[1]	RP-241624 Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3.

[2]	TR 23.700 Study on integration of satellite components in the 5G architecture.