3GPP TSG-RAN WG3 Meeting #125	R3-244278

Maastricht, NL, 19 -23 Aug, 2024

Title:	(TP for SON BLCR for 38.300, 38.401, 38.473, 37.340) MRO enhancement for R18 mobility mechanisms

Agenda Item:	10.2

Source:	Huawei

Document for:	discussion

# 1	Introduction

In this paper, we discuss potential MRO enhancements for R18 mobility mechanisms.

# 2	Discussion

## 2.1	MRO for LTM

The agreements in last meeting were:

RAN3 to prioritize MCG LTM over SCG LTM.

RAN3 takes RAN2#125-bis agreement on MRO for LTM scenarios as baseline for further study.

For failures due to wrong selection of candidate LTM cell, CU is in charge of root cause analysis and performs optimization.

For failures due to inappropriate cell switch triggering (e.g. wrong cell selection at cell switch, wrong cell switch timing, …) source DU performs optimization.

For failures due to inappropriate cell switch triggering (e.g. wrong cell selection at cell switch, wrong cell switch timing, …) FFS if CU or DU performs root cause analysis.

For stage-2, FFS if existing problem definitions (Connection failure due to intra-system mobility) can be reused or if a new chapter needs to be created.

RAN3 to discuss enhancements related to the information of RACH-less access and RACH-based access from UE related to MRO for LTM.

### 2.3.1	Failure scenarios

For the potential failure cases, RAN2#125bis made the following agreements on MRO for LTM:

For LTM MRO, RAN2 considers the following three connection failure cases:

- -	Too late LTM

- -	Too early LTM

- -	LTM to wrong cell

For too late LTM, the following sub-cases are considered but we may down prioritize later (not limiting):

- -	Case 1a: the UE detects RLF in source cell after receiving LTM candidate configurations and performs reestablishment procedure.

- -	Case 1b: the UE detects RLF in source cell after receiving LTM candidate configurations, selects an LTM candidate cell, detects HOF with the selected LTM cell.

- -	Case 1c: the UE detects RLF in source cell after receiving LTM candidate configurations, and successfully completes LTM execution with the selected LTM candidate cell.

For too early LTM, the following sub-cases are considered but we may down prioritize later (not limiting):

- -	Case 2a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure with the source cell.

- -	Case 2b: the UE detects HOF/RLF in the LTM target cell, selects the source cell which is also an LTM candidate cell, detects HOF with the source cell, and performs reestablishment procedure.

- -	Case 2c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected source cell which is also an LTM candidate cell.

LTM to wrong cell, the following sub-cases are considered but we may down prioritize later (not limiting):

- -	Case 3a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure with the source cell.

- -	Case 3b: the UE detects HOF/RLF in the LTM target cell, selects an LTM candidate cell which is different from the source or target one, detects HOF with the selected LTM candidate cell, and performs reestablishment procedure.

- -	Case 3c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected LTM candidate cell which is different from the source or target one.

RAN2 considers SHR, RA report and RLF for MCG LTM SON.

RAN2 will start work on MCG LTM.

RAN2#126 meeting has agreed to correct the definition of the above case 3a as below:

We correct the definition of scenario 3a as follows: Case 3a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure in a cell other than the source cell and the target cell.

The following sub-cases are illustrated as below:

Fig.1 too late LTM

Fig.2 too early LTM

Fig.3 LTM to wrong cell

The division of failure cases for LTM is similar as those for handover, since the LTM is built and augmented on top of L3 handover. For handovers, the SON features including the radio link failure (RLF) report and successful handover report (SHR) provides the UE feedback information. The RLF and SHR could be augmented to also provide useful feedback information to address the LTM mobility issues.

Upon reception of RLF report addressing LTM failure cases, potential optimizations performed by the gNB to avoid the failure cases needs to be investigated. To facilitate the further discussion, we illustrate the steps during the LTM procedure to have a retro on the design of Rel-18 LTM feature, compared to those during legacy L3 handover.

UE reconfiguration

Early DL Sync

Early UL Sync

Cell Switch

UE reconfiguration

DL Sync

UL Sync

Beam management

1. LTM handover preparation

(candidate Cell config)

2. L1 meas

report

3. Candidate TCI state activation MAC CE

4. PDCCH

order

5. preamble

6. HO command MAC CE

7. HO complete

1. handover preparation

(candidate Cell config)

2. preamble

3. RAR

4. TCI

Legacy L3 HO

possible LTM procedure-1

RACH-less access based on network TA

UE reconfiguration

Early DL Sync

Early UL Sync

Cell Switch

1. LTM handover preparation

(candidate Cell config)

2. L1 meas

report

3. Candidate TCI state activation MAC CE

5. HO command MAC CE

6. HO complete

possible LTM procedure-2

RACH-less access based on UE measured TA

4. UE based TA meas

UE reconfiguration

Early DL Sync

Cell Switch including UL sync

1. LTM handover preparation

(candidate Cell config)

2. L1 meas

report

3. Candidate TCI state activation MAC CE

4. HO command MAC CE

7. HO complete

possible LTM procedure-3

RACH-based access

5. preamble

6. RAR

Figure 4. Steps during L3 handover and 3 possible LTM approaches

We could see that the LTM is still a network triggered handover. Similar as the L3 handover, the CU provides the candidate cell set based on L3 measurement report. However, during the LTM procedure, the DU makes the handover decision to select a target cell based on UE assisted L1 measurement reporting, in contrary to that the CU makes the handover decision to select a target cell based on UE assisted L3 measurement reporting. We identify that there is a role split between CU and DU for selection of target cell. Firstly, the CU provides the candidate Cell set to the source DU. Secondly, based on the L1 measurement report, the source DU select one from the candidate Set as the target Cell and decides the cell switch timing. So, both the CU and the source DU should take part in the optimization of selecting a proper target cell.

From the above analysis, both the CU and the source DU need to receive the RLF report. According to the current SON mechanism, the receiving gNB, which fetches the RLF/SHR report from the UE, forwards the RLF/SHR report to the last serving gNB CU. In order for DU to participate in the MRO analysis, the CU shall further make the RLF/SHR report available to the DU. The existing procedure (i.e., Access and Mobility Indication) over F1 could be reused for LTM.

The CU shall further make the RLF/SHR report available to the source DU via the existing procedure (i.e., Access and Mobility Indication) over F1.

Depending on the failure scenario, the last serving cell may belong to the source or target DUfor a mobility failure event. As discussed before, we assume that we at least need to send the information to the sourceDU. Therefore, the CU may also need to make the RLF/SON report available to a DU other than the last serving DU in case that the last serving DU is a target DU. The possible approaches for role split between the CU, the source DU and target DU are illustrated as below:

alt.1 the CU makes initial analysis and forwards to the corresponding DU

UE

Receiving gNB

Last serving gNB

Last serving DU

CU

other DU

1. RLF report

2. Failure Indication

alt.2 the CU distributes the RLF to involved DUs for analysis

3. Distribute the RLF report to involved DUs

alt.3 the CU first forwards to the last serving DU and then the other DU

3. the RLF report to the last serving DU

4. Failure decline

5. the RLF report to the other DU

4. Distribute the RLF report to the corresponding DU

3. Analyze and decide the corresponding DU

Figure 4. possible approaches for RLF report forwarding

Regarding the miscellaneous failure cases, we propose to discuss 3 potential RLF forwarding mechanisms between the CU and the DUs:

- Alt.1: Assume that the CU is capable of classifying the error type (too late handover, too early handover and handover to wrong cell). Thus, the CU makes the initial analysis and decide the corresponding DU to forward the RLF report to. The DU is still responsible for performing the optimisation of the mobility parameters used for LTM in this node, e.g., adjust the selection criteria of target cell for an optimal target cell and timing for executing the cell switch.

- Alt.2: the CU always forwards the RLF report to the involved DUs, i.e., the source DU and the target DU(s). The involved DUs makes analysis respectively.

- Alt.3: the CU forwards the RLF report first to the last serving DU. The last serving DU makes initial analysis and may reply by indicating that the other DU causing the failure. Then the CU further forwards the RLF report to the other DU.

Alternative 3 is most aligned with the legacy principle. But for LTM there are some differences that motivates the consideration of using a different option:

- unlike legacy, there is a common node (gNB-CU) that is aware of the mobility performed in the DUs.

- there is no interface between the gNB-DUs, meaning that any message between them needs to be forwarded by the gNB-CU.

Based on this, we would prefer alternative 1 or 2. Alternative 1 would guarantee a consistent handling of each event, whereas alternative 2 would rely on the two gNB-DUs reaching the same conclusion based on the report from the same event. Hence, we would prefer to go for alternative 1. One crucial part of information needed for the node to for distinguish the exact error type is the T_storeUEcontext. This is currently configured to the CU, so alterative 1 would have enough information to make the analysis.

When gNB-CU receives an RLF report from another node, the gNB-CU analyse the report and determines the failure type to determine which of the involved gNB-DUs node should receive the information.

In order to enable the forwarding of information, the gNB receiving the RLF report from another node needs to understand whether the failure report is due to a legacy mobility or an LTM mobility event. This is already agreed in RAN2.

For failures shortly after successful handover, the source cell need to identify the UE context based on the C-RNTI used in the source to perform the MRO analysis. It is assumed that the gNB-CU that supports this MRO feature will be able to store the association between the C-RNTI used in the target cell (last serving cell) and the corresponding C-RNTI used in the source cell. gNB-CU translate the C-RNTI in the target cell to the C-RNTI used in the source cell and convey it outside the RLF report when forwarding the information to the source gNB-DU.

Additional information regarding source cell information and C-RNTI used in the source cell is added to the existing procedure (i.e., Access and Mobility Indication) over F1 for failure cases with RLF shortly after successful HO.

Although the failure scenarios are similar to the legacy, the detection mechanism is very different from the legacy. One example is the forwarding mechanism. In addition to this, the mechanism between CU/DU should be described in 38.401. Therefore, it is proposed to create a new section in stage 2, at least for the detection mechanism. These two are included as a TP in the Annex. In addition to this, we also include a TP to stage3 carrying the additional information indicated above.

### 2.3.2	Near failure scenarios

Another distinct nature of LTM is that the UE could perform a RACH-less cell switch towards the target cell with valid TA acquired from the PDCCH order’s TA acquisition or UE-based TA measurement. It is beneficial for network to recognize the RA-based cell switch as a near failure, since the UE is supposed to benefit from the LTM feature by performing a RACH-less access. Therefore, a RA-based access could be viewed as a sub-optimal LTM mobility incident. We recommend, the RA-based access is a SHR trigger for LTM feature.

In case the UE is configured with LTM config, the RACH-based access shall trigger the SHR report.

One thing we would like to clarify is that the UE attempt the RACH-less access until T304 expiry, which is equivalent to declaration of HOF. Before T304 expiry, there is no such thing as fallback from RACH-less to RACH-based. Only HOF occurs, the UE may perform LTM based recovery, which is based on RA using LTM cell configuration. However, we call it a LTM recovery procedure instead of fallback procedure and this is to be considered as a failure case.

According to TS 38.300, the LTM defines the UP/HO interruption as depicted in below Figures in order to measure the latency of RACH-less and RACH-based procedure:

Figure G-2: Mobility Latency for RACH-based LTM

Figure G-3: Mobility Latency for RACH-less LTM

Reducing HO interruption time is one of the main motives for LTM and is counted from UE receives the cell switch command to UE performs the first DL/UL reception/transmission on the indicated beam of the target cell. The HO interruption time could be set as a trigger for SHR report to reflect the sub-optimal latency case caused by LTM mobility.

In case the UE is configured with LTM config, the HO interruption time shall trigger the SHR report.

### 2.3.3	Successful scenarios

Also, a potential risk has been raised during the Rel-18 discussion of supporting subsequent LTM feature, which is about the ping-pong issue. The L1 measurement may be unstable and leading the handover decision directing UE to access to the same cell back and forth in a limited time. Therefore, we suggest to consider the UHI enhancement for LTM to identify the intra-gNB ping-pong issue. The CU should be responsible to collect the UHI and further provide the latest UHI to the new DU timely. Specifically, the CU collect the UHI in a new cell upon reception of the LTM notification or access success message from the DU.

The CU collect the UHI in a new cell upon reception of the LTM notification or access success message from the DU and further provide the latest UHI to the new DU timely.

## 2.2	MRO for CHO with candidate SCGs

The agreements in last meeting were:

Do not distinguish between fast MCG recovery/no-fast MCG recovery for now (to simplify use cases).

Concurrent error cases (MCG+SCG) are FFS. The definition of these error cases needs to be further clarified.

RAN3 will start with the failure scenarios with UEs configured with CHO with candidate SCGs.

Whether to include failure and near failure scenarios related with configuration of CHO with candidate SCG(s) and CHO only is FFS.

### 2.3.1	The configuration

According to RAN2#126 Chair Notes, for the assumptions on configurations, the following options were proposed for discussions:

Option 1: R18 configurations of CHO with candidate SCGs

Option 2: R17 configurations of CHO with target SCG

Option 3: R16 configurations of CHO

In RAN2, some companies proposed to discuss Option 1 only, while some companies proposed to discuss mixed configurations, e.g. Option 1 + Option 2, or Option 1 + Option 3. Since the option1 is the target case for this Rel-18 feature objective, the usage of mixed configurations is just a supplement to option1 in case that these parameters of option1 could not be configured properly and thus raise the risk of mobility failure issues. We think RAN3 can focus on Option 1 first, and other options (including mixed configuration) can be de-prioritized. Once the MRO solution for option 1 works well, the mixed configuration is not needed as a safety net anymore.

Other options of mixed configuration can be de-prioritized.

### 2.3.2	Failure scenarios

CHO with candidate SCG(s) is defined as a PCell change with PSCell addition/change that is executed by the UE when the execution conditions for both candidate PCell and the associated candidate PSCell are met as specified in TS 37.340. In another word, the UE does not execute CHO with candidate SCG(s) until the execution conditions for both the candidate PCell and the associated candidate PSCell are met.

Using the agreed R18 configurations of CHO with candidate SCGs only, we are trying to address the specification for the following failure cases:

For case1, identified as too late failure case, we believe that unlike legacy, the key point is to identify which of the two criteria are blocking a handover. Without this information, it is not possible to determine which parameter to potentially adjust. Therefore, we believe it would be beneficial to indicate which of the criteria are met first and also the time between the criteria is met and the failure.

Indicate which if the two criteria were triggered first (PCell or PSCell) and the time between the trigger and the failure event.

For RLF, it is beneficial to indicate the last configured HO type as CHO with candidate SCGs to distinguish from the legacy handover type. That way the network can perform optimizations to this kind of handover. For now, we see no strong need to enhance SCGFailureInformation since MN knows the exact handover type upon reception of SCGFailureInformation. This could be implicitly signalled by the information above.For case 2 and case 3, it can be divided into those 3 sub-cases: execution failure/RLF at the target PCell, execution failure/RLF at the target PSCell, concurrent execution failures/RLFs at both the target PCell and the associated target PSCell. The former two case are single failure either at the target MCG or at the target SCG, which could be reflected by the RLF report and SCGFailureInformation, respectively. Otherwise, no specification impact is foreseen.

For single failure either at the target MCG or target SCG, no extra specification impact is foreseen.

However, for concurrent execution failures/RLFs at both the target PCell and the associated target PSCell, the UE could not make SCGFailureInformation available to the network, since there is no functioning MN link at the time of SCG failure. Thus, the RLF report could be enhanced to be reflect the SCG failure information, e.g. the SCG failure type and the SCG measurement results.

For concurrent execution failures/RLFs both at the target MCG and target SCG, the RLF report could be enhanced to reflect the SCG failure information, e.g. the SCG failure type and the SCG measurement results.

### 2.3.3	Near failure scenarios

For near failure case as below illustrated we assume the legacy mechanism can be re-used to the maximum extent. For near failure in PCell mobility and PSCell mobility, the UE generate SHR and SPR.

In order for the source node to correlate the SHR and SPR reports, the UE should also report the time between the execution of event and the reporting. With this information, the current solution would be able to combine the information from SHR and SPR associated to the same mobility event.

Include a time between the execution of the event and the reporting in the SHR/SPR.

In order for the receiving node to understand that this is from this new mobility event, the UE may also indicate the time between PCell triggered and associated PSCell triggered in SHR/SPR.

Indicate the time between the two cells triggered in SPR/SHR which implicitly indicate the new handover type.

To conclude, at the moment it seems that the impact on RAN3 is limited but that information is required form the UE. We propose to send an LS to RAN2 to request the information mentioned above.

## 2.3	MRO for subsequent CPAC

The agreements in last meeting were:

CPAC failure scenarios and detection mechanism captured in stage2 used as baseline.

### 2.3.1	Failure scenarios

A subsequent Conditional PSCell Addition or Change (subsequent CPAC) is defined as a conditional PSCell addition or change procedure that is executed after a PSCell addition, a PSCell change, a PCell change or an SCG release based on pre-configured subsequent CPAC configuration of candidate PSCell(s) without reconfiguration and re-initiation of CPC/CPA. In last meeting, RAN3 agreed to consider the failures in the S-CPAC procedure. We’d like to first specify some typical S-CPAC failure/near-failure cases respectively and to discuss the potential solutions. Likewise, the failure cases of S-CPAC are similar to that of CPAC, including the too-early CPAC, to-late CPAC, and CPAC to wrong cell, as defined in TS 37.340:

- Too Late CPC Execution: UE receives CPC configuration, while a SCG failure occurs before CPC execution condition is satisfied; a suitable PSCell different from source PSCell is found based on the measurements reported from the UE.

- Too Early CPC/CPA Execution: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; in case of CPC, the source PSCell is still the suitable PSCell based on the measurements reported from the UE; in case of CPA, no suitable PSCell is found based on the measurements reported from the UE.

- CPC/CPA Execution to wrong PSCell: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; a suitable PSCell different from the source PSCell or the target PSCell is found based on the measurements reported from the UE.

The difference mainly lies on the cause of the S-CPAC failures, since some information about UE or candidate PSCells are not updated timely. Thus, RAN3 can follow the failure cases of CPAC as the reference for S-CPAC. The main difference here is that there is an initiating node determining the candidate list, and each candidate will determine the exit condition (when to handover to another candidate). Hence, the mechanism must enable the forwarding of correct information to all involved nodes.

MN shall decide which SN should be informed when failure occurs. In the case of SN initiated S-CPAC, the source SN may not be aware of the failures and what is happening to the UE, since SN is not responsible for collecting the related information. Therefore, MN should inform the initiating SN for optimization such as candidate cell selection, etc.

Likewise, it is beneficial to let the last serving candidate SN to know such information to analyse the failure causes and further CPAC strategy optimization, e.g., by informing the “source” SN of the failure information. According to above, we conclude the mechanism enhancements for S-CPAC failure scenarios as follows.

MN is responsible for determining where to forward the information for failed S-CPAC events.

If an SN is initiating the S-CPAC, the MN forwards information to this SN in order to optimise the candidate cell selection.

MN also forwards the information to the last serving candidate SN in order to optimise the mobility parameters

The last serving SN analyses if the problem is in his domain, and may forward to the “source SN” if not.

### 2.3.2	Near failure scenarios

For near failure cases, we assume the legacy mechanism and the above enhancements can be re-used.

### 2.3.3	Successful scenarios

Similar as the root causes of the failure, we’d like to address two main aspects related to S-CPAC. The first one is that the expired UE information. As standardized in Rel-18, the target SN(s) receive the SCG UHI from the MN via SN Addition Request message for CPC, which is at the preparation phase of the subsequent CPAC. Hence, for further CPC, the subsequent target SNs are not able to access the latest SCG UHI, which may hinder the timely and precise operations of target SNs, e.g., parameter optimization, resource allocation, etc. Notice that the UE information is updated to MN, we propose to update the UHI for SNs in the S-CPAC.

Even if the information will not impact a current CPAC configuration, it would still make sense to update the SN as early as possible since the initiating node may configure additional UE with S-CPAC. Another possibility would be to deliver the information once, e.g., when the UE leaves the cell. This would probably be a bit more complicated to specify. Another option is including the information on stage but not mandate when this update is sent.

RAN3 to agree that the updated UHI shall be possible to send to subsequent target SNs from MN, e.g., via SN Reconfiguration Complete message.

Another similar information exchange that may be beneficial is that the initiating node may be informed of the outcome of the S-CPAC configuration. S-CPAC requires reservation of resources in advance. The initiating node (MN or SN) could evaluate the selection of candidate cells and possibly the time spent in each cell to evaluate if S-CPAC was beneficial or not.

In case of MN-initiated S-CPAC, we believe the MN already can get this information, since MN is already responsible for collecting the UHI. But in the case of SN initiated S-CPAC, the source SN may not be aware of what is happening to this UE, since SN is not responsible for collecting this information. The MN could however inform the initiating SN.

RAN3 to discuss whether it is beneficial to inform the source SN about the outcome of mobility events for SN initiated S-CPAC.

# 3	Conclusion

This document proposes:

Proposal 1:	The CU shall further make the RLF/SHR report available to the source DU via the existing procedure (i.e., Access and Mobility Indication) over F1.

Proposal 2:	When gNB-CU receives an RLF report from another node, the gNB-CU analyse the report and determines the failure type to determine which of the involved gNB-DUs node should receive the information.

Proposal 3:	Additional information regarding source cell information and C-RNTI used in the source cell is added to the existing procedure (i.e., Access and Mobility Indication) over F1 for failure cases with RLF shortly after successful HO.

Proposal 4:	In case the UE is configured with LTM config, the RACH-based access shall trigger the SHR report.

Proposal 5:	In case the UE is configured with LTM config, the HO interruption time shall trigger the SHR report.

Proposal 6:	The CU collect the UHI in a new cell upon reception of the LTM notification or access success message from the DU and further provide the latest UHI to the new DU timely.

Proposal 7:	Other options of mixed configuration can be de-prioritized.

Proposal 8:	Indicate which if the two criteria were triggered first (PCell or PSCell) and the time between the trigger and the failure event.

Proposal 9:	For single failure either at the target MCG or target SCG, no extra specification impact is foreseen.

Proposal 10:	For concurrent execution failures/RLFs both at the target MCG and target SCG, the RLF report could be enhanced to reflect the SCG failure information, e.g. the SCG failure type and the SCG measurement results.

Proposal 11:	Include a time between the execution of the event and the reporting in the SHR/SPR.

Proposal 12:	Indicate the time between the two cells triggered in SPR/SHR which implicitly indicate the new handover type.

Proposal 13:	MN is responsible for determining where to forward the information for failed S-CPAC events.

Proposal 14:	If an SN is initiating the S-CPAC, the MN forwards information to this SN in order to optimise the candidate cell selection.

Proposal 15:	MN also forwards the information to the last serving candidate SN in order to optimise the mobility parameters

Proposal 16:	The last serving SN analyses if the problem is in his domain, and may forward to the “source SN” if not.

Proposal 17:	RAN3 to agree that the updated UHI shall be possible to send to subsequent target SNs from MN, e.g., via SN Reconfiguration Complete message.

Proposal 18:	RAN3 to discuss whether it is beneficial to inform the source SN about the outcome of mobility events for SN initiated S-CPAC.

# Annex	TP for LTM for 38.300

##### 15.5.2.2.2	Connection failure due to intra-system mobility

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows:

-	Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-	Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-	Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure.

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell.

In case of LTM, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late LTM cell switch, Too Early LTM cell switch and LTM cell switch to Wrong Cell.

Detection mechanism

A failure indication may be initiated after a UE attempts to re-establish the radio link connection at NG-RAN node B after a failure at NG-RAN node A. NG-RAN node B may initiate the Failure Indication procedure towards multiple NG-RAN nodes if they control cells which use the PCI signalled by the UE during the re-establishment procedure. The NG-RAN node receiving this selects the UE context that matches the received Failure Cell ID and C-RNTI, and, if available, uses the shortMAC-I to confirm this identification, by calculating the shortMAC-I and comparing it to the received IE.

A failure indication may also be sent to the node last serving the UE when the NG-RAN node fetches the RLF REPORT from UE by triggering:

-	The Failure Indication procedure over Xn;

-	The Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG.

The detailed detection mechanisms for too late handover, too early handover and handover to wrong cell are carried out through the following in the NG-RAN node that served the UE before the reported connection failure:

-	Intra-system Too Late Handover: there is no recent handover for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if CHO is configured but the CHO execution is not initiated for the UE prior to the connection failure, e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt).

-	Intra-system Too Early Handover: there is a recent handover for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the successful re-connect cell is the cell that served the UE at the last handover initialisation or fall back to the source cell configuration in case of DAPS HO.

-	Intra-system Handover to Wrong Cell: there is a recent handover for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/ the cell UE attempts to re-connect/the cell UE attempts CHO recovery is neither the cell that served the UE at the last handover initialisation nor the cell that served the UE where the RLF happened or the cell that the handover was initialized toward.

The "UE reported timer" above indicates the time elapsed since the last handover initialisation until connection failure or the time elapsed since the HO execution including CHO and LTM until connection failure.

In case of Too Early Handover or Handover to Wrong Cell, the NG-RAN node receiving the failure indication may inform the NG-RAN node controlling the cell where the mobility configuration caused the failure by means of the Handover Report procedure over Xn or the Uplink RAN Configuration Transfer procedure over NG. This may include the RLF report.

For MRO analysis, in case of RLFs or handover failures, the source gNB may take into account the information regarding the LBT failures occurred in UL and DL for a specific UE. For UL, the information may be collected and provided by the UE to the network in the RLF report. For DL, the information may be collected at the source and target gNBs. The target gNB may provide the information to the source gNB via the Access and Mobility Indication procedure over Xn, if requested by the source gNB during the handover preparation.

Retrieval of information needed for problem analysis

In order to retrieve relevant information collected at the network side as part of the UE context, the UE provides C-RNTI used in the last serving cell. If the cause for the failure is identified as a "Too Early HO" or a "HO to Wrong Cell", the NG-RAN node controlling the last serving cell shall, include in the HANDOVER REPORT message the C-RNTI used in the source cell of the last completed handover before the failure. If the NG RAN node controlling that source cell provided the Mobility Information, it is also included in the HANDOVER REPORT message. If used, the Mobility Information is prepared at the source NG RAN node of a handover and may refer to or identify any handover-related data at this NG RAN node. In the Handover Preparation procedure, the source gNB can request the target gNB to provide information on DL LBT failures at the target gNB during handover execution.

Handling multiple reports from a single failure event

In case the RRC re-establishment fails and the RRC connection setup succeeds, MRO evaluation of intra-RAT mobility connection failures may be triggered twice for the same failure event. In this case, only one failure event should be counted.

# Annex	TP for LTM for 38.401

## 7.X	MRO support for LTM

The gNB-CU receives a RLF report associated to an LTM mobility event from the UE or via the Failure indication message over Xn. The detailed procedure is carried out by gNB-CU:

- If the RLF report indicates that the connection failure type is RLF and there is a recent LTM cell switch for the UE prior to the connection failure, e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), the CU sends the RLF report to the DU which initialised this recent LTM cell switch and may include the C-RNTI used by the UE in this DU (if available).

- Otherwise, the CU send the RLF report to the DU where the UE was connected prior to the connection failure.

# Annex	TP for LTM for 38.473

### 9.2.10	Self Optimisation Support Messages

#### 9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 .. <maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| >>UE Identifier | O |  | 9.3.1.32 | C-RNTI allocated at the gNB-DU | - |  |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | - |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| Successful PSCell Change Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful PSCell Change Report information Item |  | 1..<maxnoofSuccessfulPSCellChangeReports> |  |  | - |  |
| >>Successful PSCell Change Report Container | M |  | OCTET STRING | Includes the SuccessPSCell-Report IE as defined in TS 38.331 [8]. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |
| maxnoofSuccessfulPSCellChangeReports | Maximum no. of Successful PSCell Change Reports. Value is 64. |

# Annex	TP for CHO with candidate SCGs for 37.340

## 10.X	Self-optimisation for CHO with candidate SCGs

Handling multiple reports from a single CHO with candidate SCGs event

In case both the execution towards the target PCell and the execution towards the associated target PSCell succeed, MRO evaluation of sub-optimal conditional PCell execution and condition PSCell execution may be triggered simultaneously for the same mobility event. In this case, the corresponding SHR and SPR should be correlated.

# Annex	LS to RAN2 on MRO for CHO with candidate SCGs

Title:	[DRAFT] LS on MRO for CHO with candidate SCGs

Response to:

Release:	Rel-19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	Huawei [will be RAN3]

To:	RAN2

Cc:

Contact person:	Henrik.olofsson@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN3 has discussed MRO for CHO with candidate SCGs and agreed it is beneficial if the following information can be reported by the UE to the network:

- Indicate which if the two criteria were triggered first (PCell or PSCell) and the time between the trigger and the failure event.

- Include a time between the execution of the event and the reporting in both the SHR/SPR. This enables the correlation of any SHR/SPR coming from the same mobility event.

RAN3 further discussed the case when a connection failure during a mobility event occurs in the PSCell and where this failure cannot be reported in the PCell due to a connection failure in the PCell. RAN3 would like to ask RAN2 if it would be feasible to instead attach the SCG failure information to the RLF report used to indicate he connection failure for the PCell.

# 2	Actions

To RAN2

ACTION: 	RAN3 kindly ask RAN2 to take the above into account.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#125-bis	2024-10-14 – 2024-10-18 	China, CN

RAN3#126	2024-11-18  – 2024-11-22 	Orlando, US