3GPP TSG-RAN WG3 Meeting #125	R3-244279

Maastricht, NL, 19 -23 Aug, 2024

Title:	SON for Network Slicing

Agenda Item:	10.3.2

Source:	Huawei

Document for:	discussion

# 1	Introduction

The following agreements have been reached so far:

RAN3#123bis

Both the logged MDT and immediate MDT enhancements based on scenarios to be worked.

Work on the Slice aware MRO scenarios.

RAN3#124

RAN3 to support immediate MDT measurements collection on specific area per slice(s).

FFS on which immediate MDT measurements can be collected per slice.

FFS on whether to support logged MDT on specific area per slice(s).

RAN3 to work on logged MDT enhancement for slice-based cell reselection.

For successful handover, FFS if solutions are needed to optimize handovers in the case of slicing discontinuity, e.g., target cell not supporting slice.

# 2	Discussion

## 2.1	General

In the past, we have often seen an initial discussion about what is included in a SON/MDT work item. We think it would be beneficial to discuss and agree what the scope of SON/MDT work item normally consists of.

- SON implies optimization, i.e., where we use information exchanged between nodes to optimize NG-RAN node behaviour. We normally do not specify optimization that can be performed without exchange of information. If the actions do not require interaction with another node, we also do not describe the behaviour (e.g., Intra-system ping pong) and instead consider this to be up to implementation.

- MDT is a mechanism to collect per UE information across different nodes. The information is reported to OAM (TCE) and the intention is to replace drive test with dedicated mobiles. MDT is slightly more costly for both UE and RAN and is therefore likely to be used in specific situations where an error situation needs to be analysed a bit more thoroughly.  The configuration of these is defined in RAN3 and the contents of the reporting is defined in SA5. Some of the measurements are defined in RAN2

- KPI includes various indicators as defined by SA5. The requirements for these measurements are discussed in SA5.

## 2.2	Enhancement for the MDT area scope

RAN3 agreed to support immediate MDT measurements collection on specific area per slice(s).

In the NG-RAN, different NG-RAN nodes may support different slices due to the operator policy. From the perspective of a specific slice, it can be supported by the specific tracking areas, which could vary from that of another slice. The operator may be interested in only a few slices, for example, the operator more likely focuses on the management for some URLLC slices. There is no need for the NW to configure the UE to perform MDT measurement when the UE is out of the supporting area for the interested slice. As a result, the definition of MDT area scope in the MDT configuration can be enhanced by considering the slice-specific area limitation.

There is however a difference in usage between management based and signalling basaed. For management based, the intention is to collect information within a certain area. To further filter out the area scope based on the slice identity seems to make sense in this scenario. For signalling based however, the usage is typically for an operator to monitor the behvaiour of a specific UE. For this case, there is probably no need to further filter on slices. Hence, the enhancement on MDT area scope could be considered for management based only.

The details for management based is defined in SA5. Therefore, a potential agreement in RAN3 could be that we agree to not add this to signalling based MDT.

RAN3 agrees that no enhacement to MDT area scope over RAN3 interfaces for signalling based MDT are needed.

If this is to be discussed for management based MDT, he next question is which S-NSSAI to use for this filtering. In connected more, it makes sense to filter according to the S-NSSAI assigned to the PDU-session. Since a UE may handle multiple PDU-sessions with different S-NSSAI, it would probably make sense to trigger Immediate MDT measurement for UEs that are supporting at least one of the slices that are indicated for immediate MDT. There is also an open issue related to whether this filtering also applies to logged MDT. Logged MT spans inactive and idle mode. For idle UE, the filtering on S-NSSAI assigned to PDU sessions cannot be used. For inactive UE, there is no point of filtering on S-NSSAI assigned to PDU sessions since the all DRBs associcated to all PDU sessions from the cell of prvious RRC connection are suspended and the UE may camp on another cell. Instead, the UE is pre-configuredwith a set of allowed S-NSSAI by NAS signalling or UE is triggered to come to connected by a requesting S-NSSAI among the allowed S-NSSAI. These could be used to filter out which UEs to collect reports for logged MDT.

Since the details for management based MDT configuration is in the scope of SA5, we could send an LS to AS5 and infrm htem that RAN3 considers it beneficial to support MDT area scope on a per S-NSSAI level.

RAN3 informs SA5 that we think it is beneficial to include the S-NSSAI aspect in the the MDT area scope configured for management based MDT.

## 2.3	Enhancement for immediate MDT reporting

The UE provides NSSAI for network slice selection in RRCSetupComplete, if it has been provided as part of allowed NSSAI list by NAS. When a new PDU session needs to be established, the 5GC requests the NG-RAN to allocate resources relative to the relevant PDU sessions over NG-C. For connected UE’s mobility, S-NSSAI is introduced as part of the PDU session information which is transferred to target RAN during mobility signalling, i.e., the handover request message over NG and Xn. This enables slice-aware admission and congestion control for target NG-RAN. In a summary, the slice awareness in NG-RAN is introduced at PDU session level, by indicating the S-NSSAI to the corresponding PDU session, in all signalling containing PDU session resource information.

It may be meaningful for to include the slice ID associated with the ongoing PDU session in the MDT report to enable the NW to analyze and observe the status of different QoS requirement associated to different slices.

On the other hand, the content of the report is defined in SA5 and many reports already include reporting per S-NSSAI.

RAN3 to discuss if any further slice information is needed, and if agreed inform SA5 on possible enhancements for immediate MDT reporting.

## 2.4	Enhancement for slice-based cell reselection

The UE in RRC_IDLE mode or in RRC_INACTIVE mode may perform slice-based cell reselection for the purpose of camping on a more suitable cell. During the procedure, the UE takes both the mapping relationship between the S-NSSAI and NSAG received by NAS and the FreqPriorityListSlicing received in SIB16 or RRCRelease into account for frequency ranking and the cell selection evaluation. It can be helpful for NW to adjust the NW configuration if the UE logs and reports the NSAG information after performing cell reselection.

In its simplest form, it may make sense for the UE to store whether he is able to camp on a cell prioritised by NSAG or not. If multiple NSAG exist, it may be reasonable to report which of the NSAG the UE is able to camp on. With this information, the operator may evaluate whether the UE is able to stay within the preferred NSAG, similar to the reporting of in/out of coverage. This however requires some work from RAN2.

Send an LS to RAN2 to propose the collection of NSAG into the logged MDT report.

## 2.5	MRO for failure cases

SON functionality is meant to exchange information between NG-RAN nodes to enable optimization in RAN. As we mentioned in [2], there could be a little impact on MRO enhancement. The mobility decisions may be made based on the slice support or the slice availability at the target NG-RAN node. For instance, a UE may be handed over to a target cell supporting the slice associated with the ongoing PDU session but without good radio conditions, it could be beneficial if this aspect could be analysed in the existing MRO mechanism. Hence, the NW may disregard mobility failures triggered by slice reasons.

Another possibility would be to let MRO perform different optimization depending on the impact on slice availability on mobility. This is similar to the previous mentioned solution (disregard some failures) but where it is left to implementation whether to disregard or split the reports into different sets where the different sets are used to optimize different mobility parameters used in different conditions.

Since we already have the possibility to retrieve stored context, any vendor wanting to support this feature would need to store information about mobility decisions until the report arrives from the UE. We do not think that any additions are needed to stage3. This may not even have impact on stage2 since the solution would rely on existing framework-

The Slice aware MRO can be supported without any standards impact

## 2.6	MRO for successful cases

One use case discussed in RAN3#124 was the successful case. The proposed scenario is to try to avoid that a UE passes through a cell not supporting the slice of interest for a UE corresponding to the red path depicted in the figure below, but instead tries to maintain the slice support (blue path) and immediately handover to Cell 3.

Cell1

slice a,b

Cell3

slice a,b

Cell2

slice a

Figure 1. Successful handover use case

For this scenario, our assumption is that a node optimizing the parameters with MRO will already know the boundaries for the mobility parameters, i.e., what is possible without creating a too a large risk for failed mobility. The nodes would also already know what slices are supported in their neighbours and would always strive to perform the mobility from cell1 to cell 3 when possible. Therefore, node handling cell1 would already try its best to handover the UE to cells supporting the desired slices. Adding the possibility for other nodes to feedback information about this (the red line) would not help the node handling cell1.

No need to enhance the successful case for MRO

# 3	Conclusion

This document proposes:

Proposal 1:	RAN3 agrees that no enhacement to MDT area scope over RAN3 interfaces for signalling based MDT are needed.

Proposal 2:	RAN3 informs SA5 that we think it is beneficial to include the S-NSSAI aspect in the the MDT area scope configured for management based MDT.

Proposal 3:	RAN3 to discuss if any further slice information is needed, and if agreed inform SA5 on possible enhancements for immediate MDT reporting.

Proposal 4:	Send an LS to RAN2 to propose the collection of NSAG into the logged MDT report.

Proposal 5:	The Slice aware MRO can be supported without any standards impact

Proposal 6:	No need to enhance the successful case for MRO

# Annex – LS to SA5

Title:	[DRAFT] LS on MDT area scope for MDT

Response to:

Release:	Rel-19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	Huawei [will be RAN3]

To:	SA5

Cc:

Contact person:	Henrik.olofsson@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN3 has discussed enhancements for SON and MDT in the scope of the new slicing fcuntionality added in Rel-18. RAN3 agreed that it is beneficial include the S-NSSAI in the MDT area scope for management based MDT.

# 2	Actions

To SA5

ACTION: 	RAN3 kindly ask SA5 to take the above into account.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#125-bis	2024-10-14 – 2024-10-18 	China, CN

RAN3#126	2024-11-18  – 2024-11-22 	Orlando, US

# Annex – LS to RAN2

Title:	[DRAFT] LS on NSAG in Logged MDT

Response to:

Release:	Rel-19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	Huawei [will be RAN3]

To:	RAN2

Cc:

Contact person:	Henrik.olofsson@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN3 has discussed enhancements for SON and MDT in the scope of the new slicing functionality added in Rel-18. RAN3 agreed that it is beneficial to capture the impact of NSAG on UE mobility in the logged MDT report. RAN3 would kindly like to ask RAN2 to consider adding this this to Logged MDT, if feasible.

# 2	Actions

To RAN2

ACTION: 	RAN3 kindly ask RAN2 to take the above into account.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#125-bis	2024-10-14 – 2024-10-18 	China, CN

RAN3#126	2024-11-18  – 2024-11-22 	Orlando, US