3GPP TSG-RAN WG3 Meeting #125	R3-244280

Maastricht, NL, 19 -23 Aug, 2024

Title:	(TP for SON BLCR for 36.300, 36.423) Rel-18 SON and MDT leftovers

Agenda Item:	10.4

Source:	Huawei

Document for:	discussion

# 1	Introduction

This paper discuss the Rel-18 SON and MDT leftovers.

# 2	Discussion

## 2.1	RACH optimization for SDT

RAN3#123bis made the following agreement:

Work on scenario of RACH optimization for SDT first.

RAN3#124 made the following agreement:

There is no need for UE to report SDT configurations, e.g., RSRP threshold, data volume threshold, T319a, and other configurations.

From our point of view, at least the following information are helping the network to analyze the issues causing SDT failure and optimize the related condition parameters to maximize the performance of SDT:

- Data volume at the time of initiating the SDT or normal RA

- DL RSRP at the time of initiating the SDT or normal RA

- Configured RSRP and data volume via SIB1

The first two seems to be agreed in RAN2 and for the third one we have the agreement to not report this from the UE. We believe that one remaining issue would be whether there is a need to report something from the UE to enable the identification of the Configured RSRP and data volume. This since the RA report may be stored in the UE for up to 48 hours.

Include the time between the RA event and the report in the RA report and send an LS to RAN2

## 2.2	MRO for MR-DC SCG failure

RAN3 received an LS from RAN2 (R2-2405846) with the following information

RAN2 thanks RAN3 for the LS, and RAN2 agrees to only support EN-DC case on MRO for MR-DC SCG failure. RAN2 understands that whether also supporting NGEN-DC has no additional RAN2 impact hence RAN3 can decide. If time allows in Rel-19, RAN2 can consider NE-DC scenario.

Rel-17 SON/MDT WI developed the solution for MRO for NR-DC SCG failures according to TS 38.331 specification. In Rel-17, SON/MDT WI considered primarily two SCG failure scenarios as follows:

- SCG RLF, and

- Legacy PSCell change or addition failure.

EN-DC

For EN-DC scenario, the UE report the NR SCG failure information to LTE MN node, and then the LTM MN node may further forward the NR SCG failure information to NR SN node. The SCGFailureInformationNR message reported from UE to LTE node needs to be enhanced in TS 36.331 accordingly to align with the SCGFailureInformation defined in TS 38.331.

Table 1 Enhancement to align with the NR-DC SCG failure report in TS 38.331

| Parameter | IE/value & condition |
| --- | --- |
| failedPSCellId | Frequency and PCI |
| previousPSCellId | Frequency and PCI |
| timeSCGFailure | 0-1023 |
| perRAInfoList | Only can be included if failureType is set to synchReconfigFailureSCG, or is set to randomAccessProblem while T304 is running |

From RAN3 perspective, either a new signalling for forwarding the enhanced NR SCG failure information is developed, or the MeNB to SgNB Container IE via the SN modification request message from LTE MN to NR SN, referred to CG-ConfigInfo message as defined in TS 38.331, is reused. However, the MeNB to SgNB Container IE via the SN modification request message is not intended for SON purpose. This is how it was solved for Xn. Therefore, we propose

Introduce a new signalling over X2 for forwarding the enhanced NR SCG failure information from LTM MN to NR SN.

Regarding stage2, the current description in 37.340 seems general enough to cover this scenario. But we need to add some text to 36.300 to cover PSCell change failure.

Introduce a new section in 36.300 for PSCell change failure.

Inform RAN2 about our decision.

NGEN-DC

Similar as EN-DC case, the SCGFailureInformationNR message defined in TS 36.331 needs to be enhanced accordingly, as above discussed. For forwarding from the MN to SN, i.e. from one ng-eNB to one gNB, the SCG failure Information Report message defined in TS 38.423 for Xn could be reused to transfer SCGFailureInformationNR within SCG Failure Report container from MN to SN. Regarding stage2, the current description in 37.340 and 38.300 seems general enough to cover this scenario.

No RAN3 impact is foreseen for NGEN-DC.

Inform RAN2 about our decision.

NE-DC

For NE-DC scenario, the UE report the LTE SCG failure information to NR MN node, and then the NR MN node may further forward the LTE SCG failure information to LTE SN node. Therefore, the SCGFailureInformationEUTRA message in TS 38.331  needs to be enhanced. The SCG failure Information Report message defined in TS 38.423 for Xn could be reused to transfer SCGFailureInformationEUTRA within SCG Failure Report container from MN to SN.

No RAN3 impact is forseen for NE-DC.

Similar to the above, no major impact is expected. This topic requires RAN2 action which is however down prioritized, so RAN3 could inform RAN2 that RAN3 keeps this on hold and ask RAN2 to notify if any agreements.

The text proposals for the above proposals can be found in the Annex.

# 3	Conclusion

This document proposes:

Proposal 1:	Include the time between the RA event and the report in the RA report and send an LS to RAN2

Proposal 2:	Introduce a new signalling over X2 for forwarding the enhanced NR SCG failure information from LTM MN to NR SN.

Proposal 3:	Introduce a new section in 36.300 for PSCell change failure.

Proposal 4:	Inform RAN2 about our decision.

Proposal 5:	No RAN3 impact is foreseen for NGEN-DC.

Proposal 6:	Inform RAN2 about our decision.

Proposal 7:	No RAN3 impact is forseen for NE-DC.

Proposal 8:	Similar to the above, no major impact is expected. This topic requires RAN2 action which is however down prioritized, so RAN3 could inform RAN2 that RAN3 keeps this on hold and ask RAN2 to notify if any agreements.

# Annex	LS to RAN2 on SDT

Title:	[DRAFT] LS on SDT enhancements for SON

Response to:

Release:	Rel-19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	Huawei [will be RAN3]

To:	RAN2

Cc:

Contact person:	Henrik.olofsson@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN3 has discussed SDT enhancements for SON. RAN3 noted that RAN2 already made some progress and agreed that in addition to agreements in RAN2, RAN3 sees benefit in including the time between the RA event and the report in the RA report. This could be used by the network to determine the configuration that was used at the time the RA event occurred.

# 2	Actions

To RAN2

ACTION: 	RAN3 kindly ask RAN2 to take the above into account.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#125-bis	2024-10-14 – 2024-10-18 	China, CN

RAN3#126	2024-11-18  – 2024-11-22 	Orlando, US

# Annex	LS to RAN2 on MRO for MR-DC SCG failure

Title:	[DRAFT] Reply LS on support of MRO for MR-DC SCG failure

Response to:	R2-2405846

Release:	Rel-19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	Huawei [will be RAN3]

To:	RAN2

Cc:

Contact person:	Henrik.olofsson@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN3 thanks RAN2 for the LS on MRO for MR-DC SCG failure and would like to inform RAN2 about the agreements in RAN3:

  - RAN3 agreed to further enhance RAN3’s interfaces to support EN-DC and NGEN-DC. RAN3 will progress assuming a similar solution as used in legacy functionality and therefore asks RAN2 to inform RAN3 if any important difference to the legacy solution is identified.

  - For the NE-DC case, RAN3 will align with RAN2 and progress if any progress in RAN2. RAN3 therefore asks RAN2 to inform RAN3 in case this work is commenced in the Rel-19 WI.

# 2	Actions

To RAN2

ACTION: 	RAN3 kindly ask RAN2 to take the above into account.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#125-bis	2024-10-14 – 2024-10-18 	China, CN

RAN3#126	2024-11-18  – 2024-11-22 	Orlando, US

# Annex	TP for MR-DC SCG failure for 36.300

### 22.4.2	Support for Mobility Robustness Optimisation

#### 22.4.2.1	General

Mobility Robustness Optimisation aims at detecting and enabling correction of following problems:

-	Connection failure due to intra-LTE or inter-RAT mobility;

-	Unnecessary HO to another RAT (too early IRAT HO with no radio link failure);

-	Inter-RAT ping-pong;

-	Inter-system ping-pong;

-	PSCell change/addition failure.

#### 22.4.2.X	PSCell change/addition failure

For analysis of PSCell addition/change failures, the UE makes the SCG Failure Information available to the MN. If the MN can perform an initial analysis, it transfers the SCG Failure Information together with the analysis results to the relevant SN as defined in TS 37.340 [76].

# Annex	TP for MR-DC SCG failure for 36.423

### 8.6.x	SCG Failure Information Report

#### 8.3.x.1	General

The purpose of the SCG Failure Information Report procedure is to provide SCG mobility related information to the SeNB node.

The procedure uses UE-associated signalling.

#### 8.3.x.2	Successful Operation

Figure 8.3.x.2-1: SCG Failure Information Report, successful operation

The MeNB initiates the procedure by sending the SCG FAILURE INFORMATION REPORT message to the SeNB. Upon receiving the message, the SeNB shall assume that a PSCell change failure event was detected.

The SCG FAILURE INFORMATION REPORT message may include:

-	the SN Mobility Information IE, if the SN Mobility Information IE was sent for the PSCell change procedure from the SeNB;

-	the Source PSCell CGI IE, if the Source PSCell CGI IE was sent for the PSCell change procedure from the SeNB.

If the SCG FAILURE INFORMATION REPORT message includes the Source PSCell CGI IE, the SeNB shall, if supported, store the information.

If the SCG FAILURE INFORMATION REPORT message includes the Failed PSCell CGI IE, the SeNB shall, if supported, store the information and act as specified in TS 36.300 [15].

If received, the SeNB uses the above information for SCG failure reason detection and optimisation.

#### 8.3.17.3	Unsuccessful Operation

Not applicable.

#### 8.3.17.4	Abnormal Conditions

Void.

### 9.1.2	Messages for Dual Connectivity Procedures

#### 9.1.2.29	SCG FAILURE INFORMATION REPORT

This message is sent by MeNB to SeNB to report a PSCell change failure event for a specific UE.

Direction: MeNB  SeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | ignore |
| SeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the SeNB | YES | ignore |
| Source PSCel ID | O |  | NR CGI 9.2.111 | NR CGI of PSCell where SCG failure occurs for PSCell change procedure | YES | ignore |
| Failed PSCell ID | O |  | NR CGI 9.2.111 | NR CGI of PSCell where SCG failure occurs for PSCell change procedure | YES | ignore |
| SCG Failure Report Container | M |  | OCTET STRING | Contains the SCGFailureInformationNR message as defined in TS 36.331 [9]. | YES | ignore |