3GPP TSG-RAN WG3 Meeting #125	R3-244282

Maastricht, The Netherlands, August 19th – 23rd 2024

Agenda item:	8.3

Source: 	Ericsson

Title: 	MBS Communication Service Type for QMC

Document for:	Agreement

# Introduction

In the incoming LS R3-243028 “LS on MBS Communication Service Type”, SA5 informed RAN3 of the following:

SA5 has discussed and agreed to add MBS Communication Service Type IE into the QMCJob, which indicates whether doing QoE measurement shall be performed for MBS broadcast or multicast. This IE is used for both signaling and management based QoE measurement.

For more details, please see the agreed CR.

Meanwhile, the subsequent RAN3#124 meeting notes capture the following:

The UE should be aware whether to conduct QMC for MBS in broadcast or in multicast mode.

FFS whether the UE is informed via RRC or via the QMC configuration container or existing solutions.

In this contribution, we continue the above discussion.

# Discussion

In TS 38.413, the MBS Communication Service Type IE was introduced as a part of the UE Application Layer Measurement Configuration Information IE.

| MBS Communication Service Type | O |  | ENUMERATED (broadcast, multicast, ...) | This IE indicates the type of MBS communication service for which the QoE measurement collection should be performed. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

According to the LS from SA5, the reason to add the MBS Communication Service Type IE in the QMCJob is to indicate whether a QoE measurement shall be performed for MBS broadcast or multicast. This IE is sent from the OAM to the gNB either via the CN and NGAP (for of s-based QoE measurements), or directly from the OAM to the gNB (for m-based QoE measurements).

Observation 1: In the LS in R3-243028, SA5 informs RAN3 that MBS Communication Service Type is used to indicate whether QoE measurements shall be performed for MBS broadcast or multicast.

The discussions at the RAN3#124 meeting resulted in the following agreement:

The UE should be aware whether to conduct QMC for MBS in broadcast or in multicast mode.

It remains to be clarified whether the agreement above is already supported by current specifications or not, which is the reason for the below FFS:

FFS whether the UE is informed via RRC or via the QMC configuration container or existing solutions.

As of today, the gNB does not indicate to the UE via RRC whether to conduct QMC for MBS in broadcast or multicast. Hence, it needs to be checked whether such an indication exists in the QMC configuration container, or whether any other existing solution is available.

For example, for checking the content of QoE measurement configuration for DASH streaming, we can refer to TS 26.247, annex L2, where the format of the XML file used for QoE configuration is presented. In the XML file, there is no indication of whether QMC should be conducted for MBS in broadcast or in multicast. Based on this, our understanding is that the UE is not instructed via the QMC configuration container whether to conduct QMC in multicast or broadcast.

Observation 2: Neither the RRC signalling nor the QoE measurement configuration XML file contain an indication of whether the UE should perform QMC in MBS broadcast or in MBS multicast mode.

Another relevant clause is 6.11 in TS 23.247 (Architectural Enhancements for 5G Multicast-Broadcast Services), which describes the Service Announcement (see below). The Service Announcement “includes an MBS Service Type, which indicates whether the MBS Session for the service is multicast or broadcast.” However, the text also says that this information may (or may not) reach the UE: “Service Announcement information may be delivered to the UE via application specific means or as defined in TS 26.502”.

>>>>>>>>>>>>>Start of excerpt from TS 23.247<<<<<<<<<<<<<<<<

6.11	Service Announcement

Service Announcement provides the UE with descriptions specifying the multicast or broadcast services to be delivered as part of MBS Session.

The Service Announcement may be encoded as defined in TS 26.517 [22] or use application specific formats outside the scope of 3GPP.

Service Announcement information may be delivered to the UE via application specific means or as defined in TS 26.502 [18], using one of the following methods:

-	Pre-configured at UE side;

-	Via an MBS Session; or

-	Via a regular PDU Session.

NOTE 1:	Pre-configured service announcement is specified in TS 24.575 [26].

The Service Announcement includes the MBS Session ID(s), which is represented by TMGI or a Source Specific IP Multicast Address, for the service. When the MBS Session ID is Source Specific IP Multicast Address, the Service Announcement may include the PLMN ID of the PLMN and NID for an SNPN in which the service is delivered.

The Service Announcement includes an MBS Service Type, which indicates whether the MBS Session for the service is multicast or broadcast.

>>>>>>>>>>>>>End of excerpt from TS 23.247<<<<<<<<<<<<<<<<

Observation 3: Although MBS service type is an essential QoE configuration parameter, the MBS Service Announcement format does not guarantee its indication to the UE.

Based on this analysis, we conclude that, as of today, it is not certain that a UE application can be aware on whether to conduct QMC for MBS in broadcast or in multicast mode. Therefore, to resolve the FFS, we propose that the gNB can indicate to the UE whether QoE measurements are requested for MBS broadcast or multicast.

Proposal 1: The gNB should indicate to the UE via RRC whether QoE measurements shall be collected for MBS broadcast or multicast.

Regarding the requirement to collect QoE measurements for MBS broadcast or multicast, another aspect that can be noted is that, during a session delivered via multicast, a UE can move to an area where multicast is not supported, and the UE is hence reconfigured to use unicast. In this case, the session delivery continues but unicast will be used instead. If the original requirement from the OAM was to collect QoE measurements for MBS multicast, the QoE measurements that the UE will continue to perform while the session is delivered in unicast are not really needed. In addition, there is no indication in the reporting from the UE, about whether the QoE measurements were collected in multicast or in unicast. Therefore, the consumer of the QoE measurement cannot distinguish between the two sets of measurements, which defeats the whole purpose of requesting QoE measurements collection for multicast only.

Observation 4: The UE does not report whether QoE measurements were collected for MBS multicast, implying that a consumer cannot distinguish between QoE measurements collected for unicast or MBS multicast or broadcast in case of UE mobility from a gNB that supports MBS to a gNB that does not support MBS.

Hence, it needs to be ensured that the consumer of QoE measurements is informed about whether the measurements were collected for unicast, or for MBS broadcast, or MBS multicast.

Proposal 2: The consumer of QoE measurements should be informed about whether the measurements were collected for unicast, or for MBS broadcast, or MBS multicast.

Based on the above, we conclude that a correction is needed in relation to how QoE measurements are reported, which can impact RAN2 and CT1. To address this issue, we propose to liaise at least RAN2 and CT1 and we provide a draft LS in R3-244283. CT4, SA4 and SA5 should also remain in the loop.

Proposal 3: Agree the draft LS to RAN2 and CT1 (cc SA5, SA4 and CT4) in R3-244283.

# Conclusion

In this paper, we discuss the implications of the LS from SA5 in R3-243028. The following is observed and proposed:

Observation 1: In the LS in R3-243028, SA5 informs RAN3 that MBS Communication Service Type is used to indicate whether QoE measurements shall be performed for MBS broadcast or multicast.

Observation 2: Neither the RRC signalling nor the QoE measurement configuration XML file contain an indication of whether the UE should perform QMC in MBS broadcast or in MBS multicast mode.

Observation 3: Although MBS service type is an essential QoE configuration parameter, the MBS Service Announcement format does not guarantee its indication to the UE.

Proposal 1: The gNB should indicate to the UE via RRC whether QoE measurements shall be collected for MBS broadcast or multicast.

Observation 4: The UE does not report whether QoE measurements were collected for MBS multicast, implying that a consumer cannot distinguish between QoE measurements collected for unicast or MBS multicast or broadcast in case of UE mobility from a gNB that supports MBS to a gNB that does not support MBS.

Proposal 2: The consumer of QoE measurements should be informed about whether the measurements were collected for unicast, or for MBS broadcast, or MBS multicast.

Proposal 3: Agree the draft LS to RAN2 and CT1 (cc SA5, SA4 and CT4) in R3-244283.