3GPP TSG-RAN3 Meeting #125	R3-244285

Maastricht, Netherlands, Aug 19-23, 2024

Agenda Item:	8.3

Source: 	Qualcomm Incorporated

Title: 	Discussion on Emergency Call Support for (e)RedCap UEs in Barred Cell

Document for:	Discussion/Decision

# Background

In RAN3#124 meeting, RAN3 received an LS from RAN2 mentioning the changes RAN2 has done in their specification to support Emergency call for Redcap UEs on (e)RedCap UE barred Cell. The LS was briefly discussed in RAN3 and since few companies mentioned that the CRs are still under discussion in RAN2, RAN3 decided to address the LS in the subsequent meeting.

R3-243012 1. Overall Description: In RAN2#125bis RAN2 have endorsed the attached CRs for (e)RedCap UEs with the description of the CRs as below: If a (e)RedCap UE is barred in a cell where (e)RedCap is enabled since (e)RedCap UEs with 1Rx branch or 2Rx branches or both are barred in the cell, network may allow those (e)RedCap UEs to consider the cell as acceptable cell for emergency calls if cell selection criteria is fulfilled and, if the (e)RedCap UE supports only half duplex FDD operation, HD-FDD operation is allowed in the cell. The agreed RAN2 CRs also include the means for the cell to allow or not, the above operation via a SIB1 field. The CRs (R2-2402902, R2-2402903) when agreed, are considered to be implementable by Rel-17 UEs without any interoperability issue. 2. Actions: To RAN3 ACTION: RAN2 respectfully asks RAN3 to take the above information into account and update RAN3 specifications if needed.

In this paper we discuss our views on RAN3 impacts due to the RAN2 changes.

# Discussion

As mentioned in the LS, the SIB1 indicates whether (e)RedCap UEs are allowed for emergency call on the (e)RedCap UE barred cells. This indication needs to be propagated via Xn from neighbouring gNBs to decide on the neighbour to handover the Emergency calls. Hence Xn and F1 Served Cell information needs to contain emergency call support indication.

RAN3 agrees to include emergency call barring indicator for (e)RedCap UEs in Served Cell information IE for F1 and Xn interfaces for Emergency Call handover.

In the last meeting it was proposed by E/// to provide an indication from RAN to Core that (e)RedCap UE has triggered an Emergency call while on (e)RedCap UE barred cell. This way AMF can bar the UE from making a normal call while the emergency call is ongoing. However, we noticed that the RAN2 has agreed a CR to TS 38.304 for UE NAS to prohibit UE from establishing non-emergency calls on (e)RedCap barred cells. Hence we think no action is needed from Network side to prohibit an UE from establishing non-emergency calls on (e)RedCap barred cells.

R2-2403472 TS 38.304 - When cellBarredRedCap1Rx is set to “barred” in SIB1, a RedCap UE that supports only 1Rx branch can consider the cell as acceptable cell, only if cell selection criteria are fulfilled as defined in clause 5.2.3, cellBarred in MIB is not set to “barred” and in SIB1, barringExemptRedCap is set to “true” and, if the RedCap UE supports only half duplex FDD operation, halfDuplexRedCapAllowed is set to “true”; or - When cellBarredRedCap2Rx is set to “barred” in SIB1, a RedCap UE that supports 2Rx branches can consider the cell as acceptable cell, only if cell selection criteria are fulfilled as defined in clause 5.2.3, cellBarred in MIB is not set to “barred” and in SIB1, barringExemptRedCap is set to “true” and, if the RedCap UE supports only half duplex FDD operation, halfDuplexRedCapAllowed is set to “true”;

Based on the RAN2 CR R2-2403472, (e)RedCap UE NAS prohibits (e)RedCap UE from establishing non-emergency call on (e)RedCap barred cells. Hence additional indication over the Network (from RAN to Core) is not needed to ensure the same.

For an emergency call back, if the (e)RedCap UE is moved to RRC_Inactive, then an emergency call indication is needed in the Xn RAN Paging message, so that neighbouring gNBs can page on cells where emergency call is allowed but (e)RedCap UEs are barred.

RAN3 agrees to include (e)Redcap emergency call indicator in Xn RAN Paging message for emergency call back for neighbouring gNBs to page on cells where emergency barring exemption is allowed for RRC Inactive Paging for an emergency call back.

# Summary

Based on the discussion above, we have the following proposals:

Proposal 1:	RAN3 agrees to include emergency call barring indicator for (e)RedCap UEs in Served Cell information IE for F1 and Xn interfaces for Emergency Call handover.

Proposal 2:	Based on the RAN2 CR R2-2403472, (e)RedCap UE NAS prohibits (e)RedCap UE from establishing non-emergency call on (e)RedCap barred cells. Hence additional indication over the Network (from RAN to Core) is not needed to ensure the same.

Proposal 3:	RAN3 agrees to include (e)Redcap emergency call indicator in Xn RAN Paging message for emergency call back for neighbouring gNBs to page on cells where emergency barring exemption is allowed for RRC Inactive Paging for an emergency call back.