3GPP TSG-RAN3 Meeting #125	R3-244287

Maastricht, Netherlands, Aug 19-23, 2024

Agenda Item:	15.2

Source: 	Qualcomm Incorporated

Title: 	Discussion on Support for IoT NTN for Regenerative Payload

Document for:	Discussion/Decision

# Background

The IoT NTN WID objectives are RAN3 enhancements required to support Regenerative payload for IoT NTN with full eNB on board and to support Store and forward mechanisms in IoT NTN.

Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2, RAN4] -At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required.

In this paper we discuss our views on regenerative payload support for IoT NTN and Store and Forward support.

# Discussion

## Store and Forward support

The architecture options agreed in SA2 for regenerative payload in S2-2407186 has whole CN on board in the satellite and proxies of CN deployed in the ground. The proxies deployed in the ground are out of 3GPP scope. Another option is split MME with MME onboard and MME on ground. The MME on ground and the interface between the MMEs is out of 3GPP scope as agreed in S2-2407191.

S2-2407186 The whole CN including eNB, MME, SGW, PGW, HSS, E-SMLC, SMSC etc are on board each satellite. Proxies are deployed on the satellite and the ground for application traffic, including support of MT traffic, MO traffic, SMS, etc. - The implementation of the proxies and the interface between them is out of 3GPP scope. S2-2407191 1) In the Split MME architecture, HSS is on the ground. 2) MME functionality is split into two parts: MME-onboard - the MME part which is onboard the satellite and MME-ground – the MME part which is on the ground network with an interface out of scope of 3GPP.

In both the architecture options agreed in SA2, the whole eNB is on board and the S1 interface between eNB and MME is not impacted. RAN3 should analyse both the architecture options for RAN3 impact.

The architecture options in SA2 for S&F for Regenerative payload has whole CN on board as one option and split MME on board in another option. In both the option the whole eNB is on board and the S1 interface is also on board the satellite. In architecture option where the whole CN is on board, we see no RAN3 impact.

RAN3 to analyze both the architecture options (full CN on board and split MME) agreed in SA2 for RAN3 impact.

## Regenerative Payload

### S1 Connection Management

The agreement in NR NTN Regenerative Payload WI for NG Connection management can be replicated for S1 as well.

Agreement in NR NTN Regenerative payload for NG connection management can be replicated for S1 as well.

### Feeder Link Switch over

Feeder link switch over scenarios can be discussed in NR NTN. The agreements achieved in NR NTN can be replicated in IoT NTN as well. In S1, there is no support for multiple TNLs. Based on the agreement in NR NTN on multiple TNL support for Feeder link switch over, IoT NTN can be discussed further on how to address the lack of multiple TNL support.

Feeder link switch over scenarios can be discussed in NR NTN and the agreements in NR NTN can be replicated in IoT NTN. As S1 lacks multiple TNL associations, any agreement in made in NR NTN on the reuse of multiple TNL associations for FLSO, needs to be discussed in IoT NTN.

### Mapped Cell ID

The usage of Mapped Cell ID for Regenerative Architecture can be similar to that of Transparent Payload. NR NTN agreements on Mapped Cell ID can be replicated for IoT NTN as well.

The usage of Mapped Cell ID in Regenerative Payload can be similar to that of transparent payload.

# Summary

Based on the discussion above, we have following observations:

Observation 1.	The architecture options in SA2 for S&F for Regenerative payload has whole CN on board as one option and split MME on board in another option. In both the option the whole eNB is on board and the S1 interface is also on board the satellite. In architecture option where the whole CN is on board, we see no RAN3 impact.

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	RAN3 to analyze both the architecture options (full CN on board and split MME) agreed in SA2 for RAN3 impact.

Proposal 2:	Agreement in NR NTN Regenerative payload for NG connection management can be replicated for S1 as well.

Proposal 3:	Feeder link switch over scenarios can be discussed in NR NTN and the agreements in NR NTN can be replicated in IoT NTN. As S1 lacks multiple TNL associations, any agreement in made in NR NTN on the reuse of multiple TNL associations for FLSO, needs to be discussed in IoT NTN.

Proposal 4:	The usage of Mapped Cell ID in Regenerative Payload can be similar to that of transparent payload.