3GPP TSG-RAN3 Meeting #125	R3-244290

Maastricht, Netherlands, Aug 19-23, 2024

Agenda Item:	11.3

Source: 	Qualcomm Incorporated

Title: 	Discussion on AI/ML enabled CCO

Document for:	Discussion/Decision

# Background

In RAN#124 meeting, AI/ML based CCO was quite controversial. The CCO issue prediction is agreed to be in the gNB-CU, however there was no consensus in the CCO Configuration prediction. Companies didn’t agree to have AI/ML capabilities in the gNB-DU. However, it was agreed that for monolithic gNB the future CCO configuration can be derived via AI/ML or legacy mechanisms.

RAN3#124 For non-split architecture, future CCO state can be derived via legacy mechanism or AI/ML techniques which is up to implementation. For split architecture, the standardization of AI/ML based future CCO state inference in the gNB-DU is not supported in R19, how to derive future CCO state is up to implementation in the gNB-DU, e.g., legacy or AI/ML. Time information to be further discussed in next meeting. Finalize the TP and conclusion.

In this paper we discuss our views on the future CCO configuration and CCO configuration feedback.

# Discussion

## Future CCO Configuration

In the last meeting it was agreed that future CCO configuration can be derived using AI/ML or legacy mechanisms. However, companies raised concern that future CCO cannot be provided as output of AI/ML based CCO. We would like to have atleast for non-split architecture, future CCO as the output of AI/ML based CCO.

RAN3 to agrees to add “Future CCO state” as the output data of AI/ML based CCO for non-split architecture. Agree to TP in R3-244291

The future CCO state can be associated with a time when the CCO state will be applied. The time information indicates to the neighbouring when the new CCO state will be in effect.

RAN3 to agrees to include Time information along with Future CCO state for AI/ML based CCO.

There could be different scenarios like energy savings, interference etc due to which the CCO configuration is changed. When the CCO configuration is changed in a particular gNB, it impacts the other gNBs in the neighbourhood as well. For example, when one gNB shrinks its coverage due to NES, then the neighbouring gNB needs to expand its coverage to avoid a coverage gap.

Sample Call flow is shown below –

There could be energy savings, interference or other scenarios due to which coverage is modified in a gNB and it impacts the coverage state of the neighbouring gNBs, One example could be a gNB shrinks its coverage for NES and neighbouring gNB needs to expand its coverage to avoid coverage gap.

A gNB can propose a predicted coverage configuration for cells served by its neighboring gNBs. The neighboring gNB can either accept/reject this proposal and provide its own predicted coverage configuration upon request. To include “Assistance Information on Future CCO state of neighbour” in the Xn Impact section of the TR 38.743 for AI/ML based CCO.

## Predicted CCO Configuration Feedback

Once the predicted coverage pattern is applied, the neighboring gNBs can send feedback on Cell performance like throughput, packet loss, packet delay, RLF reports etc to the source gNB to inform the effects of applying predicted coverage pattern. The Feedback can be transferred from gNB-CU to gNB-DU to correct the CCO configuration.

The following parameters can be considered for feedback for the predicted CCO configuration and can be exchanged between neighbours over Xn and from gNB-CU to gNB-DU over F1.

- Cell throughput (DL/UL)

- Cell level Packet loss

- Cell level Packet delay

- SON reports (RLF, CEF, RA)

## Assistance Information for Prediction of CCO Issue

It was agreed the gNB-CU will predict the CCO issue and forward it to the gNB-DU. gNB-CU predicts the CCO issue based on L3 measurements. gNB-DU has L1 measurements and beam level information. Hence gNB-DU has a better visibility of the beam level information for detecting CCO issue. We therefore propose that gNB-DU can send affected beams based on L1 measurements as assistance information to gNB-CU.

gNB-CU uses L3 measurement information as one of the inputs to predict CCO issue. However gNB-CU lacks insight into the beam level information which is available in the gNB-DU. gNB-DU has better visibility of beam level information based on L1 measurements for detecting CCO issue.

For CCO issue prediction, gNB-DU can send assistance information to gNB-CU on affected beams based on L1 measurements. To include “Assistance Information on Affected beams” in the F1 Impact section of the TR 38.743 for AI/ML based CCO.

# Summary

Based on the discussion above, we have following observations:

Observation 1.	There could be energy savings, interference or other scenarios due to which coverage is modified in a gNB and it impacts the coverage state of the neighbouring gNBs, One example could be a gNB shrinks its coverage for NES and neighbouring gNB needs to exand its coverage to avoid coverage gap.

Observation 2.	gNB-CU uses L3 measurement information as one of the inputs to predict CCO issue. However gNB-CU lacks insight into the beam level information which is available in the gNB-DU. gNB-DU has better visibility of beam level information based on L1 measurements for detecting CCO issue.

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	RAN3 to agrees to add “Future CCO state” as the output data of AI/ML based CCO for non-split architecture. Agree to TP in R3-244291

Proposal 2:	RAN3 to agrees to include Time information along with Future CCO state for AI/ML based CCO.

Proposal 3:	A gNB can propose a predicted coverage configuration for cells served by its neighboring gNBs. The neighboring gNB can either accept/reject this proposal and provide its own predicted coverage configuration upon request. To include “Assistance Information on Future CCO state of neighbour” in the Xn Impact section of the TR 38.743 for AI/ML based CCO.

Proposal 4:	The following parameters can be considered for feedback for the predicted CCO configuration and can be exchanged between neighbours over Xn and from gNB-CU to gNB-DU over F1.

	Cell throughput (DL/UL)

	Cell level Packet loss

	Cell level Packet delay

	SON reports (RLF, CEF, RA)

Proposal 5:	For CCO issue prediction, gNB-DU can send assistance information to gNB-CU on affected beams based on L1 measurements. To include “Assistance Information on Affected beams” in the F1 Impact section of the TR 38.743 for AI/ML based CCO.