3GPP TSG-RAN3 Meeting #125	R3-244291

Maastricht, Netherlands, Aug 19-23, 2024

Agenda Item:	11.3

Source: 	Qualcomm Incorporated, ZTE, CATT, SS, CMCC

Title: 	(TP for TR 38.743) AI/ML enabled CCO

Document for:	Discussion/Decision

# Introduction

This paper contains the TP to capture the agreement for the AI/ML assisted CCO.

# Text proposal to TR 38.743

## 4.2	AI/ML based Coverage and Capacity Optimization

### 4.2.1	Use case description

Editor Note: Capture the description of use case

The objective of NR Coverage and Capacity Optimization (CCO) function is to detect and resolve or mitigate CCO issues. An NG-RAN node may autonomously adjust within and switch among coverage configurations. When a change is executed, a NG-RAN node may notify its neighbour NG-RAN nodes with the list of cells and SSBs with modified coverage included.

In the legacy CCO solution, a reactive approach is used: when the gNB (gNB-CU in case of CU-DU split architecture) detects a CCO issue which negatively impacts network and UE performance after it has already occurred, the gNB (gNB-DU in case of CU-DU split architecture) attempts to resolve or mitigate it.

With an AI/ML based CCO, a more proactive approach is used to prevent (or limiting at an early stage) the rise of a CCO issue with the consequent degradation of network (and UE) performance.

### 4.2.2	Solutions and standard impacts

Editor Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

#### 4.2.2.1 Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions can be considered for supporting AI/ML-based CCO:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

- AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

- AI/ML Model Training and Model Inference are both located in the gNB-CU.

#### 4.2.2.2		Input data of AI/ML based CCO

For a proactive prediction and resolution of a CCO issue, a gNB may need the following information as input data for AI/ML-based CCO:

From local node:

-	Measured/Predicted radio resource status

-	Current CCO State

From neighbouring gNBs:

-	Measured/Predicted radio resource statusFrom the UE:

-	UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

-	SON Reports (e.g., RLF, CEF, RA)

#### 4.2.2.3	Output data of AI/ML based CCO:

AI/ML-based CCO model in a gNB can generate following information as output:

-	Predicted CCO issue

-	Future CCO State for non-split architecture

#### 4.2.2.4	Feedback of AI/ML based CCO:

To optimize the performance of AI/ML-based CCO model, following feedback can be considered to be collected from gNBs:

- Measured radio resource status

- Legacy UE performance feedback for those UEs handed over from the source gNB

- SON Reports (e.g., RLF, CEF, RA)

#### 4.2.2.5	Potential standard impacts:

Following standard impacts are listed for subsequent Rel-19 normative work compared with what was specified during Rel-18.

Xn interface:

- Future CCO State

F1 interface:

- Predicted CCO Issue (including predicted cells/beams)