3GPP TSG-RAN3 Meeting #125	R3-244292

Maastricht, Netherlands, Aug 19-23, 2024

Agenda Item:	11.2

Source: 	Qualcomm Incorporated

Title: 	Discussion on NG-RAN AI/ML for Network Slicing

Document for:	Discussion/Decision

# Background

In RAN3#124 we almost made agreements on the open issues with respect to exchanging prediction with CN and slice based UE performance. The pending open issue is with respect to Slice SLA predictions.

RAN3#124 In Rel-19, predicted information from CN to NG-RAN is not considered. Agree to improve the granularity of UE performance feedback in Rel-19 for slicing. FFS whether to include measured/predicted UE throughput per slice as input from local node. FFS whether to introduce delivery of slice level UE throughput prediction/measurement during handover procedure. FFS on SLA requirements fulfilment information exchange. Finalize the TP and conclusion.

In this paper we discuss our views on the Slice SLA prediction and UE throughput per slice.

# Discussion

## Slice SLA Prediction

Every Slice has an Service level assurance associated with it and there are Accessibility, Integrity and Utilization metrics associated with the Slice. These KPIs define whether Slice SLA can be achieved or not. These Slice KPIs are generated by the gNB. Based on the resource availability in a particular slice, traffic conditions in the slice and the Slice KPIs (as provided in TS 28.544) generated, gNB can predict if slice SLA can be met. If the OAM knows whether the Slice KPI can be met or not, OAM can adjust the slice resource allocations and slice selection accordingly to ensure SLAs can be met.

gNB can exchange Slice SLA prediction with the OAM (slice SLA assurance service) and OAM can inform the NSSF and NSACF service to take precautionary actions (like slice selection control and slice admission control) to meet the Slice SLA. Alternatively gNB can also exchange measured Slice SLA fulfilment with OAM, so that OAM is aware if the node cannot meet the slice SLA requirements and can take precautionary measures. gNB can also exchange measured and predicted Slice SLA fulfilment with neighbouring gNBs for proactive Slice SLA management.

Based on the slice resource availability, traffic conditions in a particular slice and the Slice KPI metrics (accessibility, Integrity and utilization) as specified in TS 28.554, gNB can predict if Slice SLA requirements can be fulfilled.

RAN3 to agree that predicted and measured Slice SLA fulfillment from RAN can be exchanged with OAM and neighbouring gNBs for proactive Slice SLA management.

## Measured UE Slice Traffic

In the last meeting it was agreed to improve the UE performance metrics so that the reporting can indicate metrics at slice level. The UE throughput, packet loss and packet delay can be reported at slice level to assist the node with slice availability and capacity prediction. These metrics can be reported from CU-UP to CU-CP and from DU to CU-CP within the local node. The UE performance feedback collected can be enhanced to report the above metrics over Xn

Measured UE throughput, packet loss and packet delay at slice level can be shared over F1 and E1 within the local node as input and feedback for slice-based resource status prediction

UE performance feedback can be enhanced over Xn by providing Measured UE throughput, packet loss and packet delay at slice level to assist the accuracy of Slice related metrics prediction.

It was also discussed in the last meeting if it is useful to send the UE throughput per slice to the target during Handover. This can assist the target to understand the UE traffic and also to allocate resources accordingly. It was also discussed whether predicated UE throughput per slice can be send in the handover. The problem with predicting the UE throughput is that the source does not have UE traffic history to predict the UE traffic. When the UE goes Idle the UE cannot be identified in RAN. Hence gNB does not have any past information of the UE to train the ML model to predict the UE Traffic.

gNB does not have any past information on the UE Traffic history to predict the UE throughput per slice and provide it in the handover to the target.

On the other hand, if measured UE throughput per slice is sent to target from source during handover then target can –

- Allocate resources based on the UE throughput

- Choose the right slice based on UE throughput

- Predict the slice resources and available capacity

Introduce Measured UE throughput per slice in Handover Request message for target to allocate radio resources based on the UE throughput in the source.

# Summary

Based on the discussion above, we have following observations:

Observation 1.	Based on the slice resource availability, traffic conditions in a particular slice and the Slice KPI metrics (accessibility, Integrity and utilization) as specified in TS 28.554, gNB can predict if Slice SLA requirements can be fulfilled.

Observation 2.	gNB does not have any past information on the UE Traffic history to predict the UE throughput per slice and provide it in the handover to the target.

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	RAN3 to agree that predicted and measured Slice SLA fulfillment from RAN can be exchanged with OAM and neighbouring gNBs for proactive Slice SLA management.

Proposal 2:	Measured UE throughput, packet loss and packet delay at slice level can be shared over F1 and E1 within the local node as input and feedback for slice-based resource status prediction

Proposal 3:	UE performance feedback can be enhanced over Xn by providing Measured UE throughput, packet loss and packet delay at slice level to assist the accuracy of Slice related metrics prediction.

Proposal 4:	Introduce Measured UE throughput per slice in Handover Request message for target to allocate radio resources based on the UE throughput in the source.