3GPP TSG-RAN WG3 Meeting #125	R3-244294

Maastricht, Netherlands, 19th – 23rd August, 2024

Title: 	(TP to TR 38.743) Discussion on the AI/ML-enabled Network Slicing

Source: 	Huawei

Agenda item:	11.2

Document Type:	Discussion and Decision

# 1. Introduction

In RAN3#124 meeting the following agreements for the AI/ML-based Network Slicing use case were reached [1], and a new TP was agreed in [2].

In Rel-19, predicted information from CN to NG-RAN is not considered. Agree to improve the granularity of UE performance feedback in Rel-19 for slicing. FFS whether to include measured/predicted UE throughput per slice as input from local node. FFS whether to introduce delivery of slice level UE throughput prediction/measurement during handover procedure. FFS on SLA requirements fulfilment information exchange.

This paper further discusses the AI/ML-based Network Slicing use case based on the latest meeting progress.

# 2. Discussion

## 2.1 Solutions for AI/ML-based Network Slicing

This section starts with the discussion for the FFS aspects from the last RAN3 meeting in the Chairman’s notes [1] and in the TP to TR 38.743 [2]. In the end, we also propose some potential solutions for the AI/ML-based Network Slicing use case to further improve the resource management and mobility decisions performance based on the current agreed TP.

- FFSs in the RAN3#124 Chairman’s notes [1]:

FFS whether to include measured/predicted UE throughput per slice as input from local node. FFS whether to introduce delivery of slice level UE throughput prediction/measurement during handover procedure. FFS on SLA requirements fulfilment information exchange.

- FFS in the TP [2]:

Finer granularity UE performance feedback for those UEs handed over from the source gNB. FFS on the exact level of finer granularity.

## 2.1.1 Measured/predicted UE throughput

Based on the first two FFSs, the slice level measured/predicted UE throughput can be used in two cases.

- Case 1: the local node (serving the UE) can measure or predict the slice level UE throughput and use this information internally as input data to the AI/ML model in local node for other predictions, e.g., slice available capacity and/or per-slice radio resource status.

- Case 2: the neighbour node (serving the UE) can measure or predict the slice level UE throughput and send this information to the local node over Xn interface to be used as input information to the AI/ML model in local node. This should happen during the handover procedure since the local node is interested in the UEs that are going to be handed over to it.

In Case 1, the local node can first predict the UE throughput based on the measured information by implementation internally. The measured and predicted UE throughput remains internal to the local node and can be used by the AI/ML model in the local node to predict e.g. slice available capacity and/or per-slice radio resource status, which are already agreed to be the input data and output data for AI/ML based Network Slicing.

Observation 1. The local node can predict the UE throughput based on the corresponding measured information. The measured and predicted UE throughput can be used internally in the local node (hosting the AI/ML model) to predict e.g. slice available capacity and/or per-slice radio resource status.

Proposal 1. The measured/predicted UE throughput per slice from local node is not sent over Xn and can be used internally as the input data for AI/ML based Network Slicing in the local node.

While, in our understanding, the intention of Case 2 is for optimizing further the resource management in the local (target) node. In a mobility scenario, if the source node can provide some predicted slice resource requirement for the UE to target node, the target node would be able to prepare radio resources in advance. We think that the current/predicted QoS requirement can be more helpful compared to the measured/predicted UE throughput. First, the UE throughput is hard to be predicted, if the UE traffic is not periodical. Second, the QoS requirement can provide a more comprehensive view on the UE performance that are expected to be ensured after the handover, which includes the requirement on the delay, maximum bit rate, loss rate, and so on. Therefore, we propose the following UE information which can be useful for resource management and RRM decisions:

- Per-UE current/predicted QoS requirement for each slice (from both local and neighbour nodes).

Observation 2. Comparing with the measured/predicted UE throughput, the current/predicted QoS requirement can be more helpful to be provided to the target node for resource management and mobility decisions.

Proposal 2: The per-UE current/predicted QoS requirement for each slice (from both local and neighbour nodes) can be used as the AI/ML input.

## 2.1.2 Granularity of UE performance feedback

RAN3 has agreed to improve the granularity of UE performance feedback for slicing [2].

Finer granularity UE performance feedback for those UEs handed over from the source gNB. FFS on the exact level of finer granularity.

Based on the discussion from last meeting, we have the following two candidate solutions [3].

- QoS-flow-group-level

- slice level UE performance

The motivation to introduce the per QoS-flow-group-level UE performance is that the average of UE performance metrics (throughput, packet loss and packet delay) over all configured DBRs as done in Rel-18 may not reflect the actual status of each QoS flow after the handover. It happens when the traffic of the QoS flows fluctuates dramatically, thus causing the average UE performance to change dramatically as well, as shown in the example in [3]. Based on the Rel-18 mechanism, the target node feeds back the UE performance collected within a very short time window after the handover, e.g., less than 1000ms. We think it can be kind of a corner case that the traffic associated to QoS flows of the UE fluctuates dramatically in this short time window immediately after handover. Moreover, we should also consider that there could be a QoS flow – to – DRB remapping at the target node side after the handover, which could make the collection and reporting of UE performance at QoS flow level even more meaningless due to the fact that the remapping is completely under the control of the target node. Hence, we think that RAN3 needs to further discuss whether to introduce the QoS-flow-group-level UE performance collection and reporting.

Observation 3: It can be kind of a corner case that the traffic associated to QoS flows of the UE fluctuates dramatically in a short time window immediately after the handover. The possible QoS flow – to – DRB remapping at the target node side after the handover would also make the UE performance at QoS flow level meaningless at the source side.

Proposal 3: RAN3 needs further discuss whether to introduce the QoS-flow-group-level UE performance collection and reporting.

However, we should consider the slice level UE performance instead, since it can be helpful to know whether a slice can ensure the UE performance after the handover. This feedback information can be sent from the target node to the source node, which can be used by the source node to optimize the AI/ML model so that better handover decisions can be performed.

Proposal 4: RAN3 to agree that the slice-level UE performance are used as feedback information for the AI/ML-based Network Slicing in Rel-19, including e.g.,

- slice level average UE throughput in the DL/UL

- slice level average packet delay

- slice level average packet Loss in the DL.

## 2.1.3 SLA fulfilment level indication

One more FFS is shown below concerning the slice SLA requirements fulfilment information exchange.

FFS on SLA requirements fulfilment information exchange.

As defined in TS 28.541, the slice-specific service requirements are agreed between the customer and the provider based on the slice SLA. Hence, it needs to be guaranteed that certain KPI requirements derived from the slice SLA are met by the network. If a node/cell cannot ensure KPIs are met for traffic belonging to a slice, then the overall network performance for the slice are negatively impacted and there could be the risk that the slice SLA is violated. Therefore, the most straightforward solution is to offload some slice traffic to a neighbour cell/node so to improve the situation.

In past RAN3 meetings some companies proposed to introduce the slice SLA requirements fulfilment level indication to be exchanged on the Xn interface [4][5] – and also between the gNB and OAM [5] – as an additional metric to decide whether to perform the traffic offloading or not. Based on the TP in [2], RAN3 already agreed to exchange the measured/predicted radio resource status per slice and the measured/predicted slice available capacity on the Xn interface, thus source gNB can already assess whether to perform UE offloading based on slice available capacity prediction and radio resource status prediction information which are sent from the target to the source. However, in the discussion paper [4], it was claimed that spare Slice Available Capacity in the target node does not necessarily mean that the SLA requirements are met, for instance in the following cases:

- There are many UEs at cell edge and, depending on the implementation, the RAN node may decide not to use all the resources available towards such cell edge UEs because this may still result in a lack of slice requirements fulfilment. Instead, it is better to use those resources to serve other UEs in better radio conditions;

- F1-U delays are excessive, preventing fulfilment of slice delay requirement.

In our understanding, the first case can be regarded as a corner case: when a UE is handed over from the source to the target, it is unusual that such UE will be in the cell edge. Moreover, it seems unrealistic that, in case a lot of UEs are offloaded, the majority of such UEs will result in a typical cell-edge condition from radio perspective. Then, for the second case, fulfilling the slice delay requirement of UEs depends on the resource availability at the target node side, including radio resources and hardware/software resources. The F1 interface can be implemented as a wireline or wireless link and, typically, it is the radio interface which is the bottleneck, but not the resources on the F1 interface if F1 is designed properly. Furthermore, the gNB is not actually requested to help the neighbouring nodes in guaranteeing the slice SLA requirements fulfilment.

Observation 4: The source gNB can already assess whether to perform UE offloading based on slice available capacity prediction and radio resource status prediction information which are sent from target to the source.

Observation 5: The gNB is not requested to help the neighbouring nodes in guaranteeing the slice SLA requirements fulfilment.

Proposal 5: RAN3 to agree that the slice SLA requirements fulfilment level indication (both measured and predicted) is not needed to be transferred over the Xn interface.

For the latter case, that is, exchanging the slice SLA requirements fulfilment level from gNB to OAM, we noticed that OAM can already configure the RAN to get the necessary information from the network nodes for slice operations and administration purposes, e.g., the end-to-end KPIs can be provided to the network operator as indicated in TS 38.554 [6] so that it can be assessed whether slice SLA are met. Moreover, since the OAM can host the training function for AI/ML models to then be deployed in NG-RAN, the OAM itself can make prediction and have better predicted result related to slice SLA requirements fulfilment compared to a kind of binary indication representing the measured/predicted slice SLA requirements fulfilment level sent by the NG-RAN. Therefore, we think it is better to rely on slice-related KPI provided by RAN to OAM rather than the measured/predicted slice SLA requirements fulfilment level indication.

Observation 6: OAM can already configure the RAN to get information from the network nodes for slice operations and administration purposes, hence it can rely on more meaningful information compared to the slice SLA requirements fulfilment level indication.

Proposal 6:  RAN3 to agree that the slice SLA requirements fulfilment level indication (both measured and predicted) is not needed to be exchanged between gNB and OAM.

## 2.1.4 Potential solutions for the AI/ML model

According to the latest TP [2], the AI/ML function can analyse network-level and UE-level performance metrics to help improving and optimizing the resource management and mobility decisions for slicing. The cell-level predicted information for slicing include:

- Predicted radio resource status per slice

- Predicted slice available capacity.

In current XnAP specification, we have per-slice radio resource information exchanged over the Xn interface via the Resource Status Reporting procedure, and it is also feasible for the NG-RAN node to predict (and report via the Data Collection Reporting procedure) the following per-cell per-slice information:

- Predicted PRB usage of a cell

- Predicted number of connected UEs

- Predicted RRC connections.

We think the above information, along with the corresponding measured counterparts, all provided by both the local node (hosting the AI/ML model) and the neighbour nodes, can also be used to evaluate the load information on the indicated slices in the future, based on which NG-RAN node can make mobility, load balancing and Radio Resources Management decisions. It is worth to note that the above-listed metrics can already be exchanged, hence there will be no standard impact to make these items available at the AI/ML model in the local node.

Proposal 7: The following cell-level slice-level prediction information can be used for the AI/ML-based Network Slicing, provided by both the local and the neighbour nodes, with no additional standard impact:

- Measured/Predicted PRB usage of a cell

- Measured/Predicted number of connected UEs

- Measured/Predicted RRC connections.

## 2.2 Clarification for AI/ML-based Network Slicing Use case

In the latest version of the draft TR 38.743 [7], the description for AI/ML-based Network Slicing use case is as follows:

## 4.1	AI/ML based Network Slicing

### 4.1.1	Use case description

Editor Note: Capture the description of use case

Support of network slicing in NG-RAN is defined in TS38.300 [x].

The NG-RAN plays a key role in taking mobility, load balancing and Radio Resources Management decisions for the purpose of meeting target requirements derived from the SLA of each supported network slice.

AI/ML function can analyze metrics related to network and UE level performance related to perform optimal resource management and mobility decisions for network slicing to meet the requirements.

In current RAN specifications, NG-RAN nodes can exchange the slice-related information, such as the supported slice information, the load measurements and the slice available capacity for each slice. NG-RAN nodes can use such information to perform mobility, load balancing and Radio Resources Management decisions as stated above in the TR.

With the AI/ML function, NG-RAN can use predicted slice information to improve and optimize in advance the above mobility, load balancing and Radio Resources Management aspects. However, in the current TR, the load balancing part is missing, which we think should be added. For example, with the predicted radio resource status per slice and the predicted slice available capacity of each cell locally and from neighbouring nodes, the NG-RAN can perform some decision for load balancing.

Observation 7: For network slicing, the AI/ML function can also help to improve the load balancing decision of NG-RAN nodes, which is missing in the current use case description of the AI/ML-based Network Slicing use case.

Proposal 8: The predicted slice information (radio resource status per slice and slice available capacity per cell) can be used for load balancing decisions.

A TP for the TR 38.743 reflecting the above proposals is provided in the Annex of this paper.

# 3. Conclusion

Based on the above discussion, we have the following observation and proposals:

Solutions for AI/ML-based Network Slicing

Observation 1. The local node can predict the UE throughput based on the corresponding measured information. The measured and predicted UE throughput can be used internally in the local node (hosting the AI/ML model) to predict e.g. slice available capacity and/or per-slice radio resource status.

Proposal 1. The measured/predicted UE throughput per slice from local node is not sent over Xn and can be used internally as the input data for AI/ML based Network Slicing in the local node.

Observation 2. Comparing with the measured/predicted UE throughput, the current/predicted QoS requirement can be more helpful to be provided to the target for resource management and mobility decisions.

Proposal 2: The per-UE current/predicted QoS requirement for each slice (from both local and neighbour nodes) can be used as the AI/ML input.

Observation 3: It can be kind of a corner case that the traffic associated to QoS flows of the UE fluctuates dramatically in a short time window immediately after the handover. The possible QoS flow – to – DRB remapping at the target node side after the handover would also make the UE performance at QoS flow level meaningless at the source side.

Proposal 3: RAN3 needs further discuss whether to introduce the QoS-flow-group-level UE performance collection and reporting.

Proposal 4: RAN3 to agree that the slice-level UE performance are used as feedback information for the AI/ML-based Network Slicing in Rel-19, including e.g.,

- slice level average UE throughput in the DL/UL

- slice level average packet delay

- slice level average packet Loss in the DL.

Observation 4: The source gNB can already assess whether to perform UE offloading based on slice available capacity prediction and radio resource status prediction information which are sent from target to the source.

Observation 5: The gNB is not requested to help the neighbouring nodes in guaranteeing the slice SLA requirements fulfilment.

Proposal 5: RAN3 to agree that the slice SLA requirements fulfilment level indication (both measured and predicted) is not needed to be transferred over the Xn interface.

Observation 6: OAM can already configure the RAN to get information from the network nodes for slice operations and administration purposes, hence it can rely on more meaningful information compared to the slice SLA requirements fulfilment level indication.

Proposal 6:  RAN3 to agree that the slice SLA requirements fulfilment level indication (both measured and predicted) is not needed to be exchanged between gNB and OAM.

Proposal 7: The following cell-level slice-level prediction information can be used for the AI/ML-based Network Slicing, provided by both the local and the neighbour nodes, with no additional standard impact:

- Measured/Predicted PRB usage of a cell

- Measured/Predicted number of connected UEs

- Measured/Predicted RRC connections.

Clarification for AI/ML-based Network Slicing Use case

Observation 7: For network slicing, the AI/ML function can also help to improve the load balancing decision of NG-RAN nodes, which is missing in the current use case description of the AI/ML-based Network Slicing use case.

Proposal 8: The predicted slice information (radio resource status per slice and slice available capacity per cell) can be used for load balancing decisions.

# 4. Reference

- 3GPP RAN3#124 Chairman’s notes.

- R3-243918, TP to TR 38.743 on support of AI/ML enabled slicing, CATT, Huawei, Nokia, ZTE, Samsung, NEC, CMCC, Ericsson, Lenovo, 3GPP RAN3#124.

- R3-243578, Discussion on definition of UE performance metrics, CATT, Deutsche Telekom, Nokia, China Telecom, China Unicom.

- R3-243416, AIML for Network Slicing, Ericsson, InterDigital, Deutsche Telekom, Telecom Italia.

- R3-243558, Discussion on NG-RAN AI/ML for Network Slicing, Qualcomm Incorporated.

- 3GPP TS 38.554, 5G end to end Key Performance Indicators (KPI), v18.5.0.

- R3-243971, 3GPP TR 38.743 Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, Release 19, V0.0.1

# 5. Annex – TP to TR 38.743 (based on R3-243971)

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 4.1	AI/ML based Network Slicing

### 4.1.1	Use case description

Editor Note: Capture the description of use case

Support of network slicing in NG-RAN is defined in TS38.300 [x].

The NG-RAN plays a key role in taking mobility, load balancing and Radio Resources Management decisions for the purpose of meeting target requirements derived from the SLA of each supported network slice.

AI/ML function can analyze metrics related to network and UE level performance to perform optimal resource management, mobility and load balancing decisions for network slicing to meet the slicerequirements.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 4.1.2.4	Input data of AI/ML based Network Slicing:

To predict the optimized network slicing decisions, a gNB may need the following information as input data for AI/ML-based network slicing:

From local node:

-	Measured/Predicted radio resource status per slice

-	Measured/Predicted slice available capacity

-	Legacy predicted UE trajectory

-	Current/Predicted QoS requirement per UE per slice

-	Measured/Predicted PRB usage of a cell

-	Measured/Predicted number of connected UEs

-	Measured/Predicted RRC connections

From neighbouring gNBs:

-	Measured/Predicted radio resource status per slice

-	Measured/Predicted slice available capacity

-	Current/Predicted QoS requirement per UE per slice

-	Measured/Predicted PRB usage of a cell

-	Measured/Predicted number of connected UEs

-	Measured/Predicted RRC connections

From the UE:

-	UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

#### 4.1.2.5	Output data of AI/ML based Network Slicing:

AI/ML-based network slicing model in a gNB can generate the following information as output:

- Predicted radio resource status per slice

- Predicted slice available capacity

- Resource management decisions for resources within RRM policies (used by gNB internally)

- Slice aware mobility decisions (used by gNB internally)

#### 4.1.2.6	Feedback of AI/ML based Network Slicing:

To optimize the performance of AI/ML-based network slicing model, the following feedback can be considered to be collected from gNBs:

- Measured Radio resource status per slice

- Measured Slice available capacity

- Legacy UE performance feedback for those UEs handed over from the source gNB

- Slice level UE performance feedback for those UEs handed over from the source gNB

#### 4.1.2.7	Potential standard impacts:

Following standard impacts is listed for subsequent Rel-19 normative work compared with what was specified during Rel-18.

Xn interface:

- Enhanced existing procedure to collect predicted information between gNBs:

  - Predicted radio resource status per slice

  - Predicted slice available capacity

  - Current/Predicted QoS requirement per UE per slice

  - Measured/Predicted PRB usage of a cell

  - Measured/Predicted number of connected UEs

  - Measured/Predicted RRC connections