3GPP TSG-RAN WG3 Meeting #125	R3-244295

Maastricht, Netherlands, 19th – 23rd August, 2024

Title: 	(TP to TR 38.743) AI/ML-based Coverage and Capacity Optimization

Source: 	Huawei

Agenda item:	11.3

Document Type:	Discussion and Decision

# 1. Introduction

A new SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” was approved for Rel-19 at RAN#102 meeting [1], then revised in RAN#103 and eventually approved in [2]. One of the objectives of the new SI is to investigate the AI/ML-based Coverage and Capacity Optimization (CCO) use case and identify enhancements in NG-RAN to support the necessary AI/ML functionalities. The agreements reached in RAN3#123bis are as follows [3]:

For AI/ML based CCO,

-	AI/ML Model Training may be located in the OAM and AI/ML Model Inference may be located in the NG-RAN node (gNB-CU).

-	AI/ML Model Training and AI/ML Model Inference may be both located in the NG-RAN node (gNB-CU).

Solution for non-split architecture

Step 0: gNB predicts the CCO issue.

Step 1: gNB generates the future coverage status based on the predicted CCO issue and other information.

Step 2: gNB sends the future coverage status to neighbour gNBs.

Solution for split-architecture

Step 0: gNB-CU predicts the CCO issue

Step 1: gNB-CU sends the predicted CCO issue to gNB-DU.

Step 2: gNB-DU generates the future coverage status based on the predicted CCO issue and other local information, whether only local information is used can be further discussed.

Step 3: gNB-DU sends the future coverage status to gNB-CU.

Step 4: gNB-CU sends the future coverage status to neighbour gNBs.

While, in the last meeting, the determination of the future CCO state (as a consequence of a predicted CCO issue) for both split and non-split architecture was heavily discussed and the following agreements were achieved [4]:

For non-split architecture, future CCO state can be derived via legacy mechanism or AI/ML techniques which is up to implementation.

For split architecture, the standardization of AI/ML based future CCO state inference in the gNB-DU is not supported in R19, how to derive future CCO state is up to implementation in the gNB-DU, e.g., legacy or AI/ML.

There is however an FFS aspect that should be dealt with, i.e., [4]:

Time information to be further discussed in next meeting.

This contribution investigates the AI/ML-based CCO further, and provides our considerations concerning the FFS aspect above.

# 2. Discussion

## 2.1 High-level principles for AI/ML-based CCO

In the legacy CCO mechanism, for the split architecture:

- the gNB-CU detects the CCO issue (i.e., either coverage issue or cell edge capacity issue) and affected cells/SSBs;

- the gNB-CU sends the CCO Assistance Information IE (within the GNB-CU CONFIGURATION UPDATE message over F1) containing the CCO issue type and affected cells/SSBs to the gNB-DU;

- the gNB-DU uses the received CCO Assistance Information IE to determine the new coverage state (CCO state) for the affected gNB-DU’s cells/SSBs;

- the gNB-DU sends the Coverage Modification Notification IE (within the GNB-DU CONFIGURATION UPDATE message over F1) containing the new CCO state for the affected gNB-DU’s cells/SSBs to the gNB-CU;

- the gNB-CU sends the Coverage Modification List IE (within the NG-RAN NODE CONFIGURATION UPDATE message over Xn) containing the detected CCO issue type and the new CCO state for the affected gNB-DU’s cells/SSBs to the neighbour gNB to allow for the neighbour gNB to determine a new CCO state for its own cells/SSBs that best matches the new CCO state of the gNB-CU’s cells/SSBs resulting from the detected CCO issue.

For the non-split architecture, instead, steps b. to d. above are internal to the gNB, hence there is no standard impact, and steps a. and e. still hold (with gNB replacing gNB-CU / gNB-DU).

For the AI/ML-based CCO use case, the main principle which drove the discussion in RAN3#123bis [3] is that the gNB(-CU) predicts the CCO issue and, as a consequence, the gNB(-DU) generates the future CCO state of affected cells/SSBs based on (at least) the predicted CCO issue. However, the way the gNB(-DU) determines the future CCO state, i.e., via legacy mechanism or via AI/ML was left for discussion and decision in RAN3#124 meeting.

Therefore, in the last meeting, RAN3 made the following agreements, for both non-split and split architectures, concerning how to determine the future CCO state (as a consequence of a predicted CCO issue) [4]:

For non-split architecture, future CCO state can be derived via legacy mechanism or AI/ML techniques which is up to implementation.

For split architecture, the standardization of AI/ML based future CCO state inference in the gNB-DU is not supported in R19, how to derive future CCO state is up to implementation in the gNB-DU, e.g., legacy or AI/ML.

While for the split architecture it is clearly stated that there is no standard support for the inference of the future CCO state (in the gNB-DU) in Rel-19, for the non-split architecture it is only stated that the future CCO state can be derived via either legacy mechanism or AI/ML techniques depending on gNB’s implementation.

Therefore, the agreement for the non-split architecture could be interpreted in such a way that, for the case the future CCO state is derived via AI/ML – and hence the future CCO state is actually a predicted CCO state – such AI/ML-derived future CCO state should be listed as an output information of the model used for the AI/ML-based CCO use case, which in turn implies that there should be standardization support of the AI/ML-based future CCO state inference in the gNB because the inference process might require some input data that needs to be collected and sent to the model over RAN interfaces (Xn).

Observation 1: With the current wording of the agreement for the non-split architecture concerning the derivation of the future CCO state (as a consequence of a predicted CCO issue) there could be the risk of interpreting the agreement in such a way that there is a need for standardization support to enable the AI/ML-based future CCO state inference in the gNB.

As a general principle we think that the standardization process of a feature should not introduce differentiations based on the node’s architecture, therefore we should aim at standardizing a feature by maximizing commonalities in both split and non-split architectures. Therefore, we do believe that also for the non-split case it should be clarified that the standardization of the AI/ML-based future CCO state inference in the gNB is not supported. Based on that we have the following proposal to reword the agreement taken in the last meeting for the non-split architecture which uses the same wording of the split architecture case:

Proposal 1: RAN3 to agree the rewording of the agreement taken in RAN3#124 as follows “For non-split architecture, the standardization of AI/ML based future CCO state inference in the gNB is not supported in R19, how to derive future CCO state is up to implementation in the gNB, e.g., legacy or AI/ML.”.

From the last RAN3 meeting there is the following FFS aspect that should be dealt with, i.e., [4]:

Time information to be further discussed in next meeting.

In the legacy CCO mechanism, after a certain CCO issue has been detected by the gNB(-CU), the gNB(-DU) should react basically immediately by selecting a new CCO state being able to cope with detected CCO issue. For the AI/ML-based CCO use case, instead, a more proactive approach is used to prevent (or limit at an early stage) the occurrence of a CCO issue resulting in a degradation of network (and UE) performance. This implies that, after a CCO issue is predicted by the gNB(-CU), the gNB(-DU) is requested to select a new CCO state to be applied in the future, hence not immediately because the CCO issue did not occur yet.

Observation 2: Differently from the legacy CCO mechanism, in the AI/ML-based CCO use case after a CCO issue has been predicted by the gNB(-CU), the gNB(-DU) will select a new CCO state to be applied in the future and not immediately.

Figure 1 - AI/ML-based CCO use case: timing information transfer over RAN interfaces.

Figure 1 above shows the interaction between two neighbor gNBs, i.e., gNB1 and gNB2 (where gNB1 is a split gNB) for the purpose of AI/ML-based proactive CCO. Here follows a list of steps describing such interaction:

- gNB-CU1 hosts a trained AI/ML model deployed and used for the proactive CCO; the AI/ML model predicts a CCO issue (coverage, cell edge capacity, …) for affected cells/SSBs and the corresponding timing information (labelled as predicted CCO issue time indication in Figure 1) related to the point in time in the future where the predicted CCO issue is expected to occur;

- gNB-CU1 sends the predicted CCO issue for affected cells/SSBs and the corresponding predicted CCO issue time indication to gNB-DU1 in order for the gNB-DU1 to determine a future CCO state being able to cope with the predicted CCO issue type;

- based on the received information from gNB-CU1, the gNB-DU1 determines the future CCO state for the predicted CCO issue type;

- gNB-CU1 notifies neighbor gNB2 that a CCO issue is predicted to occur at a point in time in the future (corresponding to the predicted CCO issue time indication in Figure 1) for which a future CCO state will be applied;

- gNB2 determines a future CCO state for its own cells/SSBs that best matches the future CCO state of the gNB-CU1’s cells/SSBs resulting from the predicted CCO issue that will occur at a point in time in the future corresponding to the received predicted CCO issue time indication.

Based on the above description of the AI/ML-based CCO mechanism we have the following proposal:

Proposal 2: RAN3 to agree to introduce the predicted CCO issue time indication as an output information of the AI/ML model used in the CCO use case, representing the point in time in the future where the predicted CCO issue is expected to occur.

Proposal 3: RAN3 to agree that the predicted CCO issue time indication is exchanged over F1 and Xn interfaces.

## 2.2 The solution for AI-based CCO

### 2.2.1 Input data of the AI model

To analyse the needed AI/ML model input data in the local node which hosts the AI/ML model, we can consider the data from both UE side and network side. For UE side, the AI/ML model could collect UE measurements results (e.g., RSRP, RSRQ, SINR of serving and neighbouring cells associated with UE location information), UE Mobility History Information and SON reports (e.g., RLF report, HOF report, CEF report) to perform an analysis of the current coverage and capacity situation as well as previous CCO issues. In addition, the UE measurements information at cell-edge could be helpful for the AI/ML model to predict cell-edge interference issues and determine the corresponding CCO strategy. For network side, instead, the current CCO state (from both local and neighbour nodes), the future CCO state (from the neighbour node) as well as coverage and capacity performance metrics (from both local and neighbour nodes) could be helpful for the AI/ML model to analyse the root cause of CCO issues for subsequent adjustment. Moreover, the current and predicted radio resource status information may be beneficial for the AI/ML model to infer the most appropriate CCO strategy when considering other use cases or purposes (e.g., load balancing, mobility enhancement).

Observation 3: UE measurements results, UE Mobility History Information and SON reports from UE side can be used as the input data of the AI/ML model.

Observation 4: Measured/predicted radio resource status (from both local and neighbour nodes), current CCO state (from both local and neighbour nodes), future CCO state (from the neighbour nodes), coverage and capacity performance metrics (from both local and neighbour nodes) can be used as the input data of the AI/ML model.

Based on the above AI/ML input data from the network, referring to TS 38.423, the CCO state from the local node as well as the current and predicted radio resource status information are already supported. Then, coverage and capacity performance metrics (e.g., average access success rate, average access delay, and call drop rate) could be considered to be introduced and transferred over the Xn interface, to support the AI/ML model to determine the global coverage and capacity performance information across NG-RAN nodes.

Proposal 4: RAN3 to agree to consider measured/predicted radio resource status (from both local and neighbour nodes), current CCO state (from both local and neighbour nodes), future CCO state (from both local and neighbour nodes), coverage and capacity performance metrics (from both local and neighbour nodes) to be used as the input data of the AI/ML model.

Proposal 5: RAN3 to agree to introduce the coverage and capacity performance metrics (e.g., average access success rate, average access delay, and call drop rate) over the Xn interface for AI/ML usage, to analyse the current global coverage and capacity situation across RAN nodes.

Proposal 6: UE measurements results, UE Mobility History Information and SON reports from UE side can be used as the input data of the AI/ML model.

### 2.2.2 Output data of the AI model

After the local node collected AI/ML input data from UE side and network side to perform AI/ML inference, the inferred recommended CCO strategies can be transferred to the neighbour node to resolve potential CCO issues in advance. In our view, the inferred recommended CCO strategies can include the predicted CCO issue (e.g. coverage, cell-edge interference) and corresponding predicted CCO issue time indication, future CCO state for cells and SSBs and predicted replacement information for cells (e.g. cell shaping, cell splitting, cell merging) at least. Transferring the predicted CCO issue time indication is already covered by Proposal 3, therefore we have the following proposal:

Proposal 7: RAN3 to agree to exchange the predicted CCO issue and corresponding future CCO state, as well as future replacement information for cells over Xn to improve the legacy CCO mechanism.

In addition, the local node can also notify to the neighbour node the predicted UE offload plan (e.g., the number of UEs to be offloaded from the NG-RAN node to the neighbour NG-RAN node), so that the local node could consider the accepted UE offload plan from neighbour node before performing CCO adjustments. Thus, the local node can adjust its coverage by choosing the most appropriate CCO strategy based on both the AI/ML inference output data and response information from the neighbour node.

Proposal 8: RAN3 to agree to exchange the predicted UE offload plan over Xn from local node to the neighbor node.

### 2.2.3 Feedback data of the AI model

To evaluate the general performance of predicted recommended CCO strategies and evaluate the impact on UE performance, the related UE performance feedback can be considered to be collected from NG-RAN nodes. In Rel-18, RAN3 has agreed UE-level UE performance collection and reporting over Xn (i.e., Average UE Throughput DL, Average UE Throughput UL, Average Packet Delay, Average Packet Loss DL) for AI/ML-based use cases. We think the NG-RAN node could reuse the above UE-level UE performance metrics to optimize the performance of the AI/ML-based CCO model.

In the AI/ML-based CCO use case, the inferred recommended CCO strategies include a UE-level action (e.g., handover UE to neighbour node) and a node-/cell-/SSB-level action (e.g. cell/SSB CCO state modification). Further, we think that the AI/ML model should obtain not only UE-level UE performance metrics as feedback information, but the local node should also receive node-/cell-/SSB-level UE performance metrics from the neighbour node.

Observation 5: UE performance is useful to be provided to the NG-RAN node for feedback purposes.

Proposal 9: RAN3 to agree to introduce node-/cell-/SSB-level UE performance over the Xn interface for the AI/ML-based CCO use case.

Moreover, we think it would be beneficial for the AI/ML model that the local node could receive from the neighbour gNB information related to its current CCO state after the CCO issue previously predicted occurs (and the corresponding future CCO state is actually applied by the local node to mitigate such CCO issue). Therefore, we propose the following:

Proposal 10: RAN3 to agree to enable transferring the current CCO state from the neighbour gNB to the local node hosting the AI/ML model over Xn as feedback information.

A TP for the TR 38.743 reflecting the above proposals is provided in the Annex of this paper.

# 3. Conclusion

Based on the above discussion, we have the following observations and proposals:

Observation 1: With the current wording of the agreement for the non-split architecture concerning the derivation of the future CCO state (as a consequence of a predicted CCO issue) there could be the risk of interpreting the agreement in such a way that there is a need for standardization support to enable the AI/ML-based future CCO state inference in the gNB.

Proposal 1: RAN3 to agree the rewording of the agreement taken in RAN3#124 as follows “For non-split architecture, the standardization of AI/ML based future CCO state inference in the gNB is not supported in R19, how to derive future CCO state is up to implementation in the gNB, e.g., legacy or AI/ML.”.

Observation 2: Differently from the legacy CCO mechanism, in the AI/ML-based CCO use case after a CCO issue has been predicted by the gNB(-CU), the gNB(-DU) will select a new CCO state to be applied in the future and not immediately.

Proposal 2: RAN3 to agree to introduce the predicted CCO issue time indication as an output information of the AI/ML model used in the CCO use case, representing the point in time in the future where the predicted CCO issue is expected to occur.

Proposal 3: RAN3 to agree that the predicted CCO issue time indication is exchanged over F1 and Xn interfaces.

AI/ML model’s input data

Observation 3: UE measurements results, UE Mobility History Information and SON reports from UE side can be used as the input data of the AI/ML model.

Observation 4: Measured/predicted radio resource status (from both local and neighbour nodes), current CCO state (from both local and neighbour nodes), future CCO state (from the neighbour nodes), coverage and capacity performance metrics (from both local and neighbour nodes) can be used as the input data of the AI/ML model.

Proposal 4: RAN3 to agree to consider measured/predicted radio resource status (from both local and neighbour nodes), current CCO state (from both local and neighbour nodes), future CCO state (from both local and neighbour nodes), coverage and capacity performance metrics (from both local and neighbour nodes) to be used as the input data of the AI/ML model.

Proposal 5: RAN3 to agree to introduce the coverage and capacity performance metrics (e.g., average access success rate, average access delay, and call drop rate) over the Xn interface for AI/ML usage, to analyse the current global coverage and capacity situation across RAN nodes.

Proposal 6: UE measurements results, UE Mobility History Information and SON reports from UE side can be used as the input data of the AI/ML model.

AI/ML model’s output data

Proposal 7: RAN3 to agree to exchange the predicted CCO issue and corresponding future CCO state, as well as future replacement information for cells over Xn to improve the legacy CCO mechanism.

Proposal 8: RAN3 to agree to exchange the predicted UE offload plan over Xn from local node to the neighbor node.

AI/ML model’s feedback data

Observation 5: UE performance is useful to be provided to the NG-RAN node for feedback purposes.

Proposal 9: RAN3 to agree to introduce node-/cell-/SSB-level UE performance feedback over the Xn interface for the AI/ML-based CCO use case.

Proposal 10: RAN3 to agree to enable transferring the current CCO state from the neighbour gNB to the local node hosting the AI/ML model over Xn as feedback information.

A TP for the TR 38.743 reflecting the above proposals is provided in the Annex of this paper.

# 4. Reference

- RP-234054, “New SID: Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#102

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- 3GPP RAN3#123b Chairman’s notes (RAN3_123bis_agenda_20240419_EOM.doc)

- 3GPP RAN3#124 Chairman’s notes (RAN3_124_agenda_20240524_EOM1.doc)

# 5. Annex – TP to TR 38.743 (based on R3-243971)

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 4.2	AI/ML based Coverage and Capacity Optimization

### 4.2.1	Use case description

Editor Note: Capture the description of use case

The objective of NR Coverage and Capacity Optimization (CCO) function is to detect and resolve or mitigate CCO issues. An NG-RAN node may autonomously adjust within and switch among coverage configurations. When a change is executed, a NG-RAN node may notify its neighbour NG-RAN nodes with the list of cells and SSBs with modified coverage included.

In the legacy CCO solution, a reactive approach is used: when the gNB (gNB-CU in case of CU-DU split architecture) detects a CCO issue which negatively impacts network and UE performance after it has already occurred, the gNB (gNB-DU in case of CU-DU split architecture) attempts to resolve or mitigate it.

With an AI/ML based CCO, a more proactive approach is used to prevent (or limit at an early stage) the occurrence of a CCO issue resulting in a degradation of network (and UE) performance.

### 4.2.2	Solutions and standard impacts

Editor Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

#### 4.2.2.1 Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions can be considered for supporting AI/ML-based CCO:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

- AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU(-CP)-DU split architecture, the following solutions are possible:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU(-CP).

- AI/ML Model Training and Model Inference are both located in the gNB-CU(-CP).

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 4.2.2.2	Input data of AI/ML based Coverage and Capacity Optimization:

To predict a CCO issue in advance, a gNB may need the following information as input data for AI/ML-based CCO:

From local node:

-	Measured/Predicted radio resource status

-	Current CCO state for cells and SSBs

-	Future CCO state for cells and SSBs

-	Measured coverage and capacity performance metrics, such as average access success rate, average access delay, and call drop rate.

From neighbouring gNBs:

-	Measured/Predicted radio resource status

-	Current CCO state for cells and SSBs

-	Future CCO state for cells and SSBs

-	Measured coverage and capacity performance metrics, such as average access success rate, average access delay, and call drop rate.

From the UE:

-	UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc.), including cell level and beam level UE measurements

-	UE Mobility History Information

-	SON reports (e.g., RLF report, HOF report, CEF report)

#### 4.2.2.3	Output data of AI/ML based Coverage and Capacity Optimization:

AI/ML-based CCO model in a gNB can generate following information as output:

- predicted CCO issue (e.g. coverage, cell edge capacity)

- predicted CCO issue time indication

- future CCO state for cells and SSBs

- future replacement information for cells (e.g. cell shaping, cell splitting, cell merging)

- predicted UE offload plan

#### 4.2.2.4	Feedback of AI/ML based Coverage and Capacity Optimization:

To optimize the performance of the AI/ML-based CCO model, the following feedback can be considered to be collected from gNBs:

- UE-level UE performance

- Node-/cell-/SSB-level UE performance

- Current CCO state for cells and SSBs of the neighbor NG-RAN node

#### 4.2.2.5	Potential standard impacts:

Following standard impacts is listed for subsequent Rel-19 normative work compared with what was specified during Rel-18.

Xn interface:

- Enhance existing procedure to collect information between gNBs:

  - predicted CCO issue

  - predicted CCO issue time indication

  - current/future CCO state for cells and SSBs

  - future replacement information for cells (e.g. cell shaping, cell splitting, cell merging)

  - predicted UE offload plan

  - measured coverage and capacity performance metrics (e.g., average access success rate, average access delay, and call drop rate)

  - node-/cell-/SSB-level UE performance feedback

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>