3GPP TSG-RAN WG3 Meeting #125	R3-244296

Maastricht, Netherlands, 19th – 23rd August, 2024

Title: 	(TP to TR 38.743) AI/ML-based Mobility Enhancements in NR-DC

Source: 	Huawei

Agenda item:	11.4

Document Type:	Discussion and Decision

# 1. Introduction

So far in RAN3 meetings the following agreements have been achieved for the AI/ML-based mobility enhancements discussed in Rel-19 [1][2]:

RAN3 #123bis meeting: Agree to take as baseline for the Mobility Optimization for NR-DC that the use case is studied assuming inference at the MN. The main use case is limited to dual connectivity only (e.g. no conditional procedures are in scope). It is agreed to evaluate solutions on multi hop trajectory and check their impact on specifications with the aim of minimizing that. If consensus on a solution approach is not reached at May meeting, the topic is down prioritized. RAN3 #124 meeting: Multiple-hop UE trajectory is not supported in R19. Enhance the Dual Connectivity procedure (e.g., MN-initiated SN addition, MN-initiated SN change) to trigger the collection of measured UE performance.

In this contribution we further discuss the solution details for AI/ML-based mobility enhancements in NR-DC scenario.

# 2. Discussion

Rel-18 mainly considered the scenario where a UE is connected to a single NG-RAN node, and NR-DC scenario is not considered. The enablers for the AI/ML-based Mobility Optimization use case in Rel-18 over Xn are as follows:

- UE Trajectory Collection Configuration and UE Performance Collection Configuration IEs in the DATA COLLECTION REQUEST message;

- Cell Based UE Trajectory Prediction and Data Collection ID IEs in the HANDOVER REQUEST message;

- UE Performance and Measured UE Trajectory IEs in the DATA COLLECTION UPDATE message.

In the following sub-clauses we provide our considerations for which DC procedures should include AI/ML prediction results and how to define them.

## 2.1 UE trajectory prediction in NR-DC

In the NR-DC scenario, the gNB (i.e., a “future” MN) decides whether to introduce an S-NG-RAN node to provide additional radio resources from SN to the UE. Based on the analysis of the UE reported measurement results and neighbour nodes’ reported resource status, the gNB can evaluate and choose a target SN for a specific UE to establish a NR-DC connection.

As already specified in Rel-18, the node serving a UE (i.e., a “future” MN) can predict the UE trajectory and acquire the predicted resource status information from a neighbour node (i.e., a candidate “future” SN), which could be helpful to evaluate whether a specific neighbour node is the optimal choice for establishing a future NR-DC for the UE. However, once the NR-DC has been established and from the perspective of the SN, currently the MN only provides the UE latest reported measurement results, which only reveal the past UE trajectory and is not helpful for the SN to arrange the optimal PSCells’ resources in future time.

Observation 1: Currently, the SN can only receive historical UE measurement results from MN, which is helpless for the SN to arrange the optimal PSCells’ resources in future time.

With the AI/ML method introduced, in standalone architecture, a gNB can predict the future UE trajectory. Then, as specified in TS 38.423 [3], during the Xn interface setup procedures, the same gNB (the “future” MN) is aware of the list of served NR cells in the neighbour node (the “future” SN). Therefore, in the view of establishing a NR-DC connection for the UE, the gNB (the “future” MN) could be able to predict the UE’s trajectory among PSCell(s) in the target “future” SN. In our understanding, since RAN3 has agreed it is the MN to execute the inference, after the MN predicts the UE trajectory, it can send the predicted results to target SN, e.g. in the S-NODE ADDITION REQUEST and S-NODE MODIFICATION REQUEST signalling. Thus, the AI/ML enhancements can be applied in SN Addition, MN-initiated SN Modification and MN-initiated SN Change cases.

Proposal 1: The MN can send predicted UE trajectory among PSCells to SN, which can be provided during SN Addition, MN-initiated SN Modification and MN-initiated SN Change procedures.

As specified in clause 9.2.3.181 in TS 38.423, in the standalone architecture, the current predicted UE trajectory includes the following information, which can be regarded as a baseline:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Predicted Trajectory Cell Information | M |  |  |  |
| >NG-RAN Cell |  |  |  |  |
| >>Global NG-RAN Cell Identity | M |  | 9.2.2.27 | Indicates an NR Cell Identity. |
| >>Predicted Time UE Stays in Cell | O |  | INTEGER (0..4095) | The duration of time the UE is expected to stay in the cell, in seconds. If the duration is more than 4095s, this IE is set to 4095. |

In the NR-DC scenario, the MN can provide the SN with the list of PSCells the UE is expected to visit and (optionally) for how long the UE is expected to stay in each PSCell. Similar assumption can be done for the measured UE trajectory in the SN, where the information in clause 9.2.3.183 of TS 38.423 (Measured Trajectory Cell Information) can be assumed as baseline structure.

Proposal 2: For NR-DC, the specified predicted UE trajectory information and measured UE trajectory information can be regarded as baseline structures for the UE trajectory prediction in the SN and measured UE trajectory feedback, respectively.

## 2.2 Solution for feedback

After the NR-DC connection is established, the MN should be informed of the UE performance and measured UE trajectory in the SN, for AI/ML optimization. When the MN provides prediction results to the SN, the MN should also initiate the related Data Collection Reporting Initiation procedure, where the MN configures the SN for collection and reporting of UE performance in the SN as well as measured UE trajectory. We think the existing Data Collection Reporting (Initiation) procedures can be regarded as baseline. Then, similar to HANDOVER REQUEST, the MN can include the Data Collection ID IE in the S-NODE ADDITION REQUEST and S-NODE MODIFICATION REQUEST messages. Then SN can feedback the UE performance and measured UE trajectory among visited PSCells, based on the corresponding data collection reporting configuration.

Proposal 3: For UE performance and measured UE trajectory feedback from SN to MN, the current Data Collection Reporting (Initiation) procedures can be the baseline and the MN should include the Data Collection ID IE in the S-NODE ADDITION REQUEST and S-NODE MODIFICATION REQUEST messages.

A TP for the TR 38.743 reflecting the above proposals is provided in the Annex of this paper.

# 3. Conclusion

Based on the above discussion, we have the following observations and proposals:

Observation 1: Currently, the SN can only receive historical UE measurement results from MN, which is helpless for the SN to arrange the optimal PSCells’ resources in future time.

Proposal 1: The MN can send predicted UE trajectory among PSCells to SN, which can be provided during SN Addition, MN-initiated SN Modification and MN-initiated SN Change procedures.

Proposal 2: For NR-DC, the specified predicted UE trajectory information and measured UE trajectory information can be regarded as baseline structures for the UE trajectory prediction in the SN and measured UE trajectory feedback, respectively.

Proposal 3: For UE performance and measured UE trajectory feedback from SN to MN, the current Data Collection Reporting (Initiation) procedures can be the baseline and the MN should include the Data Collection ID IE in the S-NODE ADDITION REQUEST and S-NODE MODIFICATION REQUEST messages.

# 4. Reference

- 3GPP RAN3#123b Chairman’s notes (RAN3_123bis_agenda_20240419_EOM.doc).

- 3GPP RAN3#124 Chairman’s notes (RAN3_124_agenda_20240524_eom.doc).

- 3GPP TS 38.423 V18.2.0 (2024-06).

# 5. Annex - TP to 38.743 (based on R3-243971)

-------------------------------------------------------------Start of Change-----------------------------------------------------------------

## 5.1	Mobility optimization for NR-DC

Editor Note: Capture the description and its potential standard impacts.

### 5.1.1 Use case description

Mobility Optimization for NR-DC is studied by assuming inference at the MN only. The main use case is limited to Dual Connectivity only (e.g., no conditional Dual Connectivity procedures are in scope), aiming at using AI/ML for optimizing SN Addition, MN-initiated SN Modification as well as MN-initiated SN Change procedures.

The MN can provide the SN with the predicted UE trajectory as a list of PSCells which are expected to be visited by the UE, and the MN can request the SN to feedback the measured UE trajectory across PSCells as well as the UE performance in the SN for further optimization.

### 5.1.2 Solutions and standard impacts

Based on Rel-18 AI mobility enhancements, a gNB with AI/ML functionality may predict the future cell-level UE trajectory. Based on the UE trajectory prediction and radio resource status information from neighbour gNBs, the gNB is able to decide whether to configure the UE with NR-DC by choosing the optimal candidate SN. For better PSCells’ resources arrangement, the SN should be made aware by the MN of the cell-level UE trajectory prediction among the PSCells. The Rel-18 Cell Based UE Trajectory Prediction IE can be baseline to describe the predicted UE trajectory within the SN.

Once the MN has provided the SN with the predicted UE trajectory, the SN should feedback the measured UE trajectory among visited PSCells as well as the UE performance in the SN. In this case, the Rel-18 Data Collection Reporting Initiation procedure can be the baseline for configuring the collection and reporting of feedback information, while the actual reporting can be done via the Data Collection Reporting procedure.

-------------------------------------------------------------Start of Change-----------------------------------------------------------------