3GPP TSG-RAN WG3 Meeting #125	R3-244298

Maastricht, Netherlands, 19th – 23rd August, 2024

Title: 	Rel-18 AI/ML for NG-RAN leftovers: Energy Saving Enhancements

Source: 	Huawei

Agenda item:	11.4

Document Type:	Discussion and decision

# 1. Introduction

In RAN#102 a new Rel-19 SI “New SID: Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (RAN3-led) was approved in [1], then revised in RAN#103 and eventually approved in [2], whose objective are listed as follows:

The aim of this study item is to further investigate new AI/ML based use cases and identify enhancements to support AI/ML functionality, and further discussions on the Rel-18 leftovers.

The detailed objectives of the SI are listed as follows:

- Study two new AI/ML based use cases, i.e., Network Slicing and CCO, with existing NG-RAN interfaces and architecture (including non-split architecture and split architecture).

- Rel-18 leftovers as candidates for normative work, based on the Rel-18 principles, as follows:

-   Mobility optimization for NR-DC

-   Split architecture support for Rel-18 use cases based on the conclusions from Rel-18 WI

-   Energy Saving enhancements, e.g., Energy Cost Prediction

-   Continuous MDT collection targeting the same UE across RRC states

-   Multi-hop UE trajectory across gNBs

Note: RAN3 should take the Rel-18 discussions into account.

In the last RAN3 meeting, as part of the leftovers from the Rel-18 AI/ML for NG-RAN WI, the following results from the discussion on the Energy Saving enhancements, as excerpted from [3]:

Measured Energy Cost can be transferred from DU to CU.

AI/ML based network energy saving is an important use case for operators. The solutions based on the predicted energy cost can not reach consensus during Rel-19 phase. It can be revisited in the future release.

Only discuss on finer granularity of energy cost in Aug meeting and make the conclusion.

This contribution addresses the FFS highlighted in yellow above as per the last RAN3 meeting.

# 2. Discussion

In the last RAN3 meeting new solutions [4][5] (i.e., not based on the Additional Load concept) to enable an AI/ML model to predict the Energy Cost (EC) were discussed but no consensus was reached in RAN3. Therefore, it was decided that this topic could be part of a future release.

The only aspect that RAN3 should consider for enhancing the Rel-18 AI/ML-based ES is whether and how to introduce a finer granularity of the EC metric with respect to the node-level granularity introduced in Rel-18.

RAN3 introduced the metric of Energy Cost (EC) in RAN3#119 meeting [6]:

Introduce the metric of Energy Cost (EC) as the AI/ML metric to be shared over the Xn interface among gNBs.

while in RAN3#119bis-e meeting [7] the granularity of the EC in Rel-18 was agreed as follows:

It is agreed that the Energy Cost is a node level parameter. Further EC granularities are out of scope of Rel18.

The meaning, configuration and usage of the EC within the NG-RAN was eventually agreed in RAN3#120 meeting [8]:

EC is represented as an index, which should be normalized and defined by OAM. The index value could be encoded as an integer from 0 to a maximum. The maximum value should guarantee enough accuracy.

Finally, in RAN3#121 meeting [9], the encoding of the EC in the RAN3 specifications (TS 38.423) was agreed:

Define the Energy Cost IE as an INTEGER (0..10000,…), it can be revisited based on reply from SA5.

The scenario considered by RAN3 for the AI/ML-based ES use case in Rel-18 is to derive AI/ML-based ES actions aiming at saving energy consumption in a certain area of the Operator’s network, e.g. UE/traffic offloading from capacity cells to larger neighbour coverage cells and subsequent switch-off of the offloaded capacity cells. This is enabled when at least one gNB in the area (i.e., the gNB serving capacity cells) requests neighbour gNBs in the same area (i.e., gNBs providing coverage cells) to report their own measured energy consumptions (mapped into EC values) so that the gNB can assess the overall energy consumption of the area and then derive via AI/ML which is the most appropriate ES action (i.e., traffic/UE offloading) to perform in that area ensuring a reduction in the overall energy consumption. Reporting of EC values from gNBs is also possible after the ES action has been performed to provide e.g. feedback information to the AI/ML model which derived the ES action.

Observation 1: In Rel-18 the EC is a metric corresponding to a node-level measured energy consumption, and its exchange over Xn among multiple gNBs in a certain area of the Operator’s network is the basis of the AI/ML-based ES use case in Rel-18.

When discussing the EC metric granularity in Rel-18 it was argued that it could be difficult to determine the impact of a single ES action (e.g., switch-off of a cell) with respect to the total gNB’s energy consumption if the corresponding EC value is still derived from a node-level energy consumption. This is more evident in case the gNB subject to cell switch-off handles multiple cells, for which it is expected that the measured gNB’s energy consumption after a single cell is switched-off hardly varies, and hence the corresponding EC value will basically remain the same as prior the cell switch-off.

To address the issue back then it was proposed to enhance the granularity of the EC to be a cell-level metric. However, it should be recalled that the EC metric results from an energy consumption measurement by applying a mapping rule, hence in principle a cell-level EC metric should be derived on the basis of a cell-level energy consumption measurement.

By still referring to the discussions from RAN3#119bis-e [7][10], the majority of companies acknowledged that measuring the energy consumption of a single cell (and hence deriving the corresponding cell-level EC metric) is not feasible, due to the fact that gNB implementations typically consider multiple cells sharing the same physical hardware resources (e.g., antenna, amplifiers, baseband, etc.) and splitting the energy consumption of the shared hardware resources among the cells served by such resources to achieve the energy consumption of a single cell simply does not solve the issue – how the receiving node could understand the reported cell-level EC if it is not made aware of how the corresponding measured energy consumption has been derived within the reporting node?

Observation 2: Measuring the energy consumption of a single cell (and hence deriving the corresponding cell-level EC metric) is not feasible.

In the last meeting it was proposed in [11] to introduce the EC metric at cell group level: a gNB may group together cells that share the same physical hardware resources, measure the associated energy consumption and then provide the corresponding EC metric at cell group level. In this manner it is claimed that the issue of per-cell EC in case of shared hardware resources is overcome due to the fact that multiple groups of cells do not share hardware among them (it is only the cells of a single group that share hardware resources). If a gNB has more than just one group of cells it is feasible to provide a finer granularity (cell group level) than the Rel-18 node-level EC, and our understanding is that, for the (degenerate) case of groups with just one cell associated to them, the cell group level EC actually results in cell-level EC. It is also stated in [11] that it is up to the gNB implementation how cells are grouped, since the mapping between cells and groups in a gNB would depend on the specific hardware that is used.

The main concern we have with the EC at cell group level is related to how the cell groups should be determined: in a typical RAN equipment there are different layers (RF, baseband, etc.) where it is possible to enable the hardware resource sharing, and the number of cells that could be part of a group may differ depending on the layer at which hardware resource sharing is implemented. For instance, Cell 1, Cell 2 and Cell 3 can be in Group A while Cell 4, Cell 5 and Cell 6 can be in Group B if hardware resource sharing is at RF level. But if the sharing is done at baseband level, then all cells (Cell 1 to Cell 6) can be within a single Group C.

As a consequence, we do think that by reporting the EC metric at cell group level may create interpretability issues at the receiving node side if the receiving node is unaware of how groups are defined in the reporting node. In other words, there could be the case where two different gNBs (say gNB1 and gNB2) implement cell groups at different layers (e.g., gNB1 at RF level, while gNB2 at baseband level), or at the same layer (e.g., RF for both gNBs) but with a different number of cells in each group (e.g., 5 cells for gNB1 and 10 cells for gNB2). It could happen that both gNB1 and gNB2 report the same measured cell group level EC value (e.g., 5000), because gNB2 is less loaded than gNB1: in this case, the receiving node would wrongly evaluate that gNB1 and gNB2 consume the same amount of energy, hence making the determination of proper AI/ML-based ES actions at the receiving node side inaccurate even though the reported EC has a finer granularity than node-level. And this makes the introduction of a finer granularity EC reporting questionable in terms of benefits.

It could be argued that the cell group level EC could be reported over Xn along with additional information concerning the cells’ group the reported EC is associated to, e.g., number of cells within the group and the layer at which the hardware resource sharing is implemented. However, this will expose sensitive information over an open interface as the Xn related to implementation details of the gNB, which should in principle be avoided.

Observation 3: Reporting of the measured EC metric at cell group level may create interpretability issues at the receiving node side if the receiving node is unaware of how the groups are defined by the reporting node, hence making the determination of proper AI/ML-based ES actions at the receiving node side inaccurate even though the reported EC has a finer granularity than node-level.

Observation 4: Reporting of the measured EC metric at cell group level along with additional information on how the group the reported EC is associated to is implemented (e.g., number of cells within the group as well as at which layer the hardware resource sharing is implemented) should be avoided as this exposes sensitive information related to implementation details of the gNB over open interfaces.

Still in the last RAN3 meeting, while discussing the solution for predicting the EC in [4], the possibility to measure the consumed energy corresponding to a handover event emerged. Such approach is based on investigating the interaction between the Handover procedure and the new Data Collection Reporting (Initiation) procedures in regards of the measured EC after a UE has been handed over from source to target gNB, in a similar way as done in Rel-18 for the purpose of reporting UE performance and measured UE trajectory for a UE after the handover. More in detail, after the UE handover the target gNB can measure the amount of consumed energy associated to the handover event. It is claimed in [4] that the information within the HANDOVER REQUEST message about the services set-up at the source gNB for the UE subject to handover could be used by the target gNB to determine the energy consumption of the handover event (and hence the corresponding EC value) out of the total node-level target gNB’s energy consumption. The proponents of [4] also stated that the approach can be extended to consider the reporting of the measured EC corresponding to a number of handover events.

Our understanding of the per handover event measured EC is that it turns into a per UE measured EC metric, since the handover procedure is a UE-specific procedure, for which we see some similarities with the discussion at RAN Plenary level concerning the standard support for a solution allowing for the determination of the per-UE RAN energy consumption triggered by the SA2 LS in [12]. In the Reply LS to SA2 in [13] RAN Plenary basically put on hold such discussion as there was no consensus (i.e. no agreement) on how to proceed as the use cases, and feasibility of obtaining per UE network energy consumption information require further clarifications.

Based on this, it seems that the determination of the per handover event (i.e., per UE) measured EC can be done by gNB implementation only, with the potential risk of misinterpreting the reported measured EC at the receiving node side. We also think that the information in the HANDOVER REQUEST message could not be sufficient to determine the per handover event EC at the target node because the reported EC could also be influenced by the load situation and power allocation in the target gNB itself, as well as by the UE location in the target cell.

Observation 5: Measuring the EC at per handover event (i.e., per UE) can be done by gNB implementation only and may result in misinterpretation of the reported measured EC at the receiving node side.

On top of the actual feasibility to determine the per handover event measured EC via standardized means, it is also not clear to us how this per handover event measured EC reporting could fit into the Rel-18 AI/ML-based ES use case mechanism. It was previously stated that the Rel-18 AI/ML-based ES use case is based on the reporting of node-level EC values from neighbour gNBs in a certain area so that the receiving gNB can assess the overall energy consumption of the area and then derive via AI/ML which is the most appropriate ES action (i.e., traffic/UE offloading) to perform in that area in such a way that the overall energy consumption will be reduced. Therefore, and considering what has been stated in Observation 5, we do not think that the reported per handover event measured EC could help the AI/ML model to improve its inference capabilities for determining the most appropriate ES action to perform, as it is expected that the receiving node (hosting the AI/ML model) needs to configure the reporting of the measured EC not only on a per-node granularity but also for each single handover event that is predicted to occur as part of the offloading action (or for a certain number of handover events towards a certain reporting node), hence over-complexifying the Rel-18 solution in terms of signalling overhead with no clear benefits.

Observation 6: The per (group of) handover event(s) measured EC over-complexifies the Rel-18 AI/ML-based ES use case mechanism with no clear benefits, and at same time it introduces significant signalling overhead for its configuration and reporting.

To conclude, we think that finer granularities of the measured EC metric other than the Rel-18 node-level granularity could have significant drawbacks compared to the minimal/negligible benefits that they could bring in enhancing the Rel-18 AI/ML ES use case mechanism.

Observation 7: Finer granularities of the measured EC metric have significant drawbacks compared to the minimal/negligible benefits that they could bring.

Based on the above Observations, and considering that this is the last meeting to complete the Rel-19 SI, we have the following proposal:

Proposal: RAN3 to agree that finer granularities of the measured EC are not pursued in Rel-19.

# 3. Conclusion

Based on the discussion in this paper, we make the following observations and proposal:

Observation 1: In Rel-18 the EC is a metric corresponding to a node-level measured energy consumption, and its exchange over Xn among multiple gNBs in a certain area of the Operator’s network is the basis of the AI/ML-based ES use case in Rel-18.

Observation 2: Measuring the energy consumption of a single cell (and hence deriving the corresponding cell-level EC metric) is not feasible.

Observation 3: Reporting of the measured EC metric at cell group level may create interpretability issues at the receiving node side if the receiving node is unaware of how the groups are defined by the reporting node, hence making the determination of proper AI/ML-based ES actions at the receiving node side inaccurate even though the reported EC has a finer granularity than node-level.

Observation 4: Reporting of the measured EC metric at cell group level along with additional information on how the group the reported EC is associated to is implemented (e.g., number of cells within the group as well as at which layer the hardware resource sharing is implemented) should be avoided as this exposes sensitive information related to implementation details of the gNB over open interfaces.

Observation 5: Measuring the EC at per handover event (i.e., per UE) can be done by gNB implementation only and may result in misinterpretation of the reported measured EC at the receiving node side.

Observation 6: The per (group of) handover event(s) measured EC over-complexifies the Rel-18 AI/ML-based ES use case mechanism with no clear benefits, and at same time it introduces significant signalling overhead for its configuration and reporting.

Observation 7: Finer granularities of the measured EC metric have significant drawbacks compared to the minimal/negligible benefits that they could bring.

Proposal: RAN3 to agree that finer granularities of the measured EC are not pursued in Rel-19.

# 4. Reference

- RP-234054, “New SID: Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#102

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- 3GPP RAN3#124 Chairman’s notes (RAN3_124_agenda_20240524_EOM1.doc)

- R3-243493, AI/ML Energy Saving Enhancements, Nokia, Deutsche Telekom, T-Mobile USA, Vodafone, Verizon, Telecom Italia, BT

- R3-243449, AI/ML Network Energy Saving, Ericsson, AT&T, T-Mobile US, NTT DoCoMo, Deutsche Telekom, BT, Vodafone, Telecom Italia, Verizon

- 3GPP RAN3#119 Chairman’s notes (RAN3_119_agenda_20230303_EOM2.doc)

- 3GPP RAN3#119bis-e Chairman’s notes (RAN3_119bis-e_agenda_202304026_EOM1.doc)

- 3GPP RAN3#120 Chairman’s notes (RAN3_120_agenda_20230530_EOM1.doc)

- 3GPP RAN3#121 Chairman’s notes (RAN3_121_agenda_20230825_EOM1.doc)

- R3-232028, SoD on CB: # AIRAN4_ES, Ericsson

- R3-243460, Improved granularity for Energy Cost, Ericsson

- R3-241522 (S2-2403733 / RP-240029), LS on per UE energy consumption in RAN, SA2 (Vodafone)

- R3-241526 (RP-240825), Reply LS RP-240029 on per UE energy consumption in RAN, RAN (InterDigital)