3GPP TSG-RAN WG3 Meeting #125	R3-244299

Maastricht, Netherlands, 19th – 23rd August, 2024

Title: 	On Continuous MDT for AI/ML

Source: 	Huawei

Agenda item:	11.4

Document Type:	Discussion and decision

# 1. Introduction

In last RAN3 the discussion on the Rel-18 leftovers for AI/ML for NG-RAN progressed [1], as per the objectives in the new Rel-19 SI “New SID: Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (RAN3-led) which was approved in RAN#102 [2], then revised in RAN#103 and eventually approved in [3].

Below the list of agreements taken in last RAN3 meeting concerning the whole Rel-18 leftovers SI objective [1]:

Energy Saving Enhancements:

Measured Energy Cost can be transferred from DU to CU.

Only discuss on finer granularity of energy cost in Aug meeting and make the conclusion.

AI/ML based network energy saving is an important use case for operators. The solutions based on the predicted energy cost can not reach consensus during Rel-19 phase. It can be revisited in the future release.

Multiple-hop UE trajectory:

No consensus on the solution the multiple-hop UE trajectory.

Multiple-hop UE trajectory is not supported in R19.

NR-DC mobility optimization:

Enhance the Dual Connectivity procedure (e.g., MN-initiated SN addition, MN-initiated SN change) to trigger the collection of measured UE performance.

For the Continuous MDT topic, the following agreements from RAN3#123bis meeting as reported below have been considered during the discussion [4]:

The Issues to be solved in R19:

…

Continuous MDT collection targeting the same UE across RRC states

…

- Formulate the use case and solution for split architecture support, NR-DC, and continuous MDT use cases

…

which led RAN3 to agree the TP for TR 38.743 in [5] only containing a use case description with related issues to be solved to actually implement the Continuous MDT feature. Technical solutions coping with such issues and hence enabling the feature have not been discussed yet during the SI. This contribution provides proposals for further discussion, based on the agreement and FFS highlighted in yellow above from RAN3#123bis, including a technical solution for Continuous MDT.

# 2. Background

In the context of the Rel-18 WI “AI/ML for NG-RAN” [6] there were some initial discussions on how to collect measurements from UE(s) to be used as datasets for AI/ML models’ (re-)training. In particular, it was argued that, for the AI/ML model (re-)training to be successful, the MDT data collected from the UE should be consecutive – i.e., data series not affected by time interruptions due to, e.g., stop of UE data collection and successive resuming – and a time series of information with a certain granularity. It was claimed that, when the UE enters to connected state from idle/inactive, consecutive (without time gaps) information would not be collected, therefore negatively impacting the AI/ML model which could not be properly (re-)trained due to the use of invalid training datasets. This motivated the discussions on the specification of the Continuous MDT feature, that is, an enhancement of the MDT mechanism suited for AI/ML models’ (re-)training in OAM that allows continuous collection of MDT data from the same UE across RRC states.

The baseline framework for the data collection from the UE, the definition of Continuous MDT and its main application were already agreed in RAN3#119bis-e meeting [7]:

The existing MDT framework is used as baseline for data collection from the UE.

Continuous collection of MDT traces is beneficial only for AI/ML training in OAM. Continuous MDT collection is to enable the continuous collection of MDT data from the same UE across RRC state changes (RRC_Connected, RRC_Idle, RRC_Inactive).

The solution space for enabling the Continuous MDT feature was investigated during the Rel-18 WI, by considering both management-based and signalling-based MDT, but only shortcomings of current MDT mechanisms were acknowledged by companies, which prevented RAN3 to agree on specific enhancements for Continuous MDT in Rel-18 [8]:

The following understanding is based on R18 use cases:

Signalling-based MDT could provide continuous Data Collection from a certain UE, however, it has scalability issues when selecting large number of UEs and lacks cell-level granularity.

Management-based MDT provides the advantage of being scalable when selecting large number of UEs. However, current management-based MDT mechanism cannot identify and group together MDT reports collected from the same UE across RRC states. UE selection is left to RAN implementation and it is not controllable by OAM.

It is proposed that solutions to address the above shortcomings are not pursued in Rel.18.

During the Rel-19 SI, for Continuous MDT, the use case description and problems to be solved have been acknowledged and reported in TR 38.743 (latest version in [8]). To be specific:

### 5.4.1	Use Case Description

The problem of continuous data collection for management-based MDT can be described as follows: a UE in the NG-RAN can be configured with management-based Logged MDT when in RRC_Idle and RRC_Inactive states and with management-based Immediate MDT when in RRC_Connected state. Differently from signalling-based MDT, in management-based MDT, a UE is not uniquely identified in the MDT activation. Therefore, when a UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive (during which Logged MDT data have been collected) or when a UE is handed over between gNBs, the network does not have standardized means to select again the same UE for continuous MDT for subsequent MDT data collection.

The Data Collection continuity in this scenario can be split into two tasks as below:

- Problem A (measurement continuity): how to ensure that the same UE collecting MDT measurements during the same RRC state and across different RRC states.

- Problem B (trace correlation): how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE.

In the next Section 3 we discuss on how to properly address Problem A and Problem B above so to actually enable the Continuous MDT feature in Rel-19.

# 3. Discussion

## 3.1	Existing MDT framework for Continuous MDT

The MDT framework has been specified to mainly serve the purpose of collecting data from the UE over the Uu interface. One of the open points from RAN3#119bis-e meeting is the following [7]:

FFS on using s-based MDT, m-based MDT or both of them for data collection from the UE.

When MDT was specified in Rel-10, two types of MDT have been introduced: signalling-based MDT and management-based MDT. The former is used to collect measurements from a certain UE which is selected in OAM based on a permanent UE identity, while the final decision to activate the data collection process for that UE is taken by the UDM in CN. In management-based MDT, instead, measurements are collected from randomly chosen UEs or a group of UEs that enter a (certain) geographical area. Management-based MDT is typically activated in RAN nodes of a specific area (identified by a list of NG-RAN cells) and UE selection is performed by RAN nodes based on parameters received by the OAM, UE radio capability and other information, e.g., user consent and roaming status.

Based on the above, it is clear that the main difference between signalling- and management-based MDT is the granularity of the UE selection process. By recalling the agreement from RAN3#119bis-e meeting that [7]:

The existing MDT framework is used as baseline for data collection from the UE.

and that data collection, in turn, serves the purpose of providing sufficient amount of data as input for AI/ML model (re-)training, we need to ensure that data is collected from as many UEs as possible, under different radio environments and RRC states. This is also motivated by the fact that the input data collection for AI/ML model (re-)training is not a time-critical task, it is a long-term and continuous task. In our understanding, fulfilling these requirements for proper data collection from UEs can only be achieved via management-based MDT, therefore we propose:

Proposal 1: RAN3 to discuss and agree that the existing management-based MDT mechanism is used as baseline to ensure proper data collection from as many UEs as possible, under different radio conditions and RRC states for the purpose of AI/ML model (re-)training in OAM.

Proposal 1bis: RAN3 to discuss and agree whether also signalling-based MDT should be considered for the Continuous MDT feature, and identify enhancements needed to enable such feature.

The main issue with management-based MDT to be used as the baseline approach for Continuous MDT, however, is related to the MDT data anonymization requirement: TS 32.422, in fact, states that anonymization of MDT data gathered via management-based MDT is mandatory for both MDT types (the so-called Job Types), i.e., immediate MDT and logged MDT. In order to understand how this requirement could impact the possibility to actually implement the concept of Continuous MDT we can refer to the following Figure 1.

In Figure 1 it is assumed that the OAM has provided the NG-RAN with a management-based MDT configuration consisting of an immediate MDT configuration and a logged MDT configuration; each of these MDT configurations has been assigned with a separate Trace Reference (TR), i.e., TR#1 and TR#2 for immediate MDT and logged MDT, respectively. It is then the NG-RAN (gNB in Figure 1) that activates the MDT feature towards a certain number of UEs (for simplicity, only a single UE is represented in Figure 1). When the UE is in connected mode, it performs MDT measurements according to the immediate MDT configuration it has received from the NG-RAN and reports those measurements to the NG-RAN itself which, in turn, forwards these MDT data to the TCE. Then, when the UE goes to idle/inactive mode, it performs the logged MDT measurements as per the logged MDT measurement configuration provided by the NG-RAN. Once back in connected mode, and assuming that the UE resumes under the same gNB as in Figure 1, the UE will report the MDT measurements performed while it was in idle/inactive mode to the gNB which, again, will forward these measurements to the TCE.

Figure 1 – Management-based MDT data acquisition from the same UE.

In order to actually implement the Continuous MDT concept and, more specifically, to create datasets that are represented as a time series of continuous information for proper (re-)training of AI/ML models – i.e., data series not affected by time interruptions due to e.g. stop of UE data collection and successive resuming – it is needed that TCE identifies that the immediate and logged MDT measurement reports are related to the same UE and that it can correlate these MDT reports, so to create a unique dataset. And since this unique dataset has been built by exploiting the MDT reports provided by the same UE irrespective of its RRC state, such dataset is a consistent representation of the network, not dependent on e.g. UE capabilities, UE implementation, etc.

However, as per the anonymization requirement of management-based MDT data, the MDT reports provided to the TCE are anonymized on purpose, hence it seems that currently there are no means for the TCE to recognize which MDT reports are provided by the same UE.

It is worth to highlight that there is no need for the TCE to identify the UE which provided the immediate and logged MDT reports, but what it is actually needed is that the TCE identifies which MDT reports are provided by the same UE so that the TCE can correlate these reports and hence create a unique dataset which can be used for AI/ML (re-)training purposes.

Observation 1: To enable Continuous MDT via management-based MDT, the TCE needs to be able to identify which management-based MDT reports (logged and immediate) are provided by the same UE, so that the TCE can correlate these reports to then create a unique dataset (in the form of a continuous time series of information) which can be used for AI/ML model’s (re-)training.

Observation 2: There is no requirement for the TCE to identify the UE which provided both management-based immediate and logged MDT measurement reports.

In Rel-18 discussions there was an attempt to check whether existing identifiers, e.g. IMEI-TAC, NR CGI, TR/TRSR could be used for trace correlation at TCE level [9]. In our previous contribution in [10] we provided an in-depth analysis of such identifiers and concluded that they are not suited for being used in the trace correlation in TCE.

Observation 3: Neither IMEI-TAC, NR-CGI nor the use of TR/TRSR as per the current SA5 specifications can be used at TCE level to correlate immediate and logged MDT reports from the same UE.

One possibility to address this issue could be to introduce a new Job Type in SA5 specifications which allows to configure a MDT session encompassing immediate MDT and logged MDT simultaneously, and assigning to this MDT session the same Trace Reference (TR). This should be possible because from SA5 TS 32.422 §5.9a it is already possible to configure a Job Type for two modes – refer to, e.g., Job Type value 3 which is Immediate MDT and Trace. Hence, if a new Job Type such as “Immediate MDT and Logged MDT” is introduced, then the same TR could be assigned to the MDT session corresponding to this new Job Type. TS 32.422 states that the OAM sends a Trace Session activation request to the gNB with configuration parameters including the Job Type and the TR. Then the gNB activates the MDT functionality for the selected UE and assigns a Trace Recording Session Reference (TRSR) to the selected UE.

Observation 4: The current specifications states that the OAM provides the TR while the TRSR is assigned by the gNB on a per-UE basis within the same MDT session identified by the specific TR.

Therefore, in management-based MDT, a single TR could be assigned to an MDT session with a Job Type equal to “Immediate MDT and Logged MDT”, and this configuration is valid for all UEs selected by the NG-RAN. In order to then correlate the immediate and logged MDT reports from the same UE (properly anonymized, e.g., via IMEI-TAC), the TCE could perform such correlation by “collating” those MDT reports having the same couple TR/TRSR: the TR links the MDT reports one to each other (as they both belong to the same MDT session) while the TRSR links the MDT reports to a specific UE, hence solving Problem B (trace correlation) in [8].

Since the above solution has impacts on SA5, it is proposed to liaise with SA5 asking for feedback on its feasibility.

Proposal 2: RAN3 to discuss on the solution based on a new Job Type “Immediate and Logged MDT” for solving Problem B (trace correlation). A LS to SA5 should be sent asking for feedback on the feasibility of such solution in Rel-19.

A draft LS to SA5 can be found in the Annex of this contribution.

Aside Problem B (trace correlation), a proper Continuous MDT solution needs to address also Problem A (measurement continuity) in [8], that is, how to ensure that the same UE keeps on collecting MDT measurements both across different RRC states (i.e., after idle/inactive to connected state transition) as well as during the same RRC state (that is, after handover from source gNB to target gNB). For the first case, the scenario involves a UE which has been configured with immediate and logged MDT and, after having performed logged MDT measurements while in idle/inactive state, it re-establishes/resumes its RRC connection in a new cell and the UE reports the availability of logged MDT measurements to the (potentially new) serving gNB. In order to avoid time interruptions in the measurement process, the UE should be re-configured to perform immediate MDT immediately after it re-establishes/resumes the RRC connection, i.e., just after it moves to connected state.

To this end, and considering that providing a management-based MDT configuration with Job Type “Immediate MDT and Logged MDT” means that the UEs that will use such configuration are the ones configured with the Continuous MDT feature, it could be possible to exploit a solution based on which the serving gNB sends the MDT configuration (the one with the Job Type “Immediate MDT and Logged MDT”) to the 5GC by default immediately after it receives such configuration from the OAM. The MDT configuration can be stored within the UE context in the 5GC by default only for those UEs making use of the management-based MDT configuration with Job Type “Immediate MDT and Logged MDT”. In this manner, once the UE re-establishes/resumes its RRC connection, the (potentially new) serving gNB will retrieve the UE context from 5GC and hence the management-based MDT configuration with Job Type “Immediate MDT and Logged MDT” to be used as an indication that the UE was previously selected for Continuous MDT, so that the (potentially new) serving gNB will re-configure the UE to continue performing the immediate MDT measurements in connected mode. This removes the burden of the (potentially new) serving gNB to recognize which UE(s) have been configured with the Continuous MDT feature, because it will become aware of this by simply retrieving the UE context from the 5GC, which by default contains the management-based MDT configuration with Job Type “Immediate MDT and Logged MDT” only for those UEs subject to the Continuous MDT feature.

Observation 5: Measurement continuity across different RRC states can be achieved by storing the MDT configuration in 5GC (in the UE context) after the gNB has selected the UE and it has configured it with management based MDT only in case the MDT configuration contains the Job Type “Immediate MDT and Logged MDT”. After RRC state transition from idle/inactive to connected, the (potentially new) serving gNB retrieves the management-based MDT configuration with Job Type “Immediate MDT and Logged MDT” within the UE context from 5GC to be used as an indication that the UE was previously selected for Continuous MDT, so that the (potentially new) serving gNB can re-configure the same UE to continue performing the immediate MDT measurements in connected mode.

The above approach is summarized in the following Figure 2.

Figure 2 - 5GC-based solution for management-based Continuous MDT.

- OAM provides the NG-RAN node (gNB1) with a management-based MDT configuration having Job Type “Immediate MDT and Logged MDT” and a specific TR (TR#1);

- gNB1 configures the UE (UE1) to perform MDT measurements based on the management-based MDT configuration previously provided by the OAM and assigns a UE-specific TRSR;

- since the received management-based MDT configuration has the Job Type field set to “Immediate MDT and Logged MDT” the gNB1 stores such management-based MDT configuration in the UE context within the 5GC;

- UE1 is in connected mode, hence it performs immediate MDT measurements according to the management-based MDT configuration provided by the gNB1 and provides the immediate MDT measurement report to the gNB1 which, in turn, is sent by the gNB1 to the TCE (properly anonymized);

- UE1, which previously moved to idle/inactive mode and has performed logged MDT measurements according to the management-based MDT configuration provided by the gNB1, re-establishes/resumes its RRC connection under the coverage of a new serving NG-RAN node (gNB2) and indicates to gNB2 that it has a logged MDT measurement report available;

- gNB2 retrieves the UE context of UE1 from the 5GC including the management-based MDT configuration having Job Type “Immediate MDT and Logged MDT” and a specific TR (TR#1);

- gNB2 retrieves the logged MDT measurement report from UE1 and sends it to the TCE (properly anonymized);

- According to the presence of the management-based MDT configuration in the UE context of UE1 (previously stored by gNB1 in 5GC), gNB2 is able to reconfigure the UE1 to perform immediate and logged MDT measurements.

Based on the above we propose the following:

Proposal 3: RAN3 to discuss and agree on a solution allowing a UE to be reconfigured with immediate MDT immediately after it re-establishes/resumes its RRC connection. Such solution is based on storing and retrieving the MDT configuration having Job Type “Immediate MDT and Logged MDT” in the UE context within the 5GC.

For the handover case, instead, it needs to be further checked whether the UE stops performing immediate MDT measurements (if configured by the source gNB) when it is handed over to the target node, because the target gNB already has the MDT configuration stored if its cells belong to the area scope of the management based MDT configuration set by OAM. Hence, the target gNB could in principle already ensure continuity of the measurement process for the UE during the handover procedure.

Proposal 4: RAN3 to further check whether measurement continuity during the same RRC state (i.e., at handover) is already ensured by current specifications or enhancements are needed.

# 4. Conclusion

Based on the discussion in this paper, we provide the following observations and proposals:

Proposal 1: RAN3 to discuss and agree that the existing management-based MDT mechanism is used as baseline to ensure proper data collection from as many UEs as possible, under different radio conditions and RRC states for the purpose of AI/ML model (re-)training in OAM.

Proposal 1bis: RAN3 to discuss and agree whether also signalling-based MDT should be considered for the Continuous MDT feature, and identify enhancements needed to enable such feature.

Observation 1: To enable Continuous MDT via management-based MDT, the TCE needs to be able to identify which management-based MDT reports (logged and immediate) are provided by the same UE, so that the TCE can correlate these reports to then create a unique dataset (in the form of a continuous time series of information) which can be used for AI/ML model’s (re-)training.

Observation 2: There is no requirement for the TCE to identify the UE which provided both management-based immediate and logged MDT measurement reports.

Observation 3: Neither IMEI-TAC, NR-CGI nor the use of TR/TRSR as per the current SA5 specifications can be used at TCE level to correlate immediate and logged MDT reports from the same UE.

Observation 4: The current specifications states that the OAM provides the TR while the TRSR is assigned by the gNB on a per-UE basis within the same MDT session identified by the specific TR.

Proposal 2: RAN3 to discuss on the solution based on a new Job Type “Immediate and Logged MDT” for solving Problem B (trace correlation). A LS to SA5 should be sent asking for feedback on the feasibility of such solution in Rel-19.

A draft LS to SA5 can be found in the Annex of this contribution.

Observation 5: Measurement continuity across different RRC states can be achieved by storing the MDT configuration in 5GC (in the UE context) after the gNB has selected the UE and it has configured it with management based MDT only in case the MDT configuration contains the Job Type “Immediate MDT and Logged MDT”. After RRC state transition from idle/inactive to connected, the (potentially new) serving gNB retrieves the management-based MDT configuration with Job Type “Immediate MDT and Logged MDT” within the UE context from 5GC to be used as an indication that the UE was previously selected for Continuous MDT, so that the (potentially new) serving gNB can re-configure the same UE to continue performing the immediate MDT measurements in connected mode.

Proposal 3: RAN3 to discuss and agree on a solution allowing a UE to be reconfigured with immediate MDT immediately after it re-establishes/resumes its RRC connection. Such solution is based on storing and retrieving the MDT configuration having Job Type “Immediate MDT and Logged MDT” in the UE context within the 5GC.

Proposal 4: RAN3 to further check whether measurement continuity during the same RRC state (i.e., at handover) is already ensured by current specifications or enhancements are needed.

# 5. Reference

- 3GPP RAN3#124 Chairman’s notes (RAN3_124_agenda_20240524_eom1.doc)

- RP-234054, “New SID: Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#102

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- 3GPP RAN3#123b Chairman’s notes (RAN3_123bis_agenda_20240419_EOM.doc)

- R3-243952, (TP for TR 38.743) Continuous Management-based MDT Data Collection, Nokia, Huawei, Qualcomm, NEC, Samsung, ZTE, CMCC, InterDigital, Ericsson

- RP-233441, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (CMCC, Ericsson), 3GPP RAN#102

- 3GPP RAN3#119bis-e Chairman’s notes (RAN3_119bis-e_agenda_202304026_EOM1.doc)

- R3-243971, 3GPP TR 38.743 V0.0.1, Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN (Release 19)

- 3GPP RAN3#120 Chairman’s notes (RAN3_120_agenda_20230530_EOM1.doc)

- R3-243471, On Continuous MDT for AI/ML, Huawei

# 6. Annex – Draft LS to SA5

3GPP TSG-RAN3 Meeting #125	R3-24abcd

Maastricht, Netherlands, 19th – 23rd August, 2024

Title:	[Draft] LS on Continuous MDT for AI/ML model training

Response to:

Release:	Release 19

Work Item:	FS_NR_AIML_NGRAN_enh

Source:	Huawei [to be RAN3]

To:	SA5

Cc:	RAN2

Contact person:	Damiano Rapone

damiano.rapone@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

As part of the Rel-19 Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, RAN3 agreed to specify an enhancement of the MDT framework termed as Continuous MDT in order to continuously collect measurements from the same UE during the same RRC state and across RRC states that could be used for the AI/ML model (re-)training in OAM.

In order for the AI/ML model (re-)training to be successful the collected data from the UE should be consecutive – i.e., data series not affected by time interruptions due to, e.g., stop of UE data collection and successive resuming – and a time series of information with a certain granularity. RAN3 discussed solutions based on management-based MDT to tackle the following requirements:

- measurement continuity: how to ensure that the same UE keeps on collecting MDT measurements during the same RRC state and across different RRC states;

- trace correlation: how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE.

The above requirements could be fulfilled if a new Job Type is introduced in SA5 specifications, which allows the OAM to configure a MDT session encompassing immediate MDT and logged MDT simultaneously, and assigning to this MDT session the same Trace Reference (TR), while the Trace Recording Session Reference (TRSR) is assigned by the gNB on a per-UE basis within the same MDT session identified by the specific TR. In this manner, a single TR could be assigned to an MDT session with a Job Type equal to “Immediate MDT and Logged MDT”, and this configuration is valid for all UEs selected by the NG-RAN. In order to then correlate the immediate and logged MDT reports from the same UE (properly anonymized, e.g., via IMEI-TAC), the TCE performs such correlation by “collating” the MDT reports having the same couple TR/TRSR: the TR links the MDT reports one to each other (as they both belong to the same MDT session) while the TRSR links the MDT reports to a specific UE, hence realizing the Continuous MDT for the UE.

# 2	Actions

To SA5

ACTION: 	RAN3 respectfully asks SA5 to take the above solution description into consideration and provide feedback to RAN3 on its feasibility in Rel-19.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#125-bis			2024-10-14 – 2024-10-18				Hefei, CN

RAN3#126				2024-11-18 – 2024-11-22				Orlando, US