3GPP TSG-RAN WG3 Meeting #125	R3-244302

Maastricht, NL, 19th Aug – 23rd Aug, 2024

Title: 	Discussinos on timing aspects of AI/ML based CCO

Source: 	Ericsson, InterDigital, Deutsche Telekom, Telecom Italia, Vodfone

Agenda item:	11.3

Document Type:	Discussion and decision

# 1. Introduction

RAN3#125 is the last meeting for the SI on AI/ML in RAN in rel-19. We summarize below the agreements and open points for the AI/ML based CCO use case, as captured in the previous meetings.

At RAN3#123-bis the following agreements were reached:

For AI/ML based CCO,

-	AI/ML Model Training may be located in the OAM and AI/ML Model Inference may be located in the NG-RAN node (gNB-CU).

-	AI/ML Model Training and AI/ML Model Inference may be both located in the NG-RAN node (gNB-CU).

Solution for non-split architecture:

Step 0: gNB predicts the CCO issue.

Step 1: gNB generates the future coverage status based on the predicted CCO issue and other information.

Step 2: gNB sends the future coverage status to neighbour gNBs.

Solution for split-architecture:

Step 0: gNB-CU predicts the CCO issue

Step 1: gNB-CU sends the predicted CCO issue to gNB-DU.

Step 2: gNB-DU generates the future coverage status based on the predicted CCO issue and other local information, whether 
    only local information is used can be further discussed.

Step 3: gNB-DU sends the future coverage status to gNB-CU.

Step 4: gNB-CU sends the future coverage status to neighbour gNBs.

At RAN3#124, the following agreements were captured:

For non-split architecture, future CCO state can be derived via legacy mechanism or AI/ML techniques which is up to implementation.

For split architecture, the standardization of AI/ML based future CCO state inference in the gNB-DU is not supported in R19, how to derive future CCO state is up to implementation in the gNB-DU, e.g., legacy or AI/ML.

The following points have been captured as to be continued:

How to support the future CCO status and/or predicted CCO issue transmission over Xn?

- Alt 1: NG-RAN node 1 may send the future CCO state to NG-RAN node 2 and adjust the CCO status based on feedback from NG-RAN node 2.

- Alt 2: NG-RAN node 1 is fully responsible for prediction and only sends the future CCO state together with related time 
information to NG-RAN node 2.

Reusing the current defined CCO issue and CCO status information for predicted CCO issue and CCO status?

Time information to be further discussed in next meeting.

In this contribution, we address the issue of timing aspects for AI/ML based Coverage and Capacity Optimization (CCO).

# 2. Discussion

## Timing aspects for AI/ML based CCO

Timing related aspects for AI/ML based CCO were left for further discussion as captured at last meeting:

Time information to be further discussed in next meeting.

For split architecture, when the gNB-CU sends a predicted CCO issue to the gNB-DU, the gNB-DU does not need to apply a coverage change immediately, rather it needs to know when the new coverage state should be produced. Therefore, we think the gNB-CU needs to send to the gNB-DU some time related information, like the “reference prediction time” specified in rel-18 which will help the gNB-DU to understand when the new coverage state needs to be applied.

Proposal 1: The gNB-CU signals to the gNB-DU over F1 a reference prediction time associated with the predicted CCO issue.

Similarly, for both split and non-split architectures, NG-RAN Node 1 will signal to NG-RAN node 2 information resulting from the inferred predicted CCO issue. If the CCO issue signalled from NG-RAN node 1 to NG-RAN node 2 is predicted for a future point in time, NG-RAN node 1 would need to include in the Xn signalling a reference prediction time associated with the predicted CCO issue (just like for the equivalent F1 signalling from gNB-CU to gNB-DU).

Proposal 2: If the CCO issue signalled from NG-RAN node 1 to NG-RAN node 2 is predicted for a future point in time, NG-RAN node 1 includes in the Xn signalling a reference prediction time associated with the predicted CCO issue.

If the time at which the predicted CCO issue refers to is far in the future, before the corresponding future CCO coverage state is taken into effect, further measurements can be collected which may lead the gNB-CU to understand that the situation has changed (e.g., the interference at cell edge is no longer critical). To account for this, and particularly for the case when the reference prediction time is far in the future, we think it is good to allow the gNB-CU to send updates of the predicted CCO issue to the gNB-DU.

Equivalently, in such situation NG-RAN gNB1 should be able to send updates to NG-RAN node 2 over Xn, for the predicted CCO issue.

Proposal 3: The gNB-CU can send to the gNB-DU updates of predicted CCO issue.

Proposal 4: NG-RAN node 1 can send to NG-RAN node 2 updates of the predicted CCO issue.

In a way similar to what described above for the predicted CCO issue, the gNB-DU may decide on a future CCO coverage state and signal it to the gNB-CU. In turn, for both split and non-split architectures, NG-RAN node 1 may signal to neighbour NG-RAN nodes a future CCO coverage state. In such case, the signalling from the gNB-DU to gNB-CU over F1 as well as the signalling from NG-RAN node 1 to neighbour NG-RAN nodes over Xn, shall include a reference prediction time associated with the future coverage state.

Proposal 5: If the CCO coverage state signalled from the gNB-DU to the gNB-CU refers to a future point in time, gNB-DU shall signal to the gNB-CU a reference time associated with the future CCO coverage state.

Proposal 6: If the CCO coverage state signalled from the NG-RAN node 1 to NG-RAN node 2 refers to a future point in time, NG-RAN node 1 shall signal to NG-RAN node 2 over Xn a reference time associated with the future CCO coverage state

# 3. Conclusions

Based on the discussion in this paper, we derived the following proposals:

Proposal 1: The gNB-CU sends to the gNB-DU a reference prediction time associated with the predicted CCO issue.

Proposal 2: If the CCO issue signalled from NG-RAN node 1 to NG-RAN node 2 is predicted for a future point in time, NG-RAN node 1 includes in the Xn signalling a reference prediction time associated with the predicted CCO issue.

Proposal 3: The gNB-CU can send to the gNB-DU updates of predicted CCO issue.

Proposal 4: NG-RAN node 1 can send to NG-RAN node 2 updates of the predicted CCO issue.

Proposal 5: If the CCO coverage state signalled from the gNB-DU to the gNB-CU refers to a future point in time, gNB-DU shall signal to the gNB-CU a reference time associated with the future CCO coverage state.

Proposal 6: If the CCO coverage state signalled from the NG-RAN node 1 to NG-RAN node 2 refers to a future point in time, NG-RAN node 1 shall signal to NG-RAN node 2 over Xn a reference time associated with the future CCO coverage state.

A TP provided in R3-244438 mirrors the proposals above and is presented for agreement.

Proposal 5: Agree on the TP to TR 38.743 provided in R3-244438.