3GPP TSG-RAN WG3 #125	R3-244303

Maastricht, Netherlands, 19 – 23 August 2024

Title:	AIML for Network Slicing

Source:	Ericsson, InterDigital, T-Mobile USA, Lenovo, Telecom Italia, Qualcomm, Deutsche Telekom

Agenda Item:	11.2

Document for:	Discussion, Agreement

# Introduction

AI/ML based Network Slicing is one of the two new use cases to be studied in the framework of the Rel-19 Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Study Item [1].

During RAN3#124 meeting, several companies have manifested a high interest in considering the use case consisting of using AIML for assisting the NG-RAN nodes in fulfilling the RAN-specific requirements derived from Network Slices SLAs. Despite the considerable support for this use case and due to lack of time, the use case was not properly discussed during the last RAN3 meeting.

In this paper, we reiterate our interest in considering this use case for the Rel-19 Study Item discussions.

# Discussion

Network slicing, as defined in 3GPP TS 28.530, is a paradigm where logical networks/partitions are created, with appropriate isolation, resources and optimized topology to serve a purpose or service category (e.g. use case/traffic category, or for MNO internal reasons) or customers (logical system created "on demand"). A network slice is a logical network that provides specific network capabilities and characteristics, such that the requirements of the services running on top of the network slice are supported and/or satisfied.

As far as the NG-RAN is concerned, as presented in TS 38.300 [1], among many others, the following key principles apply for support of Network Slicing:

RAN awareness of slices

-	NG-RAN supports a differentiated handling of traffic for different network slices which have been pre-configured. How NG-RAN supports the slice enabling in terms of NG-RAN functions (i.e. the set of network functions that comprise each slice) is implementation dependent.

Resource management between slices

-	NG-RAN supports policy enforcement between slices as per service level agreements. It should be possible for a single NG-RAN node to support multiple slices. The NG-RAN should be free to apply the best RRM policy for the SLA in place to each supported slice.

A network slice Service Level Agreement (SLA) is a contractual framework agreed upon between the slice vertical and the network/service provider, characterising the services provided by the network slice in terms of committed value ranges for the services’ parameters such as delay, throughput, etc. In each network domain, the service level requirements expressed in the SLA translate into requirements on measurable metrics for each sub-domain in the network. The OAM system has the responsibility of translating the SLA to targets that every subsystem of the access, the core and the transport networks can understand and fulfill. In this way, the RAN is configured by the OAM system with clear requirements concerning each supported network slice, as it is clearly stated in TS38.300.

On the other hand, NG-RAN based AI/ML as defined in Release 18 plays a key role in support of coordinated radio resources management between neighbor gNBs. As such, the Release 18 AI/ML framework could be extended to enable coordinated radio resource management between neighbor gNBs for the purpose of fulfilling the RAN related requirements mapping to the SLA in place for each supported slice.

Proposal 1: RAN3 to study extensions of the Release 18 AI/ML framework to enable coordination actions between neighbour gNBs for the purpose of network slice RAN level requirements fulfilment.

The solution we propose to study would entail two or more RAN nodes to collaborate with one another, in terms of exchanging information, predictions, triggering actions etc., to achieve fulfilment of the requirements established for commonly supported slices.

Proposal 2: RAN3 to study the extension of the DATA COLLECTION REPORTING procedure to support exchanging of measured/predicted metrics reflecting per slice requirements to enable AI/ML based coordination of actions aimed at achieving per slice requirement fulfilment.

## Network Slice specific requirements for the NG-RAN

During RAN3-123bis some comments were made towards RAN level requirements derived from the SLA. We would like to point out that, as specified in TS38.300, the RAN is already today configured with such per slice requirements, hence the approach described in this paper would rely on “bringing such requirements to light”, rather than introducing new concepts. 
Requirements that may be typically configured at a gNB and derived for a network slice could be in the form of measurable targets to be fulfilled for a given percentage of session time.

As an example, for a given S-NSSAI, the RAN may be configured with a throughput target in DL and a throughput target in UL. The target requirement the RAN will have to fulfil for the network slice consists of a throughput at or above the established target for at least x% of the slice session time.

Per slice requirements at the gNB may be established on a per cell or per node basis. Examples of such requirements may be as follows:

- Requirement 1: Average per slice DL throughput for percentage of session time T

- Requirement 2: Average per slice UL throughput for percentage of session time T

- Requirement 3: Average per slice DL packet delay for percentage of session time T

- Requirement 4: Average per slice UL packet delay for percentage of session time T

- Requirement 5: Average per slice DL packet loss rate for percentage of session time T

- Requirement 6: Average per slice UL packet loss rate for percentage of session time T

The parameter T may be specific and different for each requirement.

Obviously, such requirements could be checked also with other WGs such as SA5.

Proposal 3: RAN3 to study RAN specific per slice requirements based on measurable metrics used to determine the level of fulfillment of such requirements.

## Requirement fulfilment status in RAN nodes

A slice may be supported across the entire network, or it may be geographically confined to a few cells or nodes. Regardless, whenever a node/cell cannot offer the required performance for traffic belonging to a slice, the overall network wide performance for the slice is lowered and the operator is at risk of not fulfilling its SLA for the slice.

In such cases, one potential action that could bring the node/cell back to fulfilling the per slice requirements is to offload part of its traffic to a neighbor cell/node. Such an action will free resources at the source node/cell and therefore allow to serve the network slice affected by poor performance in a better way.

However, to perform such an action, the source RAN node should be aware of a measured/predicted per slice resource status and requirement fulfilment status at the target node. Namely, as a minimum, the source node needs to receive from the target node the following:

- An indication of the measured available capacity and radio resource status per slice in UL and DL (this is already available via the Xn: Resource Status Reporting procedure)

- An indication of the predicted available capacity and radio resource status per slice in UL and DL

- An indication per slice of the measured requirement fulfillment level, e.g. for each network slice requirement, whether it is fulfilled or not fulfilled. 
Taking the Requirements 1-6 provided as example above, a source node may signal to a target node a flag per requirement stating whether Requirement x is fulfilled or not fulfilled, namely:

  - Requirement 1-> Fulfilled

  - Requirement 2-> Not Fulfilled

  - Requirement n-> Fulfilled

This information could also include details about the delta between current measured values and the target requirements for such targets.

- An indication per slice of the predicted requirement fulfillment level, e.g. for each network slice requirement, whether it is predicted to be fulfilled or not fulfilled. 
Taking the Requirements 1-6 provided as example above, a source node may signal to a target node a flag per requirement stating whether Requirement x is predicted to be fulfilled or not fulfilled, namely:

  - Requirement 1-> Fulfilled

  - Requirement 2-> Not Fulfilled

  - Requirement n-> Fulfilled

This information could also include details about the delta between the predicted values and the target requirements for such targets.

With this information the source node is able to determine whether an offloading action towards a neighbour node is beneficial in terms of fulfilling the per slice requirements. As an example, if a target gNB predicts that its per slice targets will be fulfilled in the future and if the source node calculates that, by offloading a certain amount of traffic towards the target node, it will move from not fulfilling to fulfilling its per slice requirements, then the offloading action would be overall beneficial, and the source node can carry it out.

During RAN3-123bis it was commented whether the Slice Available Capacity is sufficient to derive if the per slice requirements are fulfilled or not. The claim made was that if the Slice Available Capacity is available for a slice, then the slice SLA requirements are fulfilled, while if there is no Slice Available Capacity per slice, the slice SLA requirements are not fulfilled.

For clarity, this IE is defined in TS38.423 as follows:

#### 9.2.2.55	Slice Available Capacity

The Slice Available Capacity IE indicates the amount of resources per network slice that are available per cell relative to the total NG-RAN resources per cell. The Slice Available Capacity Value Downlink IE and the Slice Available Capacity Value Uplink IE can be weighted according to the ratio of the corresponding cell capacity class values contained in the Composite Available Capacity Group IE, if available.

The first observation to make is that it is not possible to derive whether per slice SLA requirements are fulfilled or not by simply looking at the Slice Available Capacity. Indeed, the following cases can occur:

- A RAN node may have no Slice Available Capacity left and still fulfil the SLA requirements.  Namely all the capacity for a given slice is used and the targets established to fulfil the SLA requirements are met.

- A RAN node may have no Slice Available Capacity left and not fulfil the SLA requirements. Namely all the capacity for a given slice is used and the targets established to fulfil the SLA requirements are not met.

- A RAN node may have spare Slice Available Capacity left and not fulfil the SLA requirements. Examples of why this could happen are:

  - F1-U delays are excessive, and they may prevent fulfilment of slice SLA delay requirements. Note that the Slice Available Capacity may not take F1-U delays into account (indeed the definition reported above from TS38.423 does not mandate this and it leaves it up to implementation)

  - Many UEs at cell edge cannot be served with appropriate resources. As a matter of implementation, a RAN node may decide not to use all the resources available towards such cell edge UEs as this may still result in lack of SLA requirement fulfilment. Instead, those resources could be used to served other UEs in better radio conditions.

On the basis of the above explanations, we conclude that measured/predicted Slice Available Capacity, measured/predicted radio resource status and measured/predicted per slice requirement fulfillment level are needed to be exchanged between RAN nodes.

Proposal 4: It is proposed to base the AI/ML supported Network Slicing use case on signalling between gNBs of measured/predicted slice available capacity, measured/predicted radio resource status as well as measured/predicted per slice requirement fulfilment level.

Furthermore, as the RAN level requirements corresponding to a specific SLA are derived by the OAM, it may be beneficial for the OAM to receive from gNBs the measured slice requirement fulfillment level. This would be beneficial for two reasons: 1) to enable the availability of training data at the OAM, in case the OAM hosts training for an AI/ML model for network slicing; 2) To enable the OAM to have observability on the per slice requirement fulfillment levels, so that the OAM can assess the overall level of SLA fulfilment.

Proposal 5: RAN3 to study the benefits of signaling to the OAM per gNB measured/predicted slice requirement fulfilment levels.

## AI/ML assisted network slicing

Based on the proposals above, the concept described in this paper in support for an AI/ML based network slicing use case relies on the following points:

- RAN nodes are configured with per slice target requirements derived from SLAs, consisting of targets established for measurable metrics at the RAN

- RAN nodes exchange measured/predicted per slice available capacity and SLA requirement fulfilment levels

- Based on the received information and internal measurements/predictions, RAN nodes infer actions allowing a better overall fulfilment of the per slice requirements, such as offloading actions or prioritizing offloading from specific source cells

- RAN nodes signal to the OAM RAN-node level measured SLA requirement fulfilment levels, to enable AI/ML model training at the OAM and to achieve per node slice requirement fulfilment observability.

# Conclusions and Proposals

In this paper, new concepts have been presented in support for AI/ML based network slicing use cases. The following proposals have been derived:

Proposal 1: RAN3 to study extensions of the Release 18 AI/ML framework to enable coordination actions between neighbour gNBs for the purpose of network slice RAN level requirements fulfilment.

Proposal 2: RAN3 to study the extension of the DATA COLLECTION REPORTING procedure to support exchanging of measured/predicted metrics reflecting per slice requirements to enable AI/ML based coordination of actions aimed at achieving per slice requirement fulfilment.

Proposal 3: RAN3 to study RAN specific per slice requirements based on measurable metrics used to determine the level of fulfilment of such requirements.

Proposal 4: It is proposed to base the AI/ML supported Network Slicing use case on signalling between gNBs of measured/predicted slice available capacity, measured/predicted radio resource status as well as measured/predicted per slice requirement fulfilment level.

Proposal 5: RAN3 to study the benefits of signalling to the OAM per gNB measured/predicted slice requirement fulfilment levels.

A TP mirroring the proposals above is available in R3-244304.

# References

RP-240323: Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN

TS 38.300  v. 18.1.0