3GPP TSG-RAN WG3 Meeting #125	R3-244304

Maastricht, Netherlands, 19 – 23 August 2024

Title: 	TP for TR 38.743 on AIML for Network Slicing

Source: 	Ericsson, InterDigital, T-Mobile USA, Lenovo, Telecom Italia, Qualcomm, Deutsche Telekom

Agenda item:	11.2

Document Type:	Discussion and decision

# TP to TR 38.743

This TP mirrors the discussions and proposals in R3-244303.

# 4	Use cases and Solutions

## 4.1	AI/ML based Network Slicing

### 4.1.1	Use case description

Editor Note: Capture the description of use case

Support of network slicing in NG-RAN is defined in TS 38.300 [x].

The NG-RAN plays a key role in taking mobility, load balancing and Radio Resources Management decisions for the purpose of meeting target requirements derived from the SLA of each supported network slice.

AI/ML function can analyze metrics related to network and UE level performance related to perform optimal resource management and mobility decisions for network slicing to meet the requirements.

### 4.1.2	Solutions and standard impacts

Editor Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

#### 4.1.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions can be considered for supporting AI/ML-based network slicing:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

#### 4.1.2.4	Input data of AI/ML based Network Slicing:

To predict the optimized network slicing decisions, a gNB may need the following information as input data for AI/ML-based network slicing:

From local node:

-	Measured/Predicted radio resource status per slice

-	Measured/Predicted slice available capacity

-	Legacy predicted UE trajectory

-	Measured/Predicted slice requirements fulfilment

From neighbouring gNBs:

-	Measured/Predicted radio resource status per slice

-	Measured/Predicted slice available capacity

-	Measured/Predicted slice requirements fulfilment

-	Per slice UE performance measurements

From the UE:

-	UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

#### 4.1.2.5	Output data of AI/ML based Network Slicing:

AI/ML-based network slicing model in a gNB can generate the following information as output:

-	Predicted radio resource status per slice

-	Predicted slice available capacity

-	Resource management decisions for resources within RRM policies (used by gNB internally)-	Slice aware mobility decisions (used by gNB internally)

-	Predicted slice requirements fulfilment

#### 4.1.2.6	Feedback of AI/ML based Network Slicing:

To optimize the performance of AI/ML-based network slicing model, the following feedback can be considered to be collected from gNBs:

-	Measured Radio resource status per slice

-	Measured Slice available capacity

-	Legacy UE performance feedback for those UEs handed over from the source gNB

-	Finer granularity UE performance feedback for those UEs handed over from the source gNB. FFS on the exact level of finer granularity

- Measured slice requirements fulfilment.

#### 4.1.2.7	Potential standard impacts:

Following standard impacts is listed for subsequent Rel-19 normative work compared with what was specified during Rel-18.

Xn interface:

-	Enhanced existing procedure to collect predicted information between gNBs:

■	Predicted radio resource status per slice

■	Predicted slice available capacity

■	Measured slice requirements fulfilment indication

■	Predicted slice requirements fulfilment indication

RAN-OAM interface:

-	Signalling of measured slice requirement fulfilment indications from RAN to OAM