3GPP TSG-RAN WG3 Meeting #125	R3-244306

Maastricht, NL, 19 - 23 August 2024

Agenda item:	 10.2

Source:		Lenovo

Title:		Discussion on MRO for LTM

Document for:		Discussion and Decision

1	Introduction

RAN3#123bis meeting agreed that:

Work on scenarios of near failure LTM

Work on scenarios for the differentiation of too early LTM, too late LTM and LTM to wrong cell

RAN3#124 meeting agreed that:

RAN3 to prioritize MCG LTM over SCG LTM.

RAN3 takes RAN2#125-bis agreement on MRO for LTM scenarios as baseline for further study.

For failures due to wrong selection of candidate LTM cell, CU is in charge of root cause analysis and performs optimization.

For failures due to inappropriate cell switch triggering (e.g. wrong cell selection at cell switch, wrong cell switch timing, …) source DU performs optimization.

For failures due to inappropriate cell switch triggering (e.g. wrong cell selection at cell switch, wrong cell switch timing, …) FFS if CU or DU performs root cause analysis.

For stage-2, FFS if existing problem definitions (Connection failure due to intra-system mobility) can be reused or if a new chapter needs to be created.

RAN3 to discuss enhancements related to the information of RACH-less access and RACH-based access from UE related to MRO for LTM.

In this paper, we would further discuss the details of MRO for LTM.

2	Discussion

In RAN2#125bis meeting, it has agreed that:

RAN2 considers SHR, RA report and RLF for MCG LTM SON.

RAN2 will start work on MCG LTM.

In RAN2#126 meeting, it has agreed that:

If available, log the L1 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure.

Reuse the existing approach of using timeConnFailure and the reconnectCellId in RLF-report also for LTM failures, details TBD.

Log the LTM cell ID upon performing recovery an LTM candidate cell, details TBD e.g. which field.

Extend lastHO-Type in RLF-Report to indicate the LTM cell switch as last executed mobility procedure.

Based on agreements in RAN2 and RAN3, in this paper, we would focus on MCG LTM to mainly discuss RLF report and SHR for MCG LTM.

For SCG LTM, it is introduced in R18 with same priority as MCG LTM to reduce interruption time during PSCell change procedure. SCG LTM failure case and even near failure successful SCG LTM case should be avoided to improve network performance, we would further discuss MRO for SCG LTM in later meetings when MRO for MCG LTM is stable.

  - 2.1 MRO for connection failure in an MCG LTM procedure

Until RAN2#126 meeting, the following failure cases were agreed to consider:

For LTM MRO, RAN2 considers the following three connection failure cases:

- -	Too late LTM

- -	Too early LTM

- -	LTM to wrong cell

For too late LTM, the following sub-cases are considered but we may down prioritize later (not limiting):

- -	Case 1a: the UE detects RLF in source cell after receiving LTM candidate configurations and performs reestablishment procedure.

- -	Case 1b: the UE detects RLF in source cell after receiving LTM candidate configurations, selects an LTM candidate cell, detects HOF with the selected LTM cell.

- -	Case 1c: the UE detects RLF in source cell after receiving LTM candidate configurations, and successfully completes LTM execution with the selected LTM candidate cell.

For too early LTM, the following sub-cases are considered but we may down prioritize later (not limiting):

- -	Case 2a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure with the source cell.

- -	Case 2b: the UE detects HOF/RLF in the LTM target cell, selects the source cell which is also an LTM candidate cell, detects HOF with the source cell, and performs reestablishment procedure.

- -	Case 2c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected source cell which is also an LTM candidate cell.

LTM to wrong cell, the following sub-cases are considered but we may down prioritize later (not limiting):

- -	Case 3a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure in a cell other than the source cell and the target cell.

- -	Case 3b: the UE detects HOF/RLF in the LTM target cell, selects an LTM candidate cell which is different from the source or target one, detects HOF with the selected LTM candidate cell, and performs reestablishment procedure.

- -	Case 3c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected LTM candidate cell which is different from the source or target one.

Based on above cases agreed by RAN2, similar as Too Late Handover, Too Early Handover, and Handover to Wrong Cell defined in TS38.300 [1], we should capture stage 2 descriptions for Too Late MCG LTM, Too Early MCG LTM, and MCG LTM to wrong cell in TS38.300 as below:

- Too Late MCG LTM: UE receives configuration for a LTM procedure, while an RLF occurs after the UE has stayed for a long period of time in the serving cell; the UE attempts to re-establish the radio link connection in a different cell.

- Too Early MCG LTM: a radio link failure occurs shortly after a successful LTM cell switch from a source cell to a target cell, or a failure occurs during the LTM cell switch execution; the UE attempts to re-establish the radio link connection in the source cell.

- MCG LTM to wrong cell: a radio link failure occurs shortly after a successful LTM cell switch from a source cell to a target cell, or a failure occurs during the LTM cell switch execution; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

Since LTM is a L1/L2 triggered mobility rather than a L3 handover procedure like normal HO or CHO or DAPS, we prefer to have a separate failure type definition for LTM to make it clearer.

Proposal 1: Too Late MCG LTM, Too Early MCG LTM, and MCG LTM to wrong cell should be considered:

- Too Late MCG LTM: UE receives configuration for a LTM procedure, while an RLF occurs after the UE has stayed for a long period of time in the serving cell; the UE attempts to re-establish the radio link connection in a different cell.

- Too Early MCG LTM: a radio link failure occurs shortly after a successful LTM cell switch from a source cell to a target cell, or a failure occurs during the LTM cell switch execution; the UE attempts to re-establish the radio link connection in the source cell.

- MCG LTM to wrong cell: a radio link failure occurs shortly after a successful LTM cell switch from a source cell to a target cell, or a failure occurs during the LTM cell switch execution; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

In LTM preparation phase, the CU provides a list of LTM candidate cell to each candidate target DU or target DU, the candidate target DU or target DU may accept or reject cell(s) in the list of LTM candidate cell provided by the CU. RAN3#124 meeting agreed that for failures due to wrong selection of candidate LTM cell, CU is in charge of root cause analysis and performs optimization, e.g. based on the RLF report, the CU makes root cause analysis to decide that the failure is caused due to wrong selection of LTM candidate cell for the case that the cell for re-establish is not one of LTM candidate cells provided by CU to candidate target DU or target DU, if so, the CU may modify the list of LTM candidate cell.

In LTM execution phase, the source DU selects the target cell for LTM cell switch execution. When a connection failure happens in a LTM procedure, it may be inappropriate cell switch triggering at source DU (e.g. wrong target cell selection, or wrong cell switch timing). Based on RAN2’ agreements, we can find that the RLF report includes necessary information e.g. L1 measurements, cell id of an LTM candidate cell in which performing LTM failure recovery, reconnect cell id and mobility type indicating the LTM cell switch. Since CU receives the RLF report earlier than DU, and CU has more information than DU, similar as the failure case due to wrong selection of candidate LTM cell, the CU is also the better node to perform root cause analysis for the failure case due to inappropriate cell switch triggering at source DU (e.g. wrong target cell selection, or wrong cell switch timing).

Proposal 2: CU performs root cause analysis for the failure case due to inappropriate cell switch triggering (e.g. wrong target cell selection, or wrong cell switch timing).

For example, for the case that the cell for re-establish is one of the LTM candidate cells provided by CU to candidate target DU or target DU, but not the target cell included in the LTM cell switch command, the CU may detect that the connection failure is due to wrong target cell selection at source DU, then CU may send RLF report and the detected failure cause (e.g. LTM to wrong cell) to source DU in F1 interface e.g. via the existing Access and Mobility Indication message, in such a way, the source DU may optimize the target cell for LTM execution.

For another example, if the cell for re-establish is one of the LTM candidate cells provided by CU to candidate target DU or target DU, and the failed PCell and previous PCell indicated in the RLF report are the source PCell, the CU may detect that the connection failure is due to late cell switch timing at source DU, CU may send RLF report and the detected failure cause (e.g. too late LTM) to source DU in F1 interface e.g. via the existing Access and Mobility Indication message.

Additionally, if the cell for re-establish is the source PCell which triggers the LTM cell switch, the CU may detect that the connection failure is due to early LTM cell switch timing at source DU, CU may send RLF report and the detected failure cause (e.g. too early LTM) to source DU in F1 interface e.g. via the existing Access and Mobility Indication message.

Proposal 3: CU sends RLF report and the detected failure cause (e.g. Too Late LTM, Too Early LTM, or LTM to wrong cell) to source DU in F1 interface e.g. via the Access and Mobility Indication message.

  - 2.2 MRO for near failure in an MCG LTM procedure

In legacy, SHR is introduced to detect sub-optimal successful handover. For one LTM cell switch execution in a MCG LTM procedure, it would be a near-failure successful LTM PCell switch. The UE may store near-failure successful MCG LTM related information to assist network to perform MRO analysis and optimisation. Since a LTM procedure is triggered based on L1 measurement report, RAN2#126 meeting agreed to log L1 measurements for serving cell, target cell and other LTM candidate cells if available in RLF report upon RLF or mobility failure, similar as failure case due to wrong selection of LTM candidate cell, it is beneficial for the UE to report L1 measurement results for source cell, target cell and other LTM candidate cells if available in SHR when the LTM cell switch execution is successful. RAN3 should send an LS to RAN2 for confirmation if we achieve agreement.

Proposal 4: It is beneficial for the UE to report L1 measurement results for source cell, target cell and other LTM candidate cells if available in SHR when LTM cell switch execution is successful.

In legacy, for analysis of such sub-optimal successful handover, the UE may collect SHR based on the triggers configured by the network, if received, and makes the SHR available to the network. For legacy SHR, T310/T312/T304 related trigger threshold is configured by network to trigger SHR storing or generating at UE side.

LTM is a procedure intended to reduce interruption time during mobility, that is why early TA acquisition procedure may be triggered by PDCCH order or realized through UE-based TA measurement. In the former case, the gNB/gNB-DU to which the LTM candidate cell belongs calculates the TA value and sends it to the gNB/gNB-DU to which the serving cell belongs via gNB-CU, the serving cell sends the TA value in the LTM cell switch command MAC CE when triggering LTM cell switch. In the latter case, the UE performs TA measurement for the LTM candidate cells after being configured by RRC, the UE applies the TA value measured by itself and performs RACH-less LTM upon receiving the cell switch command. If the valid TA value is provided in the cell switch command, the UE applies the TA value as instructed by the network. In the case where UE-based TA measurement is configured, but no valid TA value is provided in the cell switch command, the UE applies the valid TA value by itself if available. Meanwhile, the UE performs RACH-less LTM cell switch upon receiving the cell switch command. If no valid TA value is available, the UE performs RACH-based LTM cell switch. If RACH-less based LTM is not configured or is not initiated or failed, the interruption time will increase even LTM is successful. Therefore, it is necessary to store and report early TA acquisition related information or RACH based LTM related information in SHR to optimize LTM related configuration. To enable this, LTM specific trigger condition(s) for SHR needs to be considered, e.g. TA value acquired in early TA acquisition is invalid, TA value measured by UE is invalid, RACH-less LTM cell switch is not performed or performed but not successful.

Proposal 5: Consider LTM specific trigger condition(s) for SHR, e.g. TA value acquired in early TA acquisition is invalid, TA value measured by UE is invalid, RACH-less LTM cell switch is not performed or performed but not successful.

3	Conclusion

In this paper, MRO for LTM are further discussed. We have the following proposals:

Proposal 1: Too Late MCG LTM, Too Early MCG LTM, and MCG LTM to wrong cell should be considered:

- Too Late MCG LTM: UE receives configuration for a LTM procedure, while an RLF occurs after the UE has stayed for a long period of time in the serving cell; the UE attempts to re-establish the radio link connection in a different cell.

- Too Early MCG LTM: a radio link failure occurs shortly after a successful LTM cell switch from a source cell to a target cell, or a failure occurs during the LTM cell switch execution; the UE attempts to re-establish the radio link connection in the source cell.

- MCG LTM to wrong cell: a radio link failure occurs shortly after a successful LTM cell switch from a source cell to a target cell, or a failure occurs during the LTM cell switch execution; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

Proposal 2: CU performs root cause analysis for the failure case due to inappropriate cell switch triggering (e.g. wrong target cell selection, or wrong cell switch timing).

Proposal 3: CU sends RLF report and the detected failure cause (e.g. Too Late LTM, Too Early LTM, or LTM to wrong cell) to source DU in F1 interface e.g. via the Access and Mobility Indication message.

Proposal 4: It is beneficial for the UE to report L1 measurement results for source cell, target cell and other LTM candidate cells if available in SHR when LTM cell switch execution is successful.

Proposal 5: Consider LTM specific trigger condition(s) for SHR, e.g. TA value acquired in early TA acquisition is invalid, TA value measured by UE is invalid, RACH-less LTM cell switch is not performed or performed but not successful.

Reference

[1] TS38.300