3GPP TSG-RAN WG3 Meeting #125	R3-244308

Maastricht, NL, 19 - 23 August 2024

Agenda item:	 10.3.1

Source:		Lenovo

Title:		Discussion on MRO for intra-NTN mobility

Document for:		Discussion and Decision

1	Introduction

It was agreed in RAN3#123bis meeting that [1]:

MRO mechanisms for intra-NTN should be addressed based on failure scenarios first, work on near failure scenarios later.

RAN3#124 meeting further agreed that [2]:

Take the connection failure scenarios in §15.5.2.2.2 of TS 38.300 as starting point with necessary update, if needed, in MRO for intra-NTN mobility.

MRO for intra-NTN mobility shall consider the following trigger conditions:

- Measurement-based trigger condition;

- A time-based trigger condition;

- A location-based trigger condition;

- A time-based and Measurement-based combined trigger condition;

- A location-based and Measurement-based combined trigger condition;

Do not consider MRO for feeder link switch.

MRO for intra-NTN mobility shall consider both conditional and non-conditional intra-NTN handover.

FFS on whether there are any enhancements to RLF report due to support for multiple trigger conditions in intra-NTN mobility.

FFS on whether to consider any enhancement for MDT configuration e.g., area scope and trigger condition for NTN.

Discuss MRO for RACH-less HO of NTN, only for those different issues identified compared with MRO for LTM.

Time information when RLF occurs needs to be included in the RLF report, other information to be included can be checked as well in next meeting.

In this paper, we would discuss details of MRO for intra-NTN mobility.

2	Discussion

An NTN provides non-terrestrial NR access to the UE by means of an NTN payload and an NTN Gateway, depicting a service link between the NTN payload and a UE, and a feeder link between the NTN Gateway and the NTN payload.

Three types of service links are supported:

-	Earth-fixed: provisioned by beam(s) continuously covering the same geographical areas all the time (e.g., the case of GSO satellites);

-	Quasi-Earth-fixed: provisioned by beam(s) covering one geographic area for a limited period and a different geographic area during another period (e.g., the case of NGSO satellites generating steerable beams);

-	Earth-moving: provisioned by beam(s) whose coverage area slides over the Earth surface (e.g., the case of NGSO satellites generating fixed or non-steerable beams).

Currently, intra-NTN mobility is supported. UE may support mobility between gNBs operating with NTN payloads in different orbits (e.g., GSO, NGSO at different altitudes). Intra-NTN mobility may by an ordinary handover, or a CHO. Furthermore, for CHO in NTN, location based CHO and time based CHO are supported.

For normal mobility in NTN, Event D1 is introduced that measurement report is triggered when distance between UE and a reference location referenceLocation1 becomes larger than configured threshold distanceThreshFromReference1 and distance between UE and a reference location referenceLocation2 becomes shorter than configured threshold distanceThreshFromReference2.

In time based intra-NTN CHO, CondEvent T1 is introduced that the time measured at UE becomes more than configured threshold t1-Threshold but is less than t1-Threshold + duration. Time-based CHO can be performed via RACH-less.

In location based intra-NTN CHO, CondEvent D1 is introduced for Quasi-Earth-fixed case, location based trigger condition is satisfied when distance between UE and a reference location referenceLocation1 becomes larger than configured threshold distanceThreshFromReference1 and distance between UE and a reference location referenceLocation2 of conditional reconfiguration candidate becomes shorter than configured threshold distanceThreshFromReference2. On the other hand, CondEvent D2 is introduced for Earth-moving case, location based trigger condition is satisfied when distance between UE and a moving reference location determined based on referenceLocation1 becomes larger than configured threshold distanceThreshFromReference1 and distance between UE and a moving reference location determined based on referenceLocation2 of conditional reconfiguration candidate becomes shorter than configured threshold distanceThreshFromReference2.

A time-based or a location-based trigger condition is always configured together with one of the measurement-based trigger conditions (CHO events A3/A4/A5). It is up to UE implementation how the UE evaluates the time-or location-based trigger condition together with the RRM measurement-based event.

RAN3#124 meeting agreed that MRO for intra-NTN mobility shall consider both conditional and non-conditional intra-NTN handover, and the following trigger conditions for MRO for intra-NTN mobility shall be considered:

- Measurement-based trigger condition;

- A time-based trigger condition;

- A location-based trigger condition;

- A time-based and Measurement-based combined trigger condition;

- A location-based and Measurement-based combined trigger condition;

Similar as legacy handover procedure, when a connection failure happens in intra-NTN mobility, the RLF report can be reused to include NTN related information to help network to optimize mobility related confifuration (e.g. optimize the distance threshold to trigger handover, or distance based CHO execution condition, or time based CHO execution condition, etc.).

For CHO configured with a location-based trigger condition, or CHO configured with location-based and measurement-based combined trigger condition, a connection failure may happen due to configuring inappropriate parameter(s), besides measurement results of source cell, target cell and candidate cell(s) when the connection failure happens, the distance between UE and a reference location/moving reference location of source cell when the connection failure happens, and the distance between UE and a reference location/moving reference location of candidate or target cell when the connection failure happens may be stored or reported by the UE in RLF report.

For CHO configured with location-based and measurement-based combined trigger condition, there may be the case that location based CHO execution condition is fulfilled but measurement based CHO execution condition is not fulfilled, to enable network know that measurement based CHO execution condition is setting strictly, measurement results of source cell, target cell and candidate cell(s) when location based CHO execution condition is fulfilled, may be stored or reported by the UE in RLF report.

For CHO configured with location-based and measurement-based combined trigger condition, there may be the case that measurement based CHO execution condition is fulfilled but location based CHO execution condition is not fulfilled, to enable network know that location based CHO execution condition is setting strictly, the distance between UE and a reference location/moving reference location of source cell when measurement based CHO execution condition is fulfilled, or the distance between UE and a reference location/moving reference location of candidate cell(s) when measurement based CHO execution condition is fulfilled, may be stored or reported by the UE in RLF report.

Proposal 1: For connection failure in intra-NTN mobility, RLF report can be enhanced to include NTN related information, e.g. the distance between UE and a reference location/moving reference location of source cell when the connection failure happens, the distance between UE and a reference location/moving reference location of candidate or target cell when the connection failure happens, measurement results of source cell, target cell and candidate cell(s) when location based CHO execution condition is fulfilled, the distance between UE and a reference location/moving reference location of source cell when measurement based CHO execution condition is fulfilled, or the distance between UE and a reference location/moving reference location of candidate cell(s) when measurement based CHO execution condition is fulfilled.

For CHO configured with a time-based trigger condition, or CHO configured with time-based and measurement-based combined trigger condition, a connection failure may happen due to configuring inappropriate parameter(s), the time when the connection failure happens may be derived based on the existing time information IE (e.g. timeSinceCHO-Reconfig IE, timeConnFailure IE, or timeSinceFailure IE) in RLF report.

For CHO configured with time-based and measurement-based combined trigger condition, there may be the case that the absolute time value t1-Threshold + duration arrives, but measurement based CHO execution condition is not fulfilled, to enable network know that measurement based CHO execution condition is setting strictly, measurement results of source cell, target cell and candidate cell(s) at the absolute time value t1-Threshold or t1-Threshold + duration, may be stored or reported by the UE in RLF report.

Proposal 2: For connection failure in intra-NTN mobility, RLF report can be enhanced to include measurement results of source cell, target cell and candidate cell(s) at the absolute time value t1-Threshold or t1-Threshold + duration.

For failure detection and MRO related optimisation at network side, a failure indication may be sent to the node last serving the UE when the NG-RAN node fetches the RLF REPORT from UE by triggering Failure Indication procedure over Xn, or Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG. Since the NTN payload just transparently forwards the radio protocol received from the UE (via the service link) to the NTN Gateway (via the feeder link) and vice-versa, the legacy RLF report forwarding mechanism can be taken as baseline for intra-NTN mobility.

Proposal 3: The legacy RLF report forwarding mechanism can be taken as baseline for intra-NTN mobility.

Also, RAN3#124 meeting agreed not consider MRO for feeder link switch, we can find that workload to support MRO mechanism for failure case in intra-NTN mobility is not heavy, we can further discuss MRO for near failure case.

Similar as legacy handover procedure, when a near-failure successful mobility happens in NTN, the SHR can be reused to include NTN related failure information. In legacy, T310/T312/T304 related SHR trigger threshold is configured by network. Compared with legacy handover procedure, distance information or time information is important in NTN mobility, it is up to RAN2 to consider which trigger condition can be configured for SHR for NTN mobility, e.g. whether to introduce NTN specific SHR trigger condition, for example, whether to introduce distance related SHR trigger threshold or time related SHR trigger threshold. When the trigger condition is fulfilled, the UE may store and report SHR to include NTN related information to help network to optimize mobility related confifuration (e.g. optimize the distance threshold to trigger handover, or distance based CHO execution condition, or time based CHO execution condition, etc.), the distance between UE and a reference location/moving reference location of source cell when handover is successful, or the distance between UE and a reference location/moving reference location of target cell when handover is successful, may be included in SHR.

Proposal 4: For near-failure successful intra-NTN mobility, SHR can be enhanced to include NTN related information, e.g. the distance between UE and a reference location/moving reference location of source cell when handover is successful, or the distance between UE and a reference location/moving reference location of target cell when handover is successful.

For legacy SHR, it may be forwarded to the node(s) which configured the SHR trigger causing the SHR to be generated, by using the ACCESS AND MOBILITY INDICATION message over Xn or by means of the Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG. Upon retrieval of an SHR, the receiving node may analyse whether its mobility configuration needs adjustment. Since the NTN payload just transparently forwards the radio protocol received from the UE (via the service link) to the NTN Gateway (via the feeder link) and vice-versa, the legacy SHR forwarding mechanism can be taken as baseline for intra-NTN mobility.

Proposal 5: The legacy SHR forwarding mechanism can be taken as baseline for intra-NTN mobility.

3	Conclusion

In this paper, MRO enhancements for intra-NTN mobility are discussed. We have the following proposals:

Proposal 1: For connection failure in intra-NTN mobility, RLF report can be enhanced to include NTN related information, e.g. the distance between UE and a reference location/moving reference location of source cell when the connection failure happens, the distance between UE and a reference location/moving reference location of candidate or target cell when the connection failure happens, measurement results of source cell, target cell and candidate cell(s) when location based CHO execution condition is fulfilled, the distance between UE and a reference location/moving reference location of source cell when measurement based CHO execution condition is fulfilled, or the distance between UE and a reference location/moving reference location of candidate cell(s) when measurement based CHO execution condition is fulfilled.

Proposal 2: For connection failure in intra-NTN mobility, RLF report can be enhanced to include measurement results of source cell, target cell and candidate cell(s) at the absolute time value t1-Threshold or t1-Threshold + duration.

Proposal 3: The legacy RLF report forwarding mechanism can be taken as baseline for intra-NTN mobility.

Proposal 4: For near-failure successful intra-NTN mobility, SHR can be enhanced to include NTN related information, e.g. the distance between UE and a reference location/moving reference location of source cell when handover is successful, or the distance between UE and a reference location/moving reference location of target cell when handover is successful.

Proposal 5: The legacy SHR forwarding mechanism can be taken as baseline for intra-NTN mobility.

Reference

[1] Chairman notes for RAN3#123bis meeting

[2] Chairman notes for RAN3#124 meeting