3GPP TSG-RAN WG3 Meeting #125	R3-244309

Maastricht, NL, 19 - 23 August 2024

Agenda item:	 10.4

Source:		Lenovo

Title:		(TP for SON BLCR for 36.300) Discussion on MRO for MR-DC SCG failure

Document for:		Discussion and Decision

1	Introduction

In RAN3#123bis meetig, we agreed to keep R18 agreement in R19 to support MRO for SCG failure in EN-DC, NGEN-DC and NE-DC scenarios, and an LS was sent to RAN2 on the feasibility to support MRO for SCG failure in R19.

RAN2#126 meeting discussed the LS from RAN3, and agreed that

Reply to RAN3 that we will only do EN-DC. RAN2 understands that whether also supporting (NG)EN-DC has no additional RAN2 impact hence RAN3 can decide. If later we get time we can consider other options.

Based on RAN2’s LS for reply [1], in this paper, we would further discuss the details of MRO for MR-DC SCG failure.

2	Discussion

Based on RAN2’s agreements, we would focus on MRO for EN-DC scenario at this stage. If later RAN2 decides to consider other MR-DC scenarios, we can further consider the others from RAN3 point of view.

In R17, stage 2 descriptions for SCG failure in NR-DC are captured, we can find that the existing stage 2 descriptions of PSCell change failure in TS37.340 can be taken as baseline for EN-DC SCG failure, and if needed necessary updates can be added on top of it.

Proposal 1: The existing stage 2 descriptions of PSCell change failure in TS37.340 can be taken as baseline for EN-DC SCG failure, and if needed necessary updates can be added on top of it.

On the other hand, stage 2 descriptions of PSCell change failure in EN-DC are needed, descriptions should be introduced in TS36.300. The draft TP is attached in the Annex.

Proposal 2: Introduce stage 2 descriptions of PSCell change failure in EN-DC scenario in TS36.300.

When SCG failure occurs in EN-DC, SCGFailureInformationNR message may be used to report SCG failure related information. Currently, only failure type and measurement results are included in the SCGFailureInformationNR message in TS36.331. Similar as enhancements for SCGFailureInformation message for NR-DC SCG failure in R17, it is beneficial to enhance SCGFailureInformationNR message for MRO, e.g. to include previousPSCellId, failedPSCellId, timeSCGFailure and RA info. If RAN3 agrees these parameters, an LS to RAN2 is needed for confirmation.

Proposal 3: Enhance SCGFailureInformationNR message for EN-DC SCG failure, e.g. to include previousPSCellId, failedPSCellId, timeSCGFailure and RA info.

To transfer SCG failure related information between MN and SN for root cause analysis for EN-DC SCG failure, similar as procedures defined in R17 for NR-DC SCG failure, new X2 messages may be needed, e.g. introduce SCG FAILURE INFORMATION REPORT and SCG FAILURE TRANSFER over X2 interface.

Proposal 4: Enhance X2 interface to transfer SCG failure related information between MN and SN, e.g. introduce SCG FAILURE INFORMATION REPORT and SCG FAILURE TRANSFER over X2.

For EN-DC scenario, since the SCG failure information reported by the UE is always encoded in the format of the MN RAT, but the SN is in the different RAT from the MN, if the MN just forwards the SCG failure information from the UE to the SN, it is impossible for the SN to decode it. So, how to forward SCG failure information from MN to SN over X2 needs to be considered.

To solve the issue, potential solutions are summarized as below:

- Option 1: MN decodes SCG failure information encoded in the MN RAT, re-organizes the information with the SN RAT format, then introduce a new inter-node message to forward the SCG failure information from MN to SN. In Option 1, an LS to RAN2 is needed to consider the introduction of the new inter-node RRC message.

- Option 2: MN decodes SCG failure information encoded in the MN RAT, re-organizes the information with the SN RAT format, then reuse CG-ConfigInfo inter-node message to send the SCG failure information from MN to SN. In Option 2, we need to send an LS to ask RAN2 to enhance CG-ConfigInfo inter-node message.

- Option 3: MN decodes SCG failure information encoded in the MN RAT, and put the explicit IEs in X2 or Xn message.

Compared with Option 1, Option 2 is slightly preferred since it has fewer spec impacts, and it is RAN2 to enhance CG-ConfigInfo inter-node message. In Option 3, X2 interface needs to be enhanced to transfer explicit information, but compared with Option 2, the implementation at SN side is easier since it does not need to decode the inter-node message.

Proposal 5: RAN3 to discuss whether to use inter-node message or use explicit IEs over X2 to forward SCG failure information from MN to SN.

3	Conclusion

In this paper, MRO for MR-DC SCG failure are discussed. We have the following proposals:

Proposal 1: The existing stage 2 descriptions of PSCell change failure in TS37.340 can be taken as baseline for EN-DC SCG failure, and if needed necessary updates can be added on top of it.

Proposal 2: Introduce stage 2 descriptions of PSCell change failure in EN-DC scenario in TS36.300.

Proposal 3: Enhance SCGFailureInformationNR message for EN-DC SCG failure, e.g. to include previousPSCellId, failedPSCellId, timeSCGFailure and RA info.

Proposal 4: Enhance X2 interface to transfer SCG failure related information between MN and SN, e.g. introduce SCG FAILURE INFORMATION REPORT and SCG FAILURE TRANSFER over X2.

Proposal 5: RAN3 to discuss whether to use inter-node message or use explicit IEs over X2 to forward SCG failure information from MN to SN.

Reference

[1] R3-244011, Reply LS on support of MRO for MR-DC SCG failure, ZTE

Annex: TP for TS36.300

#### 22.4.2.x	PSCell change failure

For analysis of PSCell change failures, the UE makes the SCG failure information available to the MN. If the MN can perform an initial analysis, it transfers the SCG failure information together with the analysis results to the relevant SN which is responsible for the PSCell change failures. Otherwise, the MN transfers the SCG failure information to the last serving SN, which may respond using the SCG Failure Transfer procedure to inform the MN it is not responsible for the SCG failure. If needed, the MN transfer the SCG failure information to the source SN.