3GPP TSG-RAN WG3 Meeting #125	R3-244310

Maastricht, NL, 19 - 23 August 2024

Agenda item:	 11.3

Source:		Lenovo

Title:		Discussion on AI/ML based CCO

Document for:		Discussion and Decision

1	Introduction

RAN3#124 meeting agreed that:

For non-split architecture, future CCO state can be derived via legacy mechanism or AI/ML techniques which is up to implementation.

For split architecture, the standardization of AI/ML based future CCO state inference in the gNB-DU is not supported in R19, how to derive future CCO state is up to implementation in the gNB-DU, e.g., legacy or AI/ML.

In this contribution, we would further discuss details of AI/ML based CCO.

2	Discussion

  - 2.1 AI/ML Model Training and Model Inference for CCO

RAN3#123bis meeting has agreed that for AI/ML based CCO [1],

-	AI/ML Model Training may be located in the OAM and AI/ML Model Inference may be located in the NG-RAN node (gNB-CU).

-	AI/ML Model Training and AI/ML Model Inference may be both located in the NG-RAN node (gNB-CU).

Currently, to support AI/ML in NG-RAN, inputs from neighbour NG-RAN nodes (e.g. predicted information, feedback information, measurements) and/or UEs (e.g. measurement results) are required. For AI/ML based CCO, for AI/ML Model Training or AI/ML Model inference, the inference input may be collected from UE or neighbour NG-RAN node (in case of non-split architecture) or neighbour gNB-CU/gNB-DU (in case of split architecture):

Inputs from UE:

-	UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

-	UE location information (e.g., coordinates, serving cell ID, moving velocity)

-	UE Mobility History Information

-	CEF/RA/RLF report from the UE

Inputs from local node:

-	Measured/Predicted Radio Resource Status

-	Measured/Predicted UE Traffic (e.g., data volume)

-	Measured/Predicted UE trajectory

Inputs from neighboring node:

-	Measured/Predicted Radio Resource Status

-	Measured/Predicted UE Traffic (e.g., data volume)

-	Measured/Predicted UE trajectory

Proposal 1: For AI/ML based CCO, the inference input may be collected from UE or local node or neighbour node, e.g. including:

- Inputs from UE

  - UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

  - UE location information (e.g., coordinates, serving cell ID, moving velocity)

  - UE Mobility History Information

  - CEF/RA/RLF report from the UE

- Inputs from local node

  - Measured/Predicted Radio Resource Status

  - Measured/Predicted UE Traffic (e.g., data volume)

  - Measured/Predicted UE trajectory

- Inputs from neighbour node

  - Measured/Predicted Radio Resource Status

  - Measured/Predicted UE Traffic (e.g., data volume)

  - Measured/Predicted UE trajectory

In non-split architecture, it has agreed that:

-	Step 0: gNB predicts the CCO issue.

-	Step 1: gNB generates the future coverage status based on the predicted CCO issue and other information.

-	Step 2: gNB sends the future coverage status to neighbour gNBs.

And in split-architecture, it has agreed that:

-	Step 0: gNB-CU predicts the CCO issue.

-	Step 1: gNB-CU sends the predicted CCO issue to gNB-DU.

-	Step 2: gNB-DU generates the future coverage status based on the predicted CCO issue and other local information, whether only local information is used can be further discussed.

-	Step 3: gNB-DU sends the future coverage status to gNB-CU.

-	Step 4: gNB-CU sends the future coverage status to neighbour gNBs.

Refer to legacy CCO mechanism, we may find that the predicted CCO issue may be a coverage issue or a cell edge capacity issue of a certain predicted cell or a certain predicted SSB. Additionally, the future CCO state may be Cell Coverage Modification(s) or SSB Coverage Modification(s), e.g. the Cell Coverage Modification which is used to solve the predicted CCO issue of the certain predicted cell including Cell Coverage State indicating the cell is inactive/active or the coverage configuration of the concerned cell, and the SSB Coverage Modification which is used to solve the predicted CCO issue of the certain predicted SSB including SSB Coverage State indicating the SSB beam is inactive/active or the coverage configuration of the concerned SSB beam.

Proposal 2: For AI/ML based CCO, the predicted CCO issue may include:

-	type of the predicted CCO issue(s), e.g., coverage issue or cell edge capacity issue;

-	id of affected cell(s) in which there is the predicted CCO issue(s);

-	SSB index(s) of affected beam(s) in which there is the predicted CCO issue(s).

Proposal 3: For AI/ML based CCO, the future CCO state may include:

-	Cell Coverage Modification including Cell Coverage State which is used to solve the predicted CCO issue of the certain predicted cell;

-	SSB Coverage Modification including SSB Coverage State which is used to solve the predicted CCO issue of the certain predicted SSB.

One open issue left from last RAN3 meeting is how to support the future CCO state and/or predicted CCO issue transmission over Xn. The potential solutions are:

- Alt 1: NG-RAN node 1 may send the future CCO state to NG-RAN node 2 and adjust the CCO state based on feedback from NG-RAN node 2.

- Alt 2: NG-RAN node 1 is fully responsible for prediction and only sends the future CCO state together with related time information to NG-RAN node 2.

In Alt 1, RAN node 1 transfers future CCO state to RAN node 2 in order to get RAN node 2’s confirmation before executing the future scheduled CCO state, or

In Alt 2, RAN node 1 will simply inform RAN node 2 about future CCO state to be executed by the RAN node 1 and RAN node 2’s confirmation is not needed. In our understanding, the second alternative is the most straightforward way. In the legacy, when a RAN node decides to modify the CCO state, it does not require any confirmation/handshake with neighbour RAN nodes. The same principle can be applied to the execution of a future CCO state as well. Besides, if RAN node 1 has multiple neighbour RAN nodes, it will be complicated/problematic if only some of the neighbour RAN nodes confirms while other neighbour RAN nodes rejects.

Besides the future CCO state, to better enable RAN node 2 understand the reason for the future cell/SSB coverage modification in RAN node 1, the predicted COO issue should also be transferred over Xn interface. With respect to whether the future CCO state and/or predicted CCO issue should be transferred over Xn interface based on Rel18 Data Collection Report procedure or based on conventional RAN Node Configuration Update procedure, considering the predicted COO issue or generation of future CCO state is rather an event-triggered strategy than a periodically change, RAN3 can consider either

- Enhance the Rel18 Data Collection Report procedure to support predicted COO issue and/or future CCO state transfer based on subscription/notify, or

- Enhance the conventional RAN Node Configuration Update procedure to support predicted COO issue and/or future CCO state transfer.

Proposal 4: NG-RAN node can predict and execute a future CCO state without handshaking with neighbour NG-RAN nodes.

Proposal 5: NG-RAN node can inform a neighbour NG-RAN node about a future CCO state to be executed and/or predicted CCO issue over Xn interface via either:

- Enhance the Rel18 Data Collection Report procedure based on subscription/notify, or

- Enhance the conventional RAN Node Configuration Update procedure.

Another open point left is the time information related to the future CCO state or CCO issue.

Time information to be further discussed in next meeting.

From the definition point of view, the time information would indicate the future CCO state or CCO issue is predicted to happen at what time point in the future.

Proposal 6: NG-RAN node also transfers time information to neighbour NG-RAN node indicating the future CCO state and/or CCO issue is predicted to happen at what time in the future. Detailed format is left to normative phase discussion.

Since the future CCO state is a prediction or based on a predicted CCO issue, the prediction might be updated or might not be correct. For example, NG-RAN node may predict a future CCO state or CCO issue for 10 seconds later and transfer the related information to a neighbour NG-RAN node, such that the neighbour NG-RAN node would expect a CCO state change after 10 seconds. However, the NG-RAN node may later detects/predicts the CCO issue to happen at 5 seconds sooner than previously predicted. In such case, it is important for the neighbour NG-RAN node to understand the newly detected or predicted CCO issue with corresponding CCO state and the neighbour NG-RAN node should consider the previously received future CCO state and/or predicted CCO issue to be no longer valid.

An example is depicted below to illustrate the point, wherein the NG-RAN node 1 first informs NG-RAN node 2 about a predicted CCO state/issue after time T, but then trigger a CCO state modification before time T due to detected CCO issue. The NG-RAN node 2 should consider the previously received CCO state/issue prediction no longer valid. Detailed solutions can be discussed in normative phase.

Proposal 7: RAN3 supports a NG-RAN node to update a previously transferred future CCO state and/or CCO issue (e.g., due to newly detected/predicted CCO issue). Details are upon normative phase discussion.

  - 2.2 performance feedback for CCO

For AI/ML based CCO in non-split architecture,

-	if AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the serving NG-RAN node, the serving NG-RAN may send the performance feedback to the OAM if applicable.

-	if AI/ML Model Training and AI/ML Model Inference are both located in the serving NG-RAN node, the neighbour NG-RAN may send the performance feedback to the serving NG-RAN if applicable.

For AI/ML based CCO in split architecture,

-	if AI/ML Model Training is located in the OAM, and AI/ML Model Inference is located in the serving gNB-CU, the serving gNB-CU/gNB-DU may send the performance feedback to the OAM if applicable.

-	if AI/ML Model Training is located in the serving gNB-CU, and AI/ML Model Inference is located in the serving gNB-CU, the neighbour gNB-CU or the serving gNB-DU may send the performance feedback to the serving gNB-CU if applicable.

The performance feedback may include at least one of following:

-	UE performance (e.g., throughput, delay, packet loss rate after Cell Coverage Modification or resource reallocation or after handover)

-	Radio Resource Status (of serving and neighbour node)

Based on the received performance feedback, the receiver may perform re-training to update the AI/ML model, and then deploys/updates AI/ML model.

Proposal 8: In non-split architecture,

-	if AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the serving NG-RAN node, the serving NG-RAN may send the performance feedback to the OAM if applicable.

-	if AI/ML Model Training and AI/ML Model Inference are both located in the serving NG-RAN node, the neighbour NG-RAN may send the performance feedback to the serving NG-RAN if applicable.

Proposal 9: In split architecture,

-	if AI/ML Model Training is located in the OAM, and AI/ML Model Inference is located in the serving gNB-CU, the serving gNB-CU/gNB-DU may send the performance feedback to the OAM if applicable.

-	if AI/ML Model Training is located in the serving gNB-CU, and AI/ML Model Inference is located in the serving gNB-CU, the neighbour gNB-CU or the serving gNB-DU may send the performance feedback to the serving gNB-CU if applicable.

Proposal 10: Performance feedback for AI/ML based CCO may include at least one of following:

-	UE performance (e.g., throughput, delay, packet loss rate after Cell Coverage Modification or resource reallocation or after handover)

-	Radio Resource Status (of serving and neighbour node)

3	Conclusion

In this contribution, AI/ML based CCO are discussed. We have the following proposals:

Proposal 1: For AI/ML based CCO, the inference input may be collected from UE or local node or neighbour node, e.g. including:

- Inputs from UE

  - UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

  - UE location information (e.g., coordinates, serving cell ID, moving velocity)

  - UE Mobility History Information

  - CEF/RA/RLF report from the UE

- Inputs from local node

  - Measured/Predicted Radio Resource Status

  - Measured/Predicted UE Traffic (e.g., data volume)

  - Measured/Predicted UE trajectory

- Inputs from neighbour node

  - Measured/Predicted Radio Resource Status

  - Measured/Predicted UE Traffic (e.g., data volume)

  - Measured/Predicted UE trajectory

Proposal 2: For AI/ML based CCO, the predicted CCO issue may include:

-	type of the predicted CCO issue(s), e.g., coverage issue or cell edge capacity issue;

-	id of affected cell(s) in which there is the predicted CCO issue(s);

-	SSB index(s) of affected beam(s) in which there is the predicted CCO issue(s).

Proposal 3: For AI/ML based CCO, the future CCO state may include:

-	Cell Coverage Modification including Cell Coverage State which is used to solve the predicted CCO issue of the certain predicted cell;

-	SSB Coverage Modification including SSB Coverage State which is used to solve the predicted CCO issue of the certain predicted SSB.

Proposal 4: NG-RAN node can predict and execute a future CCO state without handshaking with neighbour NG-RAN nodes.

Proposal 5: NG-RAN node can inform a neighbour NG-RAN node about a future CCO state to be executed and/or predicted CCO issue over Xn interface via either:

- Enhance the Rel18 Data Collection Report procedure based on subscription/notify, or

- Enhance the conventional RAN Node Configuration Update procedure.

Proposal 6: NG-RAN node also transfers time information to neighbour NG-RAN node indicating the future CCO state and/or CCO issue is predicted to happen at what time in the future. Detailed format is left to normative phase discussion.

Proposal 7: RAN3 supports a NG-RAN node to update a previously transferred future CCO state and/or CCO issue (e.g., due to newly detected/predicted CCO issue). Details are upon normative phase discussion.

Proposal 8: In non-split architecture,

-	if AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the serving NG-RAN node, the serving NG-RAN may send the performance feedback to the OAM if applicable.

-	if AI/ML Model Training and AI/ML Model Inference are both located in the serving NG-RAN node, the neighbour NG-RAN may send the performance feedback to the serving NG-RAN if applicable.

Proposal 9: In split architecture,

-	if AI/ML Model Training is located in the OAM, and AI/ML Model Inference is located in the serving gNB-CU, the serving gNB-CU/gNB-DU may send the performance feedback to the OAM if applicable.

-	if AI/ML Model Training is located in the serving gNB-CU, and AI/ML Model Inference is located in the serving gNB-CU, the neighbour gNB-CU or the serving gNB-DU may send the performance feedback to the serving gNB-CU if applicable.

Proposal 10: Performance feedback for AI/ML based CCO may include at least one of following:

-	UE performance (e.g., throughput, delay, packet loss rate after Cell Coverage Modification or resource reallocation or after handover)

-	Radio Resource Status (of serving and neighbour node)

Reference

[1] R3-242231, (TP to TR 38.743) AI/ML enabled CCO, Samsung, Ericsson, Huawei, Telecom Italia, ZTE, Nokia, CATT, Deutsche Telekom, LG Electronics, Telecom Italia