3GPP TSG-RAN WG3 #125	R3-244315

Maastricht, NL, 19th – 23rd Aug 2024

Agenda Item:	11.4

Source:	Lenovo

Title: 		TP to 38.743 for other leftover issues

Document for:	Discussion and Approval

# TP to TR 38.743

***************************** CHANGE STARTS *****************************

## 5.1	Mobility optimization for NR-DC

### 5.1.1 Use case description

Mobility Optimization for NR-DC is studied by assuming inference at the MN only. The main use case is limited to Dual Connectivity only (e.g., no conditional Dual Connectivity procedures are in scope).

### 5.1.2	Solutions and standard impacts

#### 5.1.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions can be considered for supporting AI/ML-based NR-DC operation:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the MN.

-	AI/ML Model Training and AI/ML Model Inference are both located in the MN.

In case of CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the CU of MN.

-	AI/ML Model Training and Model Inference are both located in the CU of MN.

Note: MN is also allowed to continue model training based on AI/ML model trained in the OAM.

#### 5.1.2.2	AI/ML Model Training in OAM and AI/ML Model Inference in a NG-RAN node

A high-level signalling flow for the AI/ML use case related to NR-DC operation with Model Training in OAM and Model Inference in NG-RAN is shown below, wherein the NG-RAN node 1 is MN and the NG-RAN node 2 is the target SN in the MN initiated SN addition/change scenario.

Figure 5.1.1-1. Model Training at OAM, Model Inference at NG-RAN

Step 0:  NG-RAN node 2 is assumed to have an AI/ML model optionally, which can provide NG-RAN node 1 with useful input information, such as predicted resource status, etc.

Step 1: The NG-RAN node 1 configures the UE to provide measurements and/or location information (e.g., RRM measurements, MDT measurements, velocity, position).

Step 2: The UE collects the indicated measurement(s), e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3: The UE reports to NG-RAN node 1 requested measurements and/or location information (e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells, velocity, position).

Step 4: NG-RAN node 1 further sends UE measurement reports together with other input data for Model Training to OAM. NG-RAN node 2 also sends input data for Model Training to OAM.

Step 5: AI/ML Model Training is located at OAM. The required measurements and input data from other NG-RAN nodes are leveraged to train the AI/ML model.

Step 6: OAM deploys/updates AI/ML model into the NG-RAN node(s). The NG-RAN node is allowed to continue model training based on the received AI/ML model from OAM.

Note: This step is out of RAN3 Rel-19 scope.

Step 7: The UE collects and reports to NG-RAN node 1 requested measurements or location information.

Step 8: The NG-RAN node 1 receives from the neighbouring NG-RAN node 2 the input information for NR-DC operation model inference.

Step 9: NG-RAN node 1 performs model inference and generate NR-DC operation related predictions or decisions.

Step 10. The NG-RAN 1 sends the model performance feedback to OAM if applicable.

Note: This step is out of RAN3 scope.

Step 11: NG-RAN node 1 may take NR-DC operation actions and the UE is moved from NG-RAN node 1 to NG-RAN node 2.

Step12: NG-RAN node 1 and NG-RAN node 2 send feedback information to OAM.

#### 5.1.2.3	AI/ML Model Training and AI/ML Model Inference in a NG-RAN node

A high-level signalling flow for the AI/ML use case related to NR-DC operation with Model Training and Model Inference in a NG-RAN node is shown below, wherein the NG-RAN node 1 is MN and the NG-RAN node 2 is the target SN in the MN initiated SN addition/change scenario.

Figure 5.1.1-2. Model Training and Model Inference in a NG-RAN node

Step 0: NG-RAN node 2 is assumed to have an AI/ML model optionally, which can provide NG-RAN node 1 with useful input information, such as predicted resource status, etc.

Step 1: The NG-RAN node 1 configures UE to provide measurements and/or location information(e.g., RRM measurements, MDT measurements, velocity, position).

Step 2: The UE collects the indicated measurement(s), e.g., UE measurements related to RSRP, RSRQ, SINR of the serving cell and neighbouring cells.

Step 3: The UE reports to NG-RAN node 1 the requested measurements and/or location information (e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells, velocity, position).

Step 4: The NG-RAN node 1 receives from the neighbouring NG-RAN node 2 the input information for NR-DC operation model training.

Step 5: An AI/ML Model Training is located at NG-RAN node 1. The required measurements and input data from other NG-RAN nodes are leveraged to train the AI/ML model.

Step6: NG-RAN node 1 receives UE measurements and/or location information.

Step7: NG-RAN node 1 can receive from the neighbouring NG-RAN node 2 the input information for NR-DC operation model inference.

Step 8: NG-RAN node 1 performs model inference and generate NR-DC related predictions or decisions.

Step 9: NG-RAN node 1 may take NR-DC operation actions and the UE is moved from NG-RAN node 1 to NG-RAN node 2.

Step 10: NG-RAN node 2 sends feedback information to NG-RAN node 1 (e.g., UE performance after SN addition or SN change, etc).

#### 5.1.2.4	Input of AI/ML-based NR-DC operation

To predict the optimized NR-DC operation decisions, MN may need following information as input data for AI/ML-based NR-DC operation:

From the serving MN node:

-	UE trajectory prediction (at cell level)

-	Measured/predicted resource status

-	Measured/predicted UE Traffic (e.g., data volume)

-	Measured/Predicted Energy Cost

From the UE:

-	UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

-	UE Mobility History Information.

From the associated SN nodes (can be source SN or target SN):

-	UE’s history information from neighbour

-	Measured/predicted resource status

-	Measured Energy Cost

#### 5.1.2.5	Output of AI/ML-based NR-DC operation

AI/ML-based NR-DC operation model in MN can generate following information as output:

-	UE trajectory prediction (at cell level)

-	UE traffic load prediction

-	SN addition/change recommendation, including recommended candidate PSCells to add/change

-	Traffic steering recommendation, including steering the UE traffic between MN and SN

#### 5.1.2.6	Feedback of AI/ML-based NR-DC operation

To optimize the performance of AI/ML-based NR-DC operation model in MN, following feedback can be considered to be collected from the target SN:

-	UE performance (e.g., throughput, delay, packet loss rate after resource reallocation or after SN addition/change)

-	UE traffic load

-	Resource status information updates from.

#### 5.1.2.7	Standard impact

For Xn interface, considering the node level resource status prediction has already been supported in Rel18 Data Collection Initiation/Report procedures, we see some possible impact on DC specific XnAP signaling. For example, the existing SN Addition or SN Change procedure may be enhanced to support:

- Sending the predicted UE traffic load and UE trajectory from MN to the target SN

- MN requesting the target SN to feedback the actual UE traffic load, UE trajectory and UE performance after the SN addition/change.

For E1 interface, RAN3 may introduce new procedure to support the exchange of the following between CU-CP and CU-UP:

- Measured/Predicted UE Traffic Load from CU-UP to CU-CP

- UE performance related measurements from CU-UP to CU-CP

For F1 interface, RAN3 may introduce new procedure to support the exchange of the following between CU and DU:

- UE performance related measurements from DU to CU

## 5.2	Split architecture support for Rel-18 use cases

### 5.2.1 Use case description

The split architecture should be enhanced to support the Rel-18 use cases, e.g, Load Balancing, Energy Saving, and Mobility Optimization.

In case of CU-DU architecture, the following solutions are possible:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU(-CP);

- AI/ML Model Training and Model Inference are both located in the gNB-CU(-CP).

### 5.2.2 Potential Standard impacts

The following standard impacts are listed for subsequent Rel-19 normative work compared with what was specified during Rel-18.

E1 interface:

- Measured UE performance from gNB-CU-UP to gNB-CU-CP.

F1 interface:

-	Measured Energy Cost from gNB-DU to gNB-CU.

-	Predicted Resource Status per Cell (e.g., predicted radio resource status, number of UEs) from gNB-CU to gNB-DU

-	Predicted UE trajectory from gNB-CU to gNB-DU

***************************** CHANGE ENDS *****************************