3GPP TSG-RAN WG3 Meeting #125	R3-244316

Maastricht, Netherlands, 19th – 23th August, 2024

Agenda Item:	12.2

Source: 	Lenovo

Title: 	Discussion on architecture and QoS mapping for WAB

Document for:	Discussion and decision

- Introduction

A new SID for additional topological enhancements [1] has been approved to be discussed in Rel-19, and following detail objectives are identified for Wireless Access Backhaul (WAB) fucus on RAN3 and RAN2.

The objectives of the Wireless Access Backhaul (WAB) study are as follows: - Study the support of WAB including [RAN3, RAN2]: - Study the architecture and protocol stack of supporting a gNB with MT function providing PDU session backhaul. - Study impact of WAB mobility within an existing RAN (e.g., inter-gNB neighbour relations). - Identify necessary inter-gNB- and gNB-to-CN signalling to address the support of WAB. - Study signalling enhancements on resource multiplexing for WAB. NOTE 1: No impact on the UE. NOTE 2: Coordination with other WGs (e.g. SA2) when needed. The WAB study does not preclude any backhaul scenario (e.g. NTN or TN).

In this contribution, the architecture and QoS mapping for WAB are discussed.

- Discussion

Xn interface in WAB architecture

Different to IAB-node, WAB-node has the gNB functionality and the WAB-gNB may need to establish Xn connection for the following scenarios.

Scenario 1: Xn connection between WAB-gNBs of two WAB-nodes

Scenario 2: Xn connection between a WAB-gNB and its BH RAN node

Scenario 3: Xn connection between a WAB-gNB and a surrounding RAN node

Currently, in the WAB-node integration procedure, the WAB-gNB may establish Xn connection(s) towards the BH-RAN-node and/or other surrounding NG-RAN node(s), if needed.

For the Xn connection between the WAB-gNB and its BH RAN node, the WAB-gNB can trigger the Xn connection setup to the BH RAN node after WAB-gNB setup procedure. And it’s not optimal for the BH RAN node to trigger the Xn setup procedure because the BH RAN node is transparent for the gNB setup of the WAB-node.

Proposal 1: For Xn connection between WAB-gNB and its BH RAN node, it's the WAB-gNB to trigger the Xn setup procedure.

For the Xn connection between the WAB-gNB and the surrounding RAN node, both the WAB-gNB and the surrounding RAN node need to discovery each other before Xn connection setup. As legacy, a gNB can find another gNB for Xn connection via measurement report or OAM configuration. However, legacy procedure may be not timely enough due to the mobility of the WAB-node. Alternatively, the WAB-gNB and the surrounding RAN node can discovery each other with the help of the BH RAN node, and each of the two nodes can trigger Xn connection setup procedure.

Proposal 2: For Xn connection between WAB-gNB and surrounding RAN node, both the WAB-gNB and the surrounding RAN node can trigger the Xn setup procedure.

While whether to implement Xn connection between two WAB-nodes needs to be confirmed. The main motivation to setup Xn connection for a WAB-node is to accelerate the Xn based handover of the served UEs of the WAB-node. However, due to mobility of the WAB-node, it’s unexpected to perform UE handover between two mobile WAB-nodes. Therefore, Xn connection between two WAB-nodes needs to be avoided.

Proposal 3: Xn connection between gNB part of two WAB-nodes needs to be avoided.

LS from SA2

SA2 would like to inform RAN3 that SA2 had progressed the Rel-19 FS_VMR_Ph2 study and documented the conclusions in clause 8 in TR 23.700-06. Issue-1: SA2 has discussed the access control (to restrict MWAB-UE accessing MWAB-gNB cell) for below two cases: a) MWAB-UE accessing any MWAB-gNB in IDLE and CONNECTED mode(documented as one candidate solution in clause 6.2.3.6 of TS 23.700-06); and b) MWAB-UE accessing MWAB-gNB belonging to same MWAB. SA2 assume that those scenarios will be discussed by RAN WGs, and ask for RAN feedback. Issue-2: SA2 has discussed and concluded that Additional ULI format will be determined by RAN WG and SA2 will align based on RAN feedback. Please refer to related conclusion in clause 8.5 of TR 23.700-06.

On issue 1, RAN3 had already agreed that a WAB-gNB cannot serve a WAB-MT, both for cases a) and b). As for the mechanism to avoid a WAB-MT accessing a WAB-gNB, the simplest way to prevent WAB-MT access can be based on a barring indication or a WAB-cell indication broadcasted by the WAB-gNB. Or the network side can reject the access of WAB-MT based on the explicit or implicit indication of WAB-specific indication for the WAB-MT.

Proposal 4: WAB-gNB broadcasts a barring indication or a WAB-cell indication in SIB1 to avoid its WAB-MT and other WAB-MT to camp on the cell of WAB-gNB.

Backhaul traffic mapping

After the WAB-MT setup with 5GC, at least one PDU session needs to be established for transporting the signalling related to NG setup. While for the NG-U traffic for the served UEs, all the NG-U traffic may be mapped to the same PDU session for the NG-C signaling. Alternatively, additional PDU sessions can be established to transport the NG-U and different PDU sessions are used to the NG-U traffic with different QoS requirements. In addition, different types of control plane signalling (e.g., non-UE associated signalling and UE associated signalling) may be mapped to different PDU sessions.

Proposal 5: NG-C signalling and NG-U traffic can be mapped to the same PDU session between WAB-MT and MT’s UPF, alternatively, different types of NG-C signalling and NG-U traffic with different QoS requirements can be mapped to different PDU sessions.

For WAB network, as shown in Figure 1, the UP traffic may travel multiple hops to/from UE’s UPF, and the PDB for UE includes RAN PDB-1 and CN PDB-1. In addition, the UP traffic needs to be backhauled via the BH PDU session of WAB-MT, and the PDB for WAB-MT includes RAN PDB-2 and CN PDB-2.

Figure 1: An example of PDB for WAB

And for PER of the UE served by WAB-node, the PER of UE needs to consider the packet error rate in both air interfaces between UE and WAB-gNB (e.g., PER-1) and between WAB-MT and BH-gNB (e.g., PER-2).

Figure 2: An example of PER for WAB

In case the BH CN setups BH PDU session and determines the QoS parameters of the backhaul link firstly, from the UE’s CN point of view, it needs to set suitable CN PDB-1 to WAB-gNB based on delay of BH link, and it needs to set suitable PER-1 based on the packet error rate in the BH link because the UE’s traffic needs to be transported via the backhaul PDU session. While if it’s the UE’s CN to determine partition of the E2E QoS for the UE, then the BH CN needs to modify the BH PDU session or setup new BH PDU session to accommodate the UE’s traffic based on UE’s QoS parameters (at least for PDB and PER).

Proposal 6: UE’s CN needs to be aware of the BH QoS parameters to set suitable QoS parameters for UE, or the BH CN needs to be aware of the UE’s QoS parameters to modify existing BH PDU session or setup new BH PDU session to accommodate the UE’s traffic.

# Conclusion

This contribution aims to discuss WAB architecture and QoS mapping. And following observations and proposals are concluded.

Proposal 1: For Xn connection between WAB-gNB and its BH RAN node, it's the WAB-gNB to trigger the Xn setup procedure.

Proposal 2: For Xn connection between WAB-gNB and surrounding RAN node, both the WAB-gNB and the surrounding RAN node can trigger the Xn setup procedure.

Proposal 3: Xn connection between gNB part of two WAB-nodes needs to be avoided.

Proposal 4: WAB-gNB broadcasts a barring indication or a WAB-cell indication in SIB1 to avoid its WAB-MT and other WAB-MT to camp on the cell of WAB-gNB.

Proposal 5: NG-C signalling and NG-U traffic can be mapped to the same PDU session between WAB-MT and MT’s UPF, alternatively, different types of NG-C signalling and NG-U traffic with different QoS requirements can be mapped to different PDU sessions.

Proposal 6: UE’s CN needs to be aware of the BH QoS parameters to set suitable QoS parameters for UE, or the BH CN needs to be aware of the UE’s QoS parameters to modify existing BH PDU session ors setup new BH PDU session to accommodate the UE’s traffic.

# Reference

- RP-234041. New SID: Study on additional topological enhancements for NR.

- R3-244019. LS on questions regarding FS_VMR_Ph2. SA2.