3GPP TSG-RAN WG3 Meeting #125	R3-244317

Maastricht, Netherlands, 19th – 23th August, 2024

Agenda Item:	12.2

Source: 	Lenovo

Title: 	(TP to TR 38.799) Discussion on WAB-gNB migration

Document for:	Discussion and decision

- Introduction

A new SID for additional topological enhancements [1] has been approved to be discussed in Rel-19, and following detail objectives are identified for Wireless Access Backhaul (WAB) fucus on RAN3 and RAN2.

The objectives of the Wireless Access Backhaul (WAB) study are as follows: - Study the support of WAB including [RAN3, RAN2]: - Study the architecture and protocol stack of supporting a gNB with MT function providing PDU session backhaul. - Study impact of WAB mobility within an existing RAN (e.g., inter-gNB neighbour relations). - Identify necessary inter-gNB- and gNB-to-CN signalling to address the support of WAB. - Study signalling enhancements on resource multiplexing for WAB. NOTE 1: No impact on the UE. NOTE 2: Coordination with other WGs (e.g. SA2) when needed. The WAB study does not preclude any backhaul scenario (e.g. NTN or TN).

In this contribution, the procedure for WAB-node migration is discussed, including WAB-MT and WAB-gNB migration.

- Discussion

In RAN3 #124 meeting, following agreements had been achieved for scenarios of WAB-node migration.

Support the following scenarios for intra-PLMN WAB-node mobility: - UE´s AMF/UPF remains unchanged as WAB-gNB moves inside a PLMN. - UE´s AMF/UPF changes as WAB-gNB moves inside a PLMN. - BH UPF/AMF remains unchanged as WAB-MT moves inside a PLMN. - BH UPF/AMF changes as WAB-MT moves inside a PLMN.

BH UPF/AMF remains unchanged as WAB-MT moves inside a PLMN

In case of WAB-MT migration without BH UPF/AMF changing, the WAB-MT performs handover from a source BH RAN node to a target BH RAN node, which both connect to the same BH AMF, and the WAB-MT anchors at the same BH UPF in the source and target path. A brief flow chart and migration procedure are as follow.

Figure 1: An example of WAB-MT migration procedure with BH 5GC unchanged

1.	The WAB-MT sends a MeasurementReport message to the source BH RAN node. This report is based on a Measurement Configuration the WAB-MT received from the source BH RAN node before.

2.  The source BH RAN node sends a HANDOVER REQUEST message to the target BH RAN node over the Xn interface.

3.	The target BH RAN node performs admission control and provides the new RRC configuration as part of the HANDOVER REQUEST ACKNOWLEDGE message to the source BH RAN node.

4.	The source BH RAN node forwards the received RRCReconfiguration message to the WAB-MT.

5.	A random access procedure is performed at the target BH RAN node.

6.	The WAB-MT responds to the target BH RAN node with an RRCReconfigurationComplete message.

7.	The target BH RAN node triggers the path switch procedure for migration of the WAB-MT, then all the PDU sessions of the WAB-MT are switched to the target path, and all NG and Xn traffic are transported via the target path from/to the BH UPF over the PDU sessions.

8.	The target BH RAN node sends UE CONTEXT RELEASE message to the source BH RAN node after WAB-node migration.

And a TP to TR 38.799 for WAB-MT migration procedure with BH 5GC unchanged is provided in the Appendix part.

Proposal 1: Agree the procedure above for WAB-MT migration with BH 5GC unchanged.

BH UPF/AMF changes as WAB-MT moves inside a PLMN

For this scenario, it may be further divided into follow sub-scenarios:

- BH AMF remains unchanged and BH UPF changes as WAB-MT moves inside a PLMN

- Both BH AMF and BH UPF change as WAB-MT move inside a PLMN

Because all the NG/Xn traffic of the WAB-gNB and UP traffic of served UEs need to be backhauled via the BH PDU session, when the BH UPF changing during WAB-MT migration, the WAB-MT needs to setup new PDU session with the target BH UPF and release the old PDU session with the old BH UPF. The detail procedure has main impact on the core network and can be captured in SA2.

Proposal 2: Support the following scenarios for WAB-node mobility:

- BH AMF remains unchanged and BH UPF changes as WAB-MT moves inside a PLMN

- Both BH AMF and BH UPF change as WAB-MT move inside a PLMN

As defined in the scope, WAB-gNB cannot serve WAB-MT, which means that a WAB-MT cannot be handed over to a target WAB-node. The WAB-MT migration scenario is the same to the mIAB-MT migration, where the mIAB-MT cannot connect to another mIAB-node during migration. In order to avoid handover failure of mIAB-MT, mobile IAB cell information is exchanged via XnAP. In the same way, WAB cell information can be also exchanged between source BH RAN node and the target BH RAN node to avoid WAB-MT migrating to a WAB-gNB.

Proposal 3: WAB cell information may be shared between source BH RAN node and target BH RAN node to avoid WAB-MT migrating to a WAB-gNB.

UE´s AMF/UPF remains unchanged as WAB-gNB moves inside a PLMN

As the UE’s AMF remains unchanged, the gNB ID for the served UEs of the WAB-node also remains unchanged. While for the cell ID and TAC, they can be also remains unchanged and no additional impacts on UE are introduced.

In some other case, the cell ID, PCI and TAC may be changed without the change of the UE’s AMF(s). For example, if the TAC changes, the WAB-gNB needs to inform its AMF with the new TAC, and the served UE may need to perform a Mobility Registration Update procedure.

Proposal 4: During WAB-gNB migration, the TAC can be changed without the change of UE’s AMF.

UE´s AMF/UPF changes as WAB-gNB moves inside a PLMN

This scenario is similar to mobile IAB-DU migration, where the mobile IAB-node supports two logical mobile IAB-DUs having F1 connections with the source F1-terminating IAB-donor-CU and target F1-terminating IAB-donor-CU, respectively. To realize WAB-gNB inter-AMF migration, the WAB-node needs to support two logical WAB-gNB concurrently during the migration. If the WAB-node only has one WAB-gNB, the NG connection will suffer interruption during the migration which have impacts on service continuity of the served UEs.

Proposal 5: During inter-AMF WAB-gNB migration, the WAB-node support two logical WAB-gNB concurrently, which have NG connections with the source AMF and target AMF, respectively.

For the UE(s) served by the WAB-node, they are handed over from the cells of the source logical WAB-gNB to the cells of the target logical WAB-gNB. And after all UEs are handed over, the NG connection between source logical WAB-gNB and source AMF may be removed. A brief flow chart and migration procedure are as follow.

Figure 2: An example of WAB-gNB migration procedure with UE´s AMF/UPF change

1.	The target logical WAB-gNB sends a NG SETUP REQUEST message to the target UE’s AMF in the UE’s 5GC.

2.  The target UE’s AMF responses to the target logical WAB-gNB with a NG SETUP RESPONSE message. After the NG is set up, the target logical WAB-gNB can start serving UE(s).

3.	The source logical WAB-gNB performs NG-based UE handover from from the cells of the source logical WAB-gNB to the cells of the target logical WAB-gNB.

4.	After all the UEs are handed over, the NG connection between the source logical WAB-gNB and the source UE’s AMF may be removed.

And a TP to TR 38.799 for WAB-gNB migration procedure with UE´s AMF/UPF change is provided in the Appendix part.

Proposal 6: Agree the procedure above for WAB-gNB migration with UE´s AMF/UPF change.

# Conclusion

This contribution aims to discuss WAB-node migration procedure. And following observations and proposals are concluded.

Proposal 1: Agree the procedure above for WAB-MT migration with BH 5GC unchanged.

Proposal 2: Support the following scenarios for WAB-node mobility:

- BH AMF remains unchanged and BH UPF changes as WAB-MT moves inside a PLMN

- Both BH AMF and BH UPF change as WAB-MT move inside a PLMN

Proposal 3: WAB cell information may be shared between source BH RAN node and target BH RAN node to avoid WAB-MT migrating to a WAB-gNB.

Proposal 4: During WAB-gNB migration, the TAC can be changed without the change of UE’s AMF.

Proposal 5: During inter-AMF WAB-gNB migration, the WAB-node support two logical WAB-gNB concurrently, which have NG connections with the source AMF and target AMF, respectively.

Proposal 6: Agree the procedure above for WAB-gNB migration with UE´s AMF/UPF change.

# Reference

- RP-234041. New SID: Study on additional topological enhancements for NR.

- 3GPP TR 23.700-06. Study on architecture enhancements for vehicle-mounted relays - Phase 2 (Release 19). V0.4.0.

# Appendix: TP to TR 38.799

### 4.3.4	Mobility handling

The following scenarios for WAB-node mobility are supported:

- The UE´s AMF/UPF remains unchanged as the WAB-gNB moves.

- The UE´s AMF/UPF changes as the WAB-gNB moves.

- The BH UPF/AMF remains unchanged as the WAB-MT moves inside a PLMN.

- The BH UPF/AMF changes as the WAB-MT moves inside a PLMN.

RAN3 assumes that the WAB-gNB does not change PLMN while the WAB-node moves across the network.

#### 4.3.4.1	WAB-MT mobility

The WAB-MT reuses legacy mobility handover procedures for the UE as it moves through the BH-RAN.

##### 4.3.4.1.1 The BH-UPF/BH-AMF remains unchanged as the WAB-MT moves inside a PLMN.

In case the WAB-MT’s PSA UPF does not change during these mobility procedures, the IP addresses allocated for the WAB-MT’s PDU sessions do not change. Therefore, the NG and Xn connections of the WAB-gNB carried over the BH PDU session(s) remain unaffected.

Figure 4.3.4.1.1-1 shows an example of the WAB-MT migration procedure with BH 5GC unchanged.

Figure 4.3.4.1.1-1: An example of WAB-MT migration procedure with BH 5GC unchanged

1.	The WAB-MT sends a MeasurementReport message to the source BH RAN node. This report is based on a Measurement Configuration the WAB-MT received from the source BH RAN node before.

2.  The source BH RAN node sends a HANDOVER REQUEST message to the target BH RAN node over the Xn interface.

3.	The target BH RAN node performs admission control and provides the new RRC configuration as part of the HANDOVER REQUEST ACKNOWLEDGE message to the source BH RAN node.

4.	The source BH RAN node forwards the received RRCReconfiguration message to the WAB-MT.

5.	A random access procedure is performed at the target BH RAN node.

6.	The WAB-MT responds to the target BH RAN node with an RRCReconfigurationComplete message.

7.	The target BH RAN node triggers the path switch procedure for migration of the WAB-MT, then all the PDU sessions of the WAB-MT are switched to the target path, and all NG and Xn traffic are transported via the target path from/to the BH UPF over the PDU sessions.

8.	The target BH RAN node sends UE CONTEXT RELEASE message to the source BH RAN node after WAB-node migration.

##### 4.3.4.1.2 BH-UPF/BH-AMF changes as the WAB-MT moves inside a PLMN.

In this scenario, the BH-AMF may remain unchanged when the BH-UPF changes as the WAB-MT moves. When the WAB-MT’s PSA UPF need to be changed due to WAB-MT’s mobility, new BH PDU session(s) need to be established reusing existing mechanisms defined in TS 23.501/TS 23.502. In this case, the (outer) IP addresses used by the WAB-gNB for the transport of NG and Xn connections of the WAB-gNB will change.

##### 4.3.4.1.3 Roaming of the WAB-MT

During WAB-node movement, the WAB-MT may connect to a PLMN different than its HPLMN.

#### 4.3.4.2	WAB-gNB mobility

During WAB-node movement, establishment, and removal of the WAB-gNB’s NG and/or Xn connections may be needed.

Establishment of Xn connections of the WAB-gNB with BH-RAN nodes, as well as with surrounding RAN nodes, is supported, and it can follow legacy procedures.

##### 4.3.4.2.1 WAB-gNB mobility without change of UE’s AMF(s)

During WAB-node movement, radio configuration parameters of the WAB-gNB may be changed (e.g., cell ID, PCI and/or TAC) without the change of the UE’s AMF(s). This change of radio configuration parameters may require the UE handling by means of, e.g., intra-gNB handover and/or Mobility Registration Update as defined in TS 23.502.

##### 4.3.4.2.2 WAB-gNB mobility with change of UE’s AMF(s)

Due to WAB-node movement, the change of UE’s AMF(s) may be needed, based on, e.g., WAB-node’s current location and/or additional criteria. The NG connection handling and WAB-gNB configuration update may affect the served UEs.

- The WAB-gNB may obtain the configuration parameters needed to establish the connection to the UE’s new AMF(s).

- The WAB-gNB establishes NG connection(s) towards one or more new AMF(s).

- The WAB-gNB may activate one or more new cells, with new cell configuration parameters related to the WAB-gNB’s current location. The new cells may broadcast the radio parameters configured for the new AMF(s), e.g., TAC, etc. The old cell(s) remain(s) active.

- The UEs are handled as follows:

- A UE in RRC_CONNECTED state is handed over between an old cell and a new cell served by the WAB-gNB via NG-based handover with AMF relocation, as defined in TS 23.502.

- When all UEs in RRC_CONNECTED state have been handed over, the old cell(s) are removed from service. A UE in RRC_IDLE or RRC_INACTIVE state camping on the old cell(s) reselects a new cell, and legacy procedure (e.g., Mobility Registration Update procedure as defined in TS 23.502) is performed.

- The NG connection(s) between the WAB-gNB and the old AMF(s) are removed.

Figure 4.3.4.2.2-1 shows an example of the WAB-gNB migration procedure with change of UE’s AMF.

Figure 4.3.4.2.2-1: An example of WAB-gNB migration procedure with UE´s AMF/UPF change

1.	The target logical WAB-gNB sends a NG SETUP REQUEST message to the target UE’s AMF in the UE’s 5GC.

2.  The target UE’s AMF responses to the target logical WAB-gNB with a NG SETUP RESPONSE message. After the NG is set up, the target logical WAB-gNB can start serving UE(s).

3.	The source logical WAB-gNB performs NG-based UE handover from from the cells of the source logical WAB-gNB to the cells of the target logical WAB-gNB.

4.	After all the UEs are handed over, the NG connection between the source logical WAB-gNB and the source UE’s AMF may be removed.