3GPP TSG-RAN WG3 Meeting #125	R3-244318

Maastricht, Netherlands, 19th – 23th August, 2024

Agenda Item:	12.2

Source: 	Lenovo

Title: 	(TP to TR 38.799) Discussion on resource multiplexing for WAB node

Document for:	Discussion and decision

- Introduction

A new SID for additional topological enhancements [1] has been approved to be discussed in Rel-19, and following detail objectives are identified for Wireless Access Backhaul (WAB) fucus on RAN3 and RAN2.

The objectives of the Wireless Access Backhaul (WAB) study are as follows: - Study the support of WAB including [RAN3, RAN2]: - Study the architecture and protocol stack of supporting a gNB with MT function providing PDU session backhaul. - Study impact of WAB mobility within an existing RAN (e.g., inter-gNB neighbour relations). - Identify necessary inter-gNB- and gNB-to-CN signalling to address the support of WAB. - Study signalling enhancements on resource multiplexing for WAB. NOTE 1: No impact on the UE. NOTE 2: Coordination with other WGs (e.g. SA2) when needed. The WAB study does not preclude any backhaul scenario (e.g. NTN or TN).

In this contribution, scheduling and resource allocation/coordination for WAB-node migration is discussed, including resource type and resource coordination between access link and backhaul link or between two parent WAB-donor.

- Discussion

For IAB, following resource types had been defined to realize resource coordination and collision avoidance between the access and backhaul link for an IAB-node [2].

From an IAB-node MT point-of-view, as in Rel. 15, the following time-domain resources can be indicated for the parent link: - Downlink time resource; - Uplink time resource; - Flexible time resource. From an IAB-node DU point-of-view, the child link has the following types of time resources: - Downlink time resource; - Uplink time resource; - Flexible time resource; - Not available time resources (resources not to be used for communication on the DU child links). Each of the downlink, uplink and flexible time-resource types of the DU child link can belong to one of two categories: - Hard: The corresponding time resource is always available for the DU child link; - Soft: The availability of the corresponding time resource for the DU child link is explicitly and/or implicitly controlled by the parent node.

WAB has similarities with IAB. The resource configuration for IAB is therefore a candidate for WAB. However, due to differences in the architectures and protocol stacks of WAB and IAB, it may not be ideal to fully reuse the H/S/NA framework specified for IAB.

One main difference between IAB and WAB is that the WAB-node includes a full gNB (WAB-gNB) rather than the IAB-DU in the case of IAB. As a result, the interface between the WAB-gNB and the BH RAN node is Xn instead of F1. The F1 interface is a configuring interface, and the F1AP provides various IEs for configuring the IAB-DU by the IAB-donor-CU. However, the Xn interface has a different function, essentially connecting two peer gNBs and exchange information between the two peer gNBs. It is hence not natural to leave all the resource configurations of the WAB-gNB to be configured by the BH RAN node via the XnAP procedure.

Another issue to consider is that according to the current IAB specifications, H and NA resources have a forcing behavior at the IAB node, but the behavior of the IAB node for S resources is not fully determined by the IAB-donor or the parent node. Instead, the RAN1 specification (TS 38.213, section 14) provides a list of conditions for using S resources by the IAB-DU that includes, but is not constrained by, signaling from the parent node.

Therefore, we propose to take the H/S/NA framework as a starting point with modifications that avoid a full tedious configuration of one gNB (WAB-gNB) by another gNB (BH RAN node). A natural approach is to configure the WAB-MT resources by RRC and use L1/L2 signaling for indicating usage/availability of the resources for the WAB-MT. Then, the WAB-gNB can decide about using its own resources based on the configuration and L1/L2 indication as well as its multiplexing capability.

Proposal 1: Resource configuration based on the H/S/NA framework specified for IAB can be used as a starting point for WAB resource configurations. The resource configuration can be handled by RRC more appropriately than by XnAP.

As defined in the SID [1], the WAB network in R19 only supports one-hop backhauling, where the WAB-node is directly connected to the BH RAN node. In this WAB topology, the downlink transmission from WAB-gNB to the served UEs is scheduled by the WAB-gNB itself, and the uplink transmission from WAB-MT to the BH RAN node is scheduled by the BH RAN node.

In a centralized resource allocation to avoid inter-cell or inter-node resource interference, the BH RAN node should provide some semi-static reserved DL/UL resources for each served WAB-nodes, and how to schedule on these semi-static resources is left to the implementation of WAB-node’s scheduling algorithm. However, there is no F1 interface between the BH RAN node and the WAB-node, the legacy F1AP message used for resource configuration on the IAB-node cannot be reused. Then the resource configuration may need to be delivered via the XnAP sigalling between the BH RAN node and the WAB-gNB.

According to the IAB specifications, interpreting the resources available to the IAB-DU depends on the D/U/F configuration at the IAB-DU because the L1 availability indication indicates availability of soft DL symbols, soft UL symbols, and soft flexible symbols in a slot instead of indicating availability of each symbol separately. That, in part, reduces the overhead of the L1 availability indication signaling. A similar principle can be adopted for the L1/L2 resource usage indication for WAB. As a result, the BH RAN node may require knowledge of the D/U/F configuration at the WAB-gNB. For this purpose, the WAB-gNB can use Xn signaling to inform the BH RAN node of its own D/U/F resource configurations.

Proposal 2: The WAB-gNB can signal its D/U/F configuration to the BH RAN node over Xn.

As for the resource configuration on DU of the BH RAN node, legacy F1AP GNB-DU RESOURCE CONFIGURATION can be reused, which includes the resource configuration (D/U/F/NA/H/S) on the gNB-DU of the BH RAN node, and it further includes the resource configuration and the multiplexing info of the WAB-gNB, which is helpful for the gNB-DU of the BH RAN node to avoid scheduling the WAB-MT in the slot/symbol with hard resources in the collocated WAB-gNB.

Proposal 3: Legacy F1AP GNB-DU RESOURCE CONFIGURATION can be reused for resource configuration on gNB-DU of the BH RAN node.

# Conclusion

This contribution aims to discuss enhancements for mobility of an IAB-node together with its served UEs. And following observations and proposals are concluded.

Proposal 1: Resource configuration based on the H/S/NA framework specified for IAB can be used as a starting point for WAB resource configurations. The resource configuration can be handled by RRC more appropriately than by XnAP.

Proposal 2: The WAB-gNB can signal its D/U/F configuration to the BH RAN node over Xn.

Proposal 3: Legacy F1AP GNB-DU RESOURCE CONFIGURATION can be reused for resource configuration on gNB-DU of the BH RAN node.

# Reference

- RP-234041. New SID: Study on additional topological enhancements for NR.

- 3GPP TR 38.874. Study on Integrated Access and Backhaul (Release 16). V16.0.0.

# Appendix: TP to TR 38.799

### 4.3.5 Resource multiplexing

In scenarios where WAB-node’s access link and backhaul link mutually interference, resource coordination may be needed to facilitate the resource multiplexing of the WAB-node’s access links and backhaul link. For this purpose, the resource coordination mechanism introduced for IAB can be considered as the starting point. For resource coordination between the access link and backhaul link, the BH-gNB may need to discover co-location of the WAB-MT and the WAB-gNB, and the WAB-gNB can signal its D/U/F configuration to the BH RAN node over Xn. In addition, in case of CU-DU split for the BH RAN node, legacy F1AP GNB-DU RESOURCE CONFIGURATION can be reused for resource configuration on gNB-DU of the BH RAN node.