3GPP TSG-RAN WG3 Meeting #125	R3-244321

Maastricht, Netherlands, 19th – 23th August, 2024

Agenda Item:	12.3

Source: 	Lenovo

Title: 	Local services for NR Femto via collocated local UPF

Document for:	Discussion and decision

- Introduction

A new SID for additional topological enhancements [1] has been approved to be discussed in Rel-19, and following detail objectives are identified for 5G Femto fucus on RAN3.

- Study the overall RAN architecture and required functional and procedural impacts for supporting 5G Femto deployments [RAN3]. - Study how to define the 5G access control mechanism by (re-)using the existing CAG functionality and identify needed enhancements (if any) [RAN3]. - Clarify the access to local services from the 5G Femto via collocated local UPF and identify issues, if any [RAN3]. NOTE 1: The study involves a gap analysis of existing 5G functionality with HomeNB functionality. NOTE 2: No impact on the UE. NOTE 3: Coordination with other WGs (e.g. SA2) when needed.

In this contribution, architecture for NR Femto is discussed based on the basis of 4G Home eNB.

- Discussion

Local services for NR Femto via collocated local UPF

For architecture Option 4, the NR Femto node is just a gNB-DU, if this NR Femto node has deployed a local UPF, the UP traffic needs to be transported to the CU-UP firstly and then back to the local UPF at the gNB-DU, which lead to more latency and make the architecture too complex. Therefore, local service for the NR Femto is only considered for architecture Options 1, 2 and 3.

Proposal 1: Local service for the NR Femto is only considered for architecture Options 1, 2 and 3.

Refer to SA2’s specification, local services via collocated local UPF have already been discussed and captured in Edge Computing as below [2]. And several connectivity models to enable Edge Computing are further introduced in [3].

## 5.13 Support for Edge Computing Edge computing enables operator and 3rd party services to be hosted close to the UE's access point of attachment, so as to achieve an efficient service delivery through the reduced end-to-end latency and load on the transport network. Edge Computing support by 5GC is specified in this specification and in TS 23.548 [130]. NOTE: Edge Computing typically applies to non-roaming and LBO roaming scenarios. The 5G Core Network selects a UPF close to the UE and forwards traffic to enable the local access to the DN via a N6 interface according to the provided traffic steering rules to the UPF. This may be based on the UE's subscription data, UE location, the information from Application Function (AF) as defined in clause 5.6.7, the EAS information reported from EASDF (as defined in TS 23.548 [130]), policy or other related traffic rules. Due to user or Application Function mobility, the service or session continuity may be required based on the requirements of the service or the 5G network. The 5G Core Network may expose network information and capabilities to an Edge Computing Application Function.

Proposal 2: The mechanisms introduced by Edge Computing for local services can be used as a starting point for 5G Femto.

# Conclusion

This contribution aims to discuss architecture for 5G Femto. And following observations and proposals are concluded.

Proposal 1: Local service for the NR Femto is only considered for architecture Options 1, 2 and 3.

Proposal 2: The mechanisms introduced by Edge Computing for local services can be used as a starting point for 5G Femto.

# Reference

- RP-234041. New SID: Study on additional topological enhancements for NR.

- 3GPP TS 23.501. System architecture for the 5G System (5GS); Stage 2 (Release 18). V18.5.0 (2024-03).

- 3GPP TS 23.548. 5G System Enhancements for Edge Computing; Stage 2 (Release 18). V18.5.0 (2024-03).