3GPP TSG-RAN WG3 Meeting #125	R3-244322

Maastricht, Netherlands, 19 - 23 August, 2024

Agenda item:	13.2

Source: 	Lenovo

Title: 	General aspects of inter-CU LTM

Document for:	Discussion and Approval

1. Introduction

The inter-CU LTM was discussed at the RAN3#124 meeting with the following agreements [1].

Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM. Introduce a new procedure on Xn to transfer the TA information. A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU. Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell. The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs. Early data forwarding can be triggered before the Source gNB triggers a MAC CE Command to the UE to change cells, timing is left to implementation. Early sync configuration (TCI state and RACH configuration) can be obtained during the LTM preparation phase through the handover request and handover request acknowledge messages. A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Details are FFS. FFS on the LTM modification procedures. FFS on whether to reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

In this contribution, we discuss the remaining issues of inter-CU LTM related to RAN3 impact.

2. Discussion

Part A: LTM preparation

At the RAN3#123bis meeting, the group agreed:

Reuse existing Xn Handover Request and Handover Request ACK for Inter-CU LTM initial preparation.

Although multiple candidate cells could be within the same gNB, different candidate cells may have different load situation and then different admission control results. Current handover preparation procedure uses the parallel requests for a handover or a conditional handover, which are identified by the target cell ID when the source UE XnAP IDs are the same. In addition, R18 intra-CU LTM also uses the parallel messages over F1AP. Therefore, the parallel procedures are used to prepare the LTM candidate cell configurations.

Proposal 1: Parallel procedures are used to prepare LTM candidate configurations, i.e., one message is for one LTM candidate configuration.

In legacy handover or conditional handover, the handover preparation procedure is used to establish necessary resources in the target gNB. The handover preparation procedure could also be used for inter-CU LTM. To minimize the specification impact, an indicator could be introduced in the Handover Request message to indicate the request concerning an LTM. If the candidate gNB accepts the request, it should provide the LTM candidate cell configuration for the requested candidate cell to the source gNB using Handover Request Acknowledge message.

Proposal 2: An indicator is introduced in the Handover Request message to request the candidate gNB to provide LTM candidate configuration per candidate cell.

Proposal 3: The candidate gNB may accept the target cell of LTM and respond to the source gNB with the LTM candidate configuration in Handover Request Acknowledge message.

The source gNB initially selects the potential candidate cells for LTM, e.g., based on the existing L3 measurement result from the UE. In some cases, the source gNB may decide to release an already prepared LTM candidate cell in the candidate gNB, e.g., the L3 measurement result of the LTM candidate cell becomes worse and it is not so good anymore. In other cases, the source gNB may decide to modify an already prepared LTM candidate cell in the candidate gNB, e.g., the source gNB may update the LTM reference configuration and provide the updates to the candidate gNB.

Proposal 4: The source gNB may modify or release already prepared LTM candidate cells in the candidate gNB.

At the RAN3#124 meeting, the group agreed:

A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Details are FFS.

To support the candidate gNB initiated cancellation, there are two possible options:

- Option 1: Define a new message for the candidate gNB initiated cancellation of configured LTM candidate cell(s), e.g., LTM Cancel message. The benefit is that the purpose of the message is dedicated for LTM. The drawback is that it brings more standard works, such as new procedure and new message.

- Option 2: Reuse the CHO Cancel message. The benefit is that we can reuse the current procedure and message as much as possible, hence the standard work is limited. The drawback is that it maybe a little bit confused about the name “CHO”.

From our point of view, the IEs for the LTM Cancel and the CHO Cancel are almost the same, i.e., Candidate Cells to Be Cancelled List with the Target Cell ID. Therefore, to reduce the redundant work for the same purpose (candidate gNB initiated cancellation), we slightly prefer the option 2, i.e., reuse the CHO Cancel message.

Proposal 5: Reuse the CHO Cancel message for the candidate gNB initiated cancellation of configured LTM candidate cells.

For R18 intra-CU LTM, the LTM CSI resource configuration is included in the LTM-Config IE and is a separate configuration, i.e., outside of the LTM candidate configuration. In addition, the LTM CSI resource configuration includes the RS configurations of multiple LTM candidate cells. The benefit is that the UE doesn’t need to open the LTM candidate configuration per each candidate cell, to support the fast L1 measurement report to the source DU.

– LTM-Config The IE LTM-Config is used to provide LTM candidate configurations. LTM-Config information element -- ASN1START -- TAG-LTM-CONFIG-START LTM-Config-r18 ::= SEQUENCE { ltm-ReferenceConfiguration-r18 SetupRelease {ReferenceConfiguration-r18} OPTIONAL, -- Need M ltm-CandidateToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-CandidateId-r18 OPTIONAL, -- Need N ltm-CandidateToAddModList-r18 SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-Candidate-r18 OPTIONAL, -- Need N ltm-ServingCellNoResetID-r18 INTEGER (1..maxNrofLTM-Configs-r18-plus-1) OPTIONAL, -- Cond FirstLTM-Only ltm-CSI-ResourceConfigToAddModList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfig-r18 OPTIONAL, -- Need N ltm-CSI-ResourceConfigToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfigId-r18 OPTIONAL, -- Need N attemptLTM-Switch-r18 ENUMERATED {true} OPTIONAL, -- Cond LTM-MCG ltm-ServingCellUE-MeasuredTA-ID-r18 INTEGER (1..maxNrofLTM-Configs-r18-plus-1) OPTIONAL, -- Cond LTM ... } -- TAG-LTM-CONFIG-STOP -- ASN1STOP

On receipt of the LTM CSI resource configuration, each candidate cell prepares the LTM CSI report configuration separately. The LTM CSI report configuration is placed within the ServingCellConfig IE for each candidate cell.

In our view, the same principle should be used for the R19 inter-CU LTM, to support the fast L1 measurement report by the UE.

In addition, at the RAN2#126 meeting, RAN2 agreed [2]:

Stage-2 signaling flows and procedures - The preparation of inter-CU LTM configuration is initiated by the source gNB-CU. - For each candidate cell, the preparation of lower layer configuration is initiated by the candidate gNB-CU, based on the LTM request from the source gNB-CU. RAN2 assumes the interaction between the candidate gNB-CU and candidate gNB-DU follows the same signaling procedure for intra-CU LTM. - The source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement on candidate cells. - In order to support subsequent LTM, the source gNB-CU needs to inform the candidate gNB-CU(s) about the common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB-CUs. The candidate gNB-CU(s) responds with the candidate configuration to the source gNB-CU accordingly (if needed).

Proposal 6: The source gNB sends the common LTM CSI resource configuration to the candidate gNB, and the candidate gNB prepares the LTM CSI report configuration for each LTM candidate cell separately.

Part B: LTM execution

LTM was introduced in R18 to offer improvements in handover latency and interruption time compared to L3 based mobility. With all this in mind, the LTM is defined as a switch procedure that the network triggers via MAC CE based on L1 measurements. That is, the LTM is a procedure in which a gNB receives L1 measurement report(s) from a UE, and on their basis the gNB changes UE serving cell by a cell switch command signalled via a MAC CE.

The inter-CU LTM could use the same principle to reduce the handover latency and interruption time between different cells in different gNBs. With this principle, the source gNB changes UE serving cell by a cell switch command signalled via a MAC CE based on the L1 measurement reports from the UE.

In addition, at the RAN2#126 meeting, RAN2 agreed:

Stage-2 signaling flows and procedures - The preparation of inter-CU LTM configuration is initiated by the source gNB-CU. - For each candidate cell, the preparation of lower layer configuration is initiated by the candidate gNB-CU, based on the LTM request from the source gNB-CU. RAN2 assumes the interaction between the candidate gNB-CU and candidate gNB-DU follows the same signaling procedure for intra-CU LTM. - The source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement on candidate cells. - In order to support subsequent LTM, the source gNB-CU needs to inform the candidate gNB-CU(s) about the common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB-CUs. The candidate gNB-CU(s) responds with the candidate configuration to the source gNB-CU accordingly (if needed). Execution: LTM Cell Switch Command - R18 LTM CSC MAC CE is baseline to trigger LTM cell switch for inter-CU LTM.

Proposal 7: The UE sends the L1 measurement report to the source gNB, and the source gNB triggers UE mobility to a target cell via LTM cell switch command MAC CE.

In R18, the Cell Switch Notification procedure is to enable the source DU to inform the target DU via the CU about the initiation of the cell switch command to the UE. This procedure is also used to transfer the selected TCI state from the source DU to the target DU via the CU.

At the RAN3#124 meeting, the group agreed:

A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU.

To make it forward compatible, we propose to remove the LTM in the agreements, like we did in R18.

Proposal 8: Change the name of LTM Cell Switch Notification message to Cell Switch Notification message for forward compatibility.

Part C: LTM completion

When the UE is switched to the target cell, the target gNB should inform the source gNB of which cell the UE has successfully accessed during the LTM. The source gNB can use this information for future purpose, e.g., whether to perform normal handover based on L3 measurement further.

At the RAN3#124 meeting, the group agreed:

Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell.

Since the candidate cell could be configured for both the CHO and the LTM simultaneously, the source gNB should know whether the access of target cell is due to the CHO or the LTM. For example, the LTM configuration should be always kept by the source gNB to support the subsequent LTM, while the CHO configuration should be always released by the source gNB, after the success of the handover. If the access of target cell is due to the LTM, the source gNB should maintain the LTM configuration and CHO configuration unless receiving the UE Context Release from the target gNB. Otherwise, if the access of target cell is due to the CHO, the source gNB should maintain the LTM configuration but release the CHO configuration.

Therefore, an indicator is needed in the Handover Success message to indicate the target cell is an LTM cell.

Proposal 9: An indicator is introduced in the Handover Success message to indicate the target cell is an LTM cell.

Part D: Data forwarding

At the RAN3#123bis meeting, the group agreed:

Early data forwarding can be supported for inter-CU LTM. When to trigger can be further discussed.

At the RAN3#124 meeting, the group further agreed:

Early data forwarding can be triggered before the Source gNB triggers a MAC CE Command to the UE to change cells, timing is left to implementation.

There are two data forwarding methods as defined in TS 38.300.

Early data forwarding: data forwarding that is initiated before the UE executes the handover. Late data forwarding: data forwarding that is initiated after the source NG-RAN node knows that the UE has successfully accessed a target NG-RAN node.

Based on the definition from TS 38.300, there are two possible options to trigger the early data forwarding:

- Option 1: early data forwarding is triggered after Cell Switch Command. The data forwarding starts when Cell Switch Command is triggered, which is at an earlier stage compared to the late data forwarding. The benefit is that it does not need to forward the data to all the candidate gNBs. The drawback is that it causes delays.

- Option 2: early data forwarding is triggered before Cell Switch Command, e.g., during the LTM preparation phase. The benefit is that it reduces the delays as much as possible. The drawback is that it causes unnecessary data forwarding to the candidate gNBs.

In our point of view, both of options have the benefits and drawbacks. The source gNB can determine when to trigger the early data forwarding based on the implementation.

Proposal 10: Early data forwarding can be triggered by the source gNB after sending a MAC CE Command to the UE but before receiving Handover Success from the target gNB.

Part E: Subsequent LTM

In R18, subsequent LTM is always supported that indicates the LTM cell switch procedures between candidate cells without RRC reconfiguration by the network in between. That is, the LTM configuration will not be released by UE unless it is explicitly reconfigured by the network. Hence, the network should always maintain the LTM configuration.

To support the subsequent LTM, some LTM configurations, e.g., early sync configuration and data forwarding information, should be synchronized among candidate gNBs. In the following text, we will take the data forwarding information as an example.

The source gNB obtains the data forwarding information associated with all the candidate gNBs via Handover Preparation procedure. Therefore, for the first-time cell switch (i.e., from the source gNB to the candidate gNB), the source gNB can perform the early data forwarding directly. However, for the subsequently inter-CU LTM, the new source gNB doesn’t have the data forwarding information associated with other candidate gNBs, hence it can’t perform the early data forwarding.

Take the UE moves from the source gNB to the candidate gNB1 and then to the candidate gNB2 for example. There are three options for the new source gNB to know the data forwarding information associated with other candidate gNBs.

- Option 1: the source gNB collects the data forwarding information associated with all candidate cells from each candidate gNB, and sends it to all the candidate gNBs during the LTM preparation phase. For example, the source gNB sends the data forwarding information to the gNB1 and gNB2 during the LTM preparation phase. This option may introduce new XnAP signalling between the source gNB and the candidate gNBs.

- Option 2: the source gNB collects the data forwarding information associated with all candidate cells from each candidate gNB, and sends it to the selected target gNB, e.g., via Cell Switch Notification message. For the subsequent inter-CU LTM, the new source gNB sends the data forwarding information to the new selected target gNB. For example, the source gNB sends the data forwarding information to the gNB1 during the first-time cell switch, and then gNB1 sends the data forwarding information to the gNB2 during the second-time cell switch.

- Option 3: the source gNB collects the data forwarding information associated with all candidate cells from each candidate gNB, and sends it to the selected target gNB, no matter which time the inter-CU LTM is performed. For example, the source gNB sends the data forwarding information to the gNB1 for the first-time cell switch, and the source gNB sends the data forwarding information to the gNB2 for the second-time cell switch.

Proposal 11: RAN3 to discuss how to support early data forwarding in subsequent inter-CU LTM with following three options.

  - Option 1: the source gNB collects the data forwarding information associated with all candidate cells, and then sends the collected data forwarding information to all candidate gNBs during the LTM preparation.

  - Option 2: the source gNB sends the collected data forwarding information to the target gNB (i.e., new source gNB) for the first LTM cell switch, and then the new source gNB sends the collected data forwarding information to the new target gNB for subsequent LTM cell switch.

  - Option 3: the source gNB sends the collected data forwarding information to the target gNB for the first LTM cell switch and the subsequent LTM cell switch.

At the RAN3#124 meeting, there is an FFS [3] as below.

FFS on whether to reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

From our point of view, we think this issue should be discussed with the subsequent LTM. For example, if the option 3 is adopted by the group to support the subsequent LTM, it means the source gNB behaving as the anchor of the subsequent LTM. That is, the target gNB (selected candidate gNB) should obtain the LTM configuration from the source gNB for the subsequent LTM. Therefore, the source gNB should maintain the UE context, and the UE Context Release should not be sent to the source gNB.

Proposal 12: Postpone the discussion of UE Context Release until the subsequent LTM solution is clear.

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals.

LTM preparation

Proposal 1: Parallel procedures are used to prepare LTM candidate configurations, i.e., one message is for one LTM candidate configuration.

Proposal 2: An indicator is introduced in the Handover Request message to request the candidate gNB to provide LTM candidate configuration per candidate cell.

Proposal 3: The candidate gNB may accept the target cell of LTM and respond to the source gNB with the LTM candidate configuration in Handover Request Acknowledge message.

Proposal 4: The source gNB may modify or release already prepared LTM candidate cells in the candidate gNB.

Proposal 5: Reuse the CHO Cancel message for the candidate gNB initiated cancellation of configured LTM candidate cells.

Proposal 6: The source gNB sends the common LTM CSI resource configuration to the candidate gNB, and the candidate gNB prepares the LTM CSI report configuration for each LTM candidate cell separately.

LTM execution

Proposal 7: The UE sends the L1 measurement report to the source gNB, and the source gNB triggers UE mobility to a target cell via LTM cell switch command MAC CE.

Proposal 8: Change the name of LTM Cell Switch Notification message to Cell Switch Notification message for forward compatibility.

LTM completion

Proposal 9: An indicator is introduced in the Handover Success message to indicate the target cell is an LTM cell.

Data forwarding

Proposal 10: Early data forwarding can be triggered by the source gNB after sending a MAC CE Command to the UE but before receiving Handover Success from the target gNB.

Subsequent LTM

Proposal 11: RAN3 to discuss how to support early data forwarding in subsequent inter-CU LTM with following three options.

  - Option 1: the source gNB collects the data forwarding information associated with all candidate cells, and then sends the collected data forwarding information to all candidate gNBs during the LTM preparation.

  - Option 2: the source gNB sends the collected data forwarding information to the target gNB (i.e., new source gNB) for the first LTM cell switch, and then the new source gNB sends the collected data forwarding information to the new target gNB for subsequent LTM cell switch.

  - Option 3: the source gNB sends the collected data forwarding information to the target gNB for the first LTM cell switch and the subsequent LTM cell switch.

Proposal 12: Postpone the discussion of UE Context Release until the subsequent LTM solution is clear.

4. Reference

- RAN3#124 meeting minutes.

- R2-2405701, Report from session on V2X/SL, R19 NES and MOB.

- R3-243836, Summary of offline for R19 LTM procedure.