3GPP TSG-RAN WG3 Meeting #125	R3-244328

Maastricht, Netherlands, 19th – 23rd August 2024

Agenda item:	16.3

Source: 	Lenovo

Title: 	Inventory procedure of ambient IoT

Document for:	Discussion and Approval

1. Introduction

The ambient IoT was discussed at the RAN3#124 meeting with the following agreements.

Inventory can be sent by the AIoT CN for a single device, or a group of devices, or all devices. The Inventory Request from the AIoT CN to the AIoT RAS, may include the following: (1) AIoT Device Identification (to find a single device, a group of devices, or all devices) Note 1: The definition of this identification is out of RAN3 scope. Editor’s Note: It is FFS whether AIoT RAS needs to interpret/store/process it. (2) Scope of inventory request (e.g., a certain area in which the inventory is to be triggered)

In this contribution, we discuss the remaining issues of AIOT inventory related to RAN3 impact.

2. Discussion

The signalling following for inventory procedure in topology 1 is given in the Fig.1.

Firstly, the AIOT CN node (AIOTF or AMF with collocated AIOT function) sends an Inventory Request message to the AIOT RAN node to trigger the inventory procedure.

The inventory request message should include the following information:

-	AIoT Device identification (to find a single device, a group of devices, or all devices);

-	Scope of inventory request: our understanding is the scope of inventory request should be reader ID(s) that needs to send paging message to A-IOT devices.

Above information has been agreed in last RAN3 meeting. There are additional information needs to be provided to AIOT RAN node for inventory procedure.

Firstly, there may be multiple inventory procedures which are performed in parallel. In this case, it needs to distinguish the inventor procedures in the interface between AIOT RAN node and AIOT CN node. It makes sense to introduce a inventory session or task ID to identify different inventory procedures.

Proposal 1: Introduce an inventory session or task ID to identify different inventory procedures in the interface between AIOT RAN node and AIOT CN node.

There are two services of AIoT, namely inventory and command. For the command service, the AIoT-aware CN node performs reading data from an AIoT device, writing data to an AIoT device, or disable an AIoT device temporarily or permanently.

At RAN3#123bis meeting, the group agreed:

AIoT Paging can be used to reach one or more devices for identified AIoT services (e.g., inventory, command).

The main reason is that the AIoT-aware CN node may have the valid AIoT device ID for the AIoT device. With the agreement, the AIoT-aware CN node should indicate to the AIoT RAN node whether the paging message is originated due to the inventory service operation or the command service operation. If the paging message is originated due to the inventory service, the AIoT RAN node triggers the inventory procedure towards the AIoT device. If the paging message is originated due to the command service, the AIoT RAN node stores the command request information and triggers the random access procedure to obtain the AIoT device ID. After that, the AIoT RAN node sends the command request information to the AIoT device.

Proposal 2: The service operation type (e.g., inventory, inventory and command) is included in the Inventory Request message indicating the message is originated due to which AIoT service type.

Fig.1. Signaling flow for inventory procedure.

We also think the number of A-IOT Devices for which ID to be collected should also be provided in the inventory request message. According to RAN1’s agreement, Slotted ALOHA-based random-access procedure is used as baseline for inventory. In this case, the AIOT RAN node needs to know how many times of Query or QueryRep procedures should be performed towards A-IOT devices IDs. Following the Slotted ALOHA-based random access procedure in RFID, the random access procedure in 5G system could be:

-	DL triggering (Msg 0). A gNB sends DL signal (similar as Query or QueryRep in RFID) to trigger the A-IoT device(s) to perform access.

-	A-IoT message 1 transmission (Msg 1). Each A-IoT device joining this access round randomly selects one slot number j among N=fun.(Q) slots. If j=0, the A-IoT device performs UL transmission based on the scheduling information indicated by the Reader, including at least a (Random Number) RN16.

-	Reader Access Response (Msg 2). gNB sends ACK with the same RN16 to acknowledge the successful reception and scheduling information for next UL transmission.

-	A-IOT Device ID Reporting (Msg 3). Based on the Msg2, the A-IOT device sends corresponding parameters asked by paging for inventory, i.e., the A-IOT Device ID, as PC/XPC, EPC, packet CRC in RFID, this is referred to Msg3.

-	gNB trigger next round of random access procedure.

We think the number of A-IOT Devices for which ID to be collected should be provided for AIOT RAN Node deciding the number of time slots for A-IOT device ID reporting in air interface.

Proposal 3: The number of A-IOT Devices for which ID to be collected is provided in the Inventory Request message so that the AIOT RAN node can decide the number of A-IOT device ID reporting procedure in air interface.

Considering that the AIoT device is with limited energy storage and the energy is provided through the harvesting of radio waves, light or any other power source that could be suitable, the AIoT device can’t be always on to receive the message from the AIoT RAN node. The AIoT RAN node should determine the inventory starting time or completing time towards the AIoT device.

For example, the RAN1#116 meeting agreed the AIoT device 1 architecture as follows:

Agreement Study at least the following blocks for device 1 architecture. - Antenna could be either shared or separate for RF energy harvester and receiver/transmitter. - Matching network is to match impedance between antenna and other components (including RF energy harvester and receiver related blocks). - RF energy harvester can include rectifier performing RF signal (AC) to DC conversion. - Energy storage (e.g., capacitor) stores harvested energy from RF energy harvester. - Power management unit (PMU) manages storing energy to energy storage from energy harvester and suppling power to active component blocks which needs power supply. - Digital BB logic includes functional blocks like encoder, decoder, controller, etc. - Memory can include two types of memory: 1) Non-Volatile Memory (NVM) such as EEPROM for permanently storing device ID, etc, and 2) registers for temporarily keeping any information required for its operation only while energy is available in energy storage. - Clock generator provides required clock signal(s). - Reception related blocks - RF BPF for improving selectivity. - Depending on implementation, it may not exist. RAN4 RF requirement (if any, e.g., ACS) and peak power consumption target also need to be considered. - RF Envelope Detector converts RF signal to baseband. - BB LPF can filter out harmonics and high frequency components to improve input signal quality to comparator. - Depending on implementation, it may not exist. Presence of BB LPF is assumed for the study. - Comparator determines high/low of input signal. - Transmission related blocks - Backscatter modulator switches impedance to modulate backscattered signal with tx signal from BB logics. Waveform/Modulation type is FFS.

If the antenna is shared for RF energy harvester and receiver/transmitter, the AIoT RAN node can’t communicate with the AIoT device directly when the AIoT device is performing energy harvesting. Therefore, it is beneficial that the AIoT-aware CN node could provide some assistance information included in the Inventory Request message to the AIoT RAN node, which will be used by the AIoT RAN node to determine the inventory starting time or completing time towards the AIoT device. For example, based on the antenna type (e.g., shared or separate between the RF energy harvester and receiver/transmitter) provided by the AIoT-aware CN node, the AIoT RAN node determines the duration of providing RF for energy harvesting to the AIoT device or the waiting time for energy harvesting by the AIoT device.

Proposal 4: Assistance information (e.g., AIOT device type or category) is included in the inventory request message, to be used by the AIoT RAN node to determine the inventory starting time or completing time.

Considering the number of ambient IOT devices could be very large, it would be better to allow the AIOT RAN node to report a list of AIOT device IDs in a signal message. For example, the AIOT RAN node stores the A-IOT Device ID received from the A-IOT devices until all device IDs have been collected. The AIOT RAN node reports all A-IOT Device IDs in a single message to the AIOT CN node. According to proposal 3, the AIOT RAN node has already known the number of A-IOT Devices for which ID to be collected. This information can be used for AIOT RAN node deciding when to send the device IDs to the AIOT CN node.

Proposal 5: AIOT RAN node should be able to send a list AIOT Device IDs in a single message to AIOT CN node.

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals.

Proposal 1: Introduce an inventory session or task ID to identify different inventory procedures in the interface between AIOT RAN node and AIOT CN node.

Proposal 2: The service operation type (e.g., inventory, inventory and command) is included in the Inventory Request message indicating the message is originated due to which AIoT service type.

Proposal 3: The number of A-IOT Devices for which ID to be collected is provided in the Inventory Request message so that the AIOT RAN node can decide the number of A-IOT device ID reporting procedure in air interface.

Proposal 4: Assistance information (e.g., AIOT device type or category) is included in the inventory request message, to be used by the AIoT RAN node to determine the inventory starting time or completing time.

Proposal 5: AIOT RAN node should be able to send a list AIOT Device IDs in a single message to AIOT CN node.

4. Reference

- 3GPP TR 38.848, Study on Ambient IoT (Internet of Things) in RAN.