3GPP TSG RAN WG3 #125	R3-244331

Maastricht, Netherlands, 19th – 23rd Aug 2024

Agenda item:	17.3

Source: 	Lenovo

Title: 	Discussion on On-demand SIB1 for Idle/Inactive UE

Document for:	Discussion and Decision

Introduction

In RAN#102, a new work item “Enhancements of network energy savings for NR” was agreed [1]. The one objective of the work item is the following:

- Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105

RAN3#123bis has discuss the on demand SIB1 on a scenario and reached the agreement：

Scenario1: UE obtains the UL WUS configuration from Cell A, and transmits UL WUS to NES Cell. - UL WUS configuration from NES Cell to Cell A via Xn, e.g., before entering on-demand SIB1 mode in the NES Cell or when the UL WUS configuration is changed

RAN3#124 has discuss the on demand SIB1 on a scenario and reached the agreement：

Focus on case2 in RAN3 first.

In this contribution, we further analyse potential RAN3 impact and high-level solution based on RAN1 and RAN2 progress.

Discussion

2.1 RAN1 and RAN2 progress

In RAN1#116, RAN1 has on-demand SIB1 operation, and it was agreed:

- For discussion purpose, the following assumption will be used in RAN1 - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 studies the following options. - On target cell of UL WUS transmission: - Option 1: UE transmits UL WUS to NES Cell - Option 2: UE transmits UL WUS to Cell A - On configuration provision for UL WUS transmission - Option A: UE obtains the UL WUS configuration from NES Cell - Option B: UE obtains the UL WUS configuration from Cell A - For study of UL WUS design, consider at least PRACH as a starting point - FFS: Whether there is dedicated PRACH resource for SIB1 request - For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 to discuss triggering conditions for sending UL-WUS. - For the study of on-demand SIB1 for idle/inactive mode UE, RAN1 to further study whether feedback from gNB in response to the SIB1 request is supported including associated details.

In RAN1#116bis discussion on the on demand SIB1 with the following agreements:

For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 focuses its studies on the following cases: - Case 1: Option 1+A+X - Case 2: Option 1+B+X - Case 3: Option 2+B+Y Where the options 1/2/A/B/X/Y are defined below: - On target cell of UL WUS transmission: - Option 1: UE transmits UL WUS to NES Cell - Option 2: UE transmits UL WUS to Cell A - On configuration provision for UL WUS transmission - Option A: UE obtains the UL WUS configuration from NES Cell - Option B: UE obtains the UL WUS configuration from Cell A - On receiving of SIB1 - Option X: UE receives on-demand SIB1 from NES Cell - Option Y: UE receives on-demand SIB1 from Cell A For UL WUS design for SIB1 request, at least dedicated PRACH resource is the assumption for further study in RAN1 - FFS: Details on time, frequency, and/or PRACH preamble resources for UL WUS - FFS: whether RACH resource for SIB1 request could be used for an initial access procedure and/or an on-demand SI procedure

In RAN1#117 discussion on the on demand SIB1 with the following agreements:

For SIB1 in idle/inactive mode, prioritize RAN1 discussions on Case 2 and Case 3 - Case 2 (Option 1+B+X) is feasible from RAN1 perspective. - Further study Case 3, focusing on the additional NES benefits over Case 2, feasibility, complexity, and spec impact. From RAN1 point of view, the following is feasible. It is up to RAN2 to decide whether/how to support it. At least for Case 2 (Option 1+B+X) design, a unified configuration format that can support both Option 2 (i.e. a UL-WUS configuration applies to multiple NES cells) and Option 3 (i.e. a UL-WUS configuration applies to a single NES cell).

Based on RAN1 agreement, we have following observations.

Observation 1: RAN1 prioritize the following cases on on-demand SIB1 for idle/inactive mode UE:

- Case 2: UE transmits UL WUS to NES Cell, UE obtains the UL WUS configuration from Cell A, UE receives on-demand SIB1 from NES Cell.

- Case 3: UE transmits UL WUS to Cell A, UE obtains the UL WUS configuration from Cell A, UE receives on-demand SIB1 from Cell A.

Observation 2: At least dedicated PRACH resource is considered for UL WUS design for SIB1 request.

In RAN2#125bis initial discussion on the on demand SIB1 for Rel-19 took place with the following agreements:

Agreements on on-demand SIB1:

- At least RAN2 starts scenario 1a (Cell A SIB assisted intra-cell WUS. And WUS and SIB1 is sent to/from NES cell). Other scenarios are not excluded.

- RAN2 assume that RACH procedure is reused for UE to request on-demand SIB1.

- UL WUS configuration includes at least RACH configuration.

- A UE needs to know a UL WUS configuration to request SIB1 of which cell.

- Existing Msg 1 based on-demand procedure is reused for on-demand SIB1 acquisition procedure. FFS on Msg 3. FFS if / when the UE monitors the OD-SIB1 upon reception of RAR. FFS: whether introduce specified UE behavior if RACH failure of OD-SIB1 request.

Agreements on on-demand SIB1:

- The UE first should acquire valid SIB1 (e.g. via SIB1 request) for camping to NES cell (if the UE knows the cell doesn’t broadcast SIB1 and supports on-demand SIB1).

In RAN2#126 discussion on the on demand SIB1 for Rel-19 took place with the following agreements:

Agreements on on-demand SIB1:

- Study on-demand SIB1 provisioning for NES Cell(s) in versions of Scenario 1a with multiple Cells A and/or NES Cells:

- More than one Cell A may provide configuration for the same NES cell.

- The same Cell A may assist more than one NES Cells.

- RRC release message assisted intra-cell WUS can be discussed as option of signaling details in stage 3.

Information of WUS configuration

- Can use the PCI and frequency of a NES The same Cell A may assist more than one NES Cells.Cell to associate the UL WUS configuration with a NES Cell.

- For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline.

Transmission of WUS configuration

- Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB.

Triggering condition of WUS transmission

- If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

- After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell.

UE request SIB1 to perform initial access in NES cell

- RAN2 not to support on-demand SIB1 request that is combined with an initial access to perform RRC connection establishment/resume on the NES cell.

Legacy UE impact

- NW need to bar the legacy UE from accessing the on-demand SIB1 cell (e.g. based on the existing barring mechanism)

- How to avoid/deprioritize the legacy UE camping at the Cell A attempting to switch to the NES Cell (but allowing the R19 NES UE to do that).

Barring for R19 NES UE

- If the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment. R19 NES UE bars the cell if the UE fails to acquire SIB1 via on-demand SIB1 for NES cell.

Paging and SIB1 update in NES cell

- After Rel-19 NES UEs camp in NES cell, the UE behaviour is same as the one defined as legacy normal camped state, e.g. paging reception, SIB1 update, etc.

RAR monitoring for MSG1 based OD-SIB1 REQ

- RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

UE behaviour on RACH failure of OD-SIB1 REQ

- As baseline, upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation.

SIB1 acquisition upon SIB change notification in NES cell

- Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.

Scenario 3 (case 1 in RAN1)

- RAN2 to wait for RAN1’s progress whether to support scenario 3.

Based on RAN2 agreement, we have following observations on case 2(Option 1+B+X in RAN1 agreement).

Observation 3:  More than one Cell A may provide configuration for the same NES cell. The same Cell A may assist more than one NES Cells.

Observation 4:  A UL WUS configuration to request SIB1 is associated with at least an NES cell by using the PCI and frequency of a NES Cell. Cell A’s SIB can be used to configure UL WUS config for neighbour NES cells, e.g., via new SIB or the existing SIB.

Observation 5:  RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

Proposal 1: RAN3 should focus on Case 2: UE transmits UL WUS to NES Cell, UE obtains the UL WUS configuration from Cell A, UE receives on-demand SIB1 from NES Cell, and leave RAN1 to further study on Case 3.

2.2 on-demand SIB1 procedure

Based on RAN1 agreements, we analyse the possible issues of the UL WUS configuration provision procedure and the SIB1 request procedure based on UL WUS and potential RAN3 impacts for case 2 identified by RAN1.

UL WUS configuration provision procedure

Issue1. which node to determine to use on-demand SIB1 for a NES cell in case of CU-DU split.

One possibility is that CU determines which cells use on-demand SIB1 and transmits the command to DU to enable the on-demand SIB1 function via gNB-CU Configuration Update procedure, and according DU enables on-demand SIB1.  For example, during the enabling procedure, DU transmits UL WUS config to the CU. Another possibility is DU determines which cells use on-demand SIB1. And which cells are allowed to use on-demand SIB1 may be configured by OAM or determined by CU. Since CU has an overview of the load and coverage of cells, we slightly prefer the former.

Proposal 2: CU determines which cells to use on-demand SIB1 and transmits the command to DU to enable the on-demand SIB1 function.

Issue 2. which node to determine UL WUS configuration in case of CU-DU split.

Based on RAN1 agreement, the UL WUS may be designed from PRACH resource. Legacy RACH config in SIB1 is determined by DU. For simplicity, DU generates UL WUS config and then transmits it to CU if requested by CU, e.g., via F1 SETUP REQUEST or gNB-DU CONFIGURATION UPDATE message. CU may further transmit it to another node associated with Cell A(s) if the Cell A can provide it to UE for NES Cell. Otherwise, CU can generate UL WUS config based on RACH config in SIB1, and then transmits it to DU. We slightly prefer the former. After DU enables on-demand SIB1 for an NES cell, DU may further inform the CU of the enabling status of on-demand SIB1.

Proposal 3: DU of a NES cell determines UL WUS config for the NES cell based on the request from CU, and transmits the UL WUS config to CU for further providing it to Cell A.

Issue 3. To support of providing UL WUS config to UE from Cell A, how to provide UL WUS config to CU2 of Cell A from CU1 of NES cell, and further provide it to DU of Cell A.

In last meeting, it has been agreed that:

UL WUS configuration from NES Cell to Cell A via Xn, e.g., before entering on-demand SIB1 mode in the NES Cell or when the UL WUS configuration is changed.

According to RAN2 agreement, the same Cell A may assist more than one NES Cells, therefore, the RAN node of NES cell transmits UL WUS config of a list of NES cells in the NG-RAN NODE CONFIGURATION UPDATE message to the RAN node of Cell A by means of the NG-RAN node Configuration Update procedure. Depending on whether Cell A support to transmit UL WUS config for NES cell, Cell A can accept or reject providing UL WUS config of NES Cell.  Since additional power is consumed if NG-RAN of Cell A provides UL WUS config of the NES cell to UE. Cell A provides the UL WUS config for an NES cell and notify the decision to NES cell after Cell A has accepted the UL WUS config.

In case of CU-DU split, after CU of Cell A receives the UL WUS config of a set of NES cells, it may further separate UL WUS config(s) of some NES cells per Cell A and transmit the UL WUS config(s) and the corresponding NES cell(s) per Cell A to DU.

Proposal 4: RAN node of NES Cell sends UL WUS config of a list of NES cells to RAN node of Cell A, e.g., via NG-RAN Node Configuration Update procedure. Cell A can accept or reject providing UL WUS config of NES Cell.

Proposal 5: CU of cell A separates UL WUS config(s) per Cell A and transmits the UL WUS config(s) and the corresponding NES cell(s) per Cell A to DU, e.g., via gNB-CU Configuration Update procedure.

NES Cell on demand SIB activation/deactivation

After CU of NES cell receives accept response of UL WUS config from Cell A, CU of NES cell determines to enter NES mode, i.e., CU of NES cell transmits on-demand SIB1 activation to DU, then DU enters the OD-SIB1 mode. There is no need to wait the activation indication from the NES cell, Cell A starts providing it upon receiving the UL WUS config of NES cell. Cell A provides it for NES cell until receiving the release indication of UL WUS config of the NES cell.

Proposal 6:  CU of NES cell transmits On-demand SIB1 operation activation command to DU of NES cell based on receiving accept response of UL WUS config from Cell A.

Proposal 7:  NES cell does not transmit On-demand SIB1 operation activation/deactivation indication from the RAN node of NES cell to RAN node of the Cell A via Xn.

SIB1 request procedure based on UL WUS

Issue 4. The target cell of UL WUS is NES cell, which node handles UL WUS from UE.

The handling mechanism is similar as on-demand other SI. In last RAN2 meeting, RAN2 tend to reuse the existing Msg 1 based on-demand procedure for on-demand SIB1 acquisition. If receiving request via MSG1, DU can response via MSG2 and broadcasts SIB1. This procedure has no impact on RAN3. Since CU control a cell to use on-demand SIB1, DU may further inform CU the SIB1 is broadcasting if DU broadcasts SIB1 triggered by UE. Whether this notification has RAN3 impact is FFS, it depends on RAN1 design how to indicate SIB1 is broadcasting.

Proposal 8: DU broadcast SIB1 upon receiving MSG1 based on-demand SIB1 request, there’s no RAN3 impact foreseen.

Conclusion

In summary, we have following observations and proposals for on-demand SIB1 for Idle/Inactive UE:

Observation 1: RAN1 prioritize the following cases on on-demand SIB1 for idle/inactive mode UE:

- Case 2: UE transmits UL WUS to NES Cell, UE obtains the UL WUS configuration from Cell A, UE receives on-demand SIB1 from NES Cell.

- Case 3: UE transmits UL WUS to Cell A, UE obtains the UL WUS configuration from Cell A, UE receives on-demand SIB1 from Cell A.

Observation 2: At least dedicated PRACH resource is considered for UL WUS design for SIB1 request.

Observation 3:  More than one Cell A may provide configuration for the same NES cell. The same Cell A may assist more than one NES Cells.

Observation 4:  A UL WUS configuration to request SIB1 is associated with at least an NES cell by using the PCI and frequency of a NES Cell. Cell A’s SIB can be used to configure UL WUS config for neighbour NES cells, e.g., via new SIB or the existing SIB.

Observation 5:  RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

Proposal 1: RAN3 should focus on Case 2: UE transmits UL WUS to NES Cell, UE obtains the UL WUS configuration from Cell A, UE receives on-demand SIB1 from NES Cell, and leave RAN1 to further study on Case 3.

Proposal 2: CU determines which cells to use on-demand SIB1 and transmits the command to DU to enable the on-demand SIB1 function.

Proposal 3: DU of a NES cell determines UL WUS config for the NES cell based on the request from CU, and transmits the UL WUS config to CU for further providing it to Cell A.

Proposal 4: RAN node of NES Cell sends UL WUS config of a list of NES cells to RAN node of Cell A, e.g., via NG-RAN Node Configuration Update procedure. Cell A should be able to accept or reject for providing UL WUS config of NES Cell.

Proposal 5: CU of cell A separates UL WUS config(s) per Cell A and transmits the UL WUS config(s) and the corresponding NES cell(s) per Cell A to DU, e.g., via gNB-CU Configuration Update procedure.

Proposal 6:  CU of NES cell transmits On-demand SIB1 operation activation command to DU of NES cell based on receiving accept response of UL WUS config from Cell A.

Proposal 7:  NES cell does not transmit On-demand SIB1 operation activation/deactivation indication from the RAN node of NES cell to RAN node of the Cell A via Xn.

Proposal 8: DU broadcast SIB1 upon receiving MSG1 based on-demand SIB1 request, there’s no RAN3 impact foreseen.

References

- RP-234065, “New WID: Enhancements of network energy savings for NR”, RAN#102.

- R3-243405, Discussion on On-demand SIB1 for Idle/Inactive UE, Lenovo