3GPP TSG-RAN WG3 Meeting #125						          R3-244332

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	 21.1

Source:	 Lenovo

Title:	 Discussion on LS on FS_XRM Ph2

Document for:	Discussion and Decision

- Introduction

This paper discusses RAN3 replies to  SA2 LS on FS_XRM Ph  [1].

- Discussion

- Question1 [for SA4, RAN2 and RAN3]:

  - SA2 discusses indicating periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity and kindly asks RAN2 and RAN3 to feedback on that approach.

- Question2 [for SA4 and RAN2]: There is some discussion about time to next burst.

  - To SA4: is it possible that the application server provides the time to next burst (i.e. the time interval between the current burst and the next burst) in the 1st packet of the burst via N6?

  - Does RAN2 think the time to next burst is useful for RAN resource scheduling, especially with regard to the possible jitter between the NG-RAN and the Application Server?

- Currently, for DL 5GC may provide XR traffic assistance information to gNB through NG AP TSC Assistance Information (TSCAI). The XR traffic assistance information includes

-	UL and/or DL Periodicity;

-	N6 Jitter Information (i.e. between UPF and Data Network) associated with the DL Periodicity.

- For UL, uplink assistance information (jitter range, burst arrival time, UL data burst periodicity) per QoS flow is sent by the UE to gNB via UE Assistance Information.

- Based on above assistance information, gNB can know the periodicity and jitter range for data burst for both UL and DL data transmission. If any update in the assistance information, gNB can get the updated information by signalling i.e., 5GC can sent updated information by NG-AP signing and UE can report the updated information by UE Assistance Information. It is under assumption that the XR traffic assistance information does not change very frequently.

- In Release 18, RAN2 had discussed the inter-burst time according to the incoming LS in [2]. In the incoming LS, it is stated as:

In addition to marking the last PDU of the data burst, SA4 sees the benefit of using additional bits to indicate inter-burst time, which may change dynamically due to various reasons, including application-layer rate control. SA4 believes that this can enable the RAN to switch to the most appropriate power state. SA4 kindly requests feedback from SA2 and RAN2 on the value and feasibility of such solution and if that can be supported within Rel. 18 timeframe.

- However, RAN2 had not identified the inter-burst time as particularly useful. Firstly, RAN2 did not understand that how frequently the inter-burst time changes. It was assumed that the periodicity and jitter of a data burst do not change frequently. And RAN2 did not identify whether this information is helpful to RAN, for example whether the information can be useful for power saving or radio resource utilization.

- Following the Release 18 discussion, it is still unclear why the time interval between data bursts changes frequently and what’s the benefits for RAN using it.

It is unclear why the periodicity and time interval between data bursts changes frequently and does not identify that the time to next burst is beneficial for RAN resource scheduling. RAN3 can leave the Q1&Q2 to RAN2.

- Question3 [for RAN3]: SA2 discusses to support available data rate exposure for GBR QoS Flow via user plane, and kindly asks RAN3 to provide feedback if any.

- In last RAN3 meeting, RAN3 made a conclusion on that it is feasible to estimate available data rate for GBR QoS flows, and how to estimate available data rate for GBR QoS flow is up to RAN implementation.

- As specified in TS 38.415, NG-RAN node can send UL PDU SESSION INFORMATION in the "PDU Session Container" GTP-U Extension Header:

| Bits |  |  |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 |  | 6 | 5 |  | 4 | 3 |  | 2 | 1 | 0 |  |
| PDU Type (=1) |  |  |  |  |  | QMP | DL Delay Ind. |  | UL Delay Ind. | SNP | 1 |
| N3/N9 Delay Ind. | New IE Flag |  | QoS Flow Identifier |  |  |  |  |  |  |  | 1 |
| DL Sending Time Stamp Repeated |  |  |  |  |  |  |  |  |  |  | 0 or 8 |
| DL Received Time Stamp |  |  |  |  |  |  |  |  |  |  | 0 or 8 |
| UL Sending Time Stamp |  |  |  |  |  |  |  |  |  |  | 0 or 8 |
| DL Delay Result |  |  |  |  |  |  |  |  |  |  | 0 or 4 |
| UL Delay Result |  |  |  |  |  |  |  |  |  |  | 0 or 4 |
| UL QFI Sequence Number |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| N3/N9 Delay Result |  |  |  |  |  |  |  |  |  |  | 0 or 4 |
| New IE flag 7(E) |  | New IE Flag 6 |  | New IE Flag 5 | New IE Flag 4 | New IE Flag 3 |  | New IE Flag 2 | New IE Flag 1 | New IE Flag 0 | 0 or 1 New IE Flags Octet |
| Spare |  |  |  |  |  |  |  |  |  | D1 UL PDCPDelay Result Ind | 0 or 1 |
| Padding |  |  |  |  |  |  |  |  |  |  | 0-3 |

- As shown in the tabular, NG-RAN node has already been able to provide some QoS monitoring information such as DL delay result to the UPF directly.

- Observation 1	Currently, the NG-RAN node has been able to provide some QoS monitoring information such as DL delay result to the UPF directly in the "PDU Session Container" GTP-U Extension Header.

- According to the observation 1 and observation 2, we believe that there is no any technical issue that NG-RAN provides the QoS Notification Control information in the "PDU Session Container" GTP-U Extension Header to UPF.

Respond SA2 that RAN3 does not see any technical issue that the NG-RAN indicates that " available data rate " for a GBR flow via GTP-U (e.g., in the "PDU Session Container" GTP-U Extension Header) to UPF.

- For control plane-based solution, as the QoS Notification Control (QNC), there are two possible ways to report the available bit rate for a QoS flow to avoid exploring the detailed data rate information which relies on gNB’s implementation: First one is that the SMF can provide different bitrate span to the RAN, e.g. the three level bitrate span are 0~500kbps, 500~2500kbps, above 2500kpb ~ Ͼ. Only after the available mean bitrate is changed from one bitrate level to another bitrate level, the RAN can generate the non-GBR QoS Notification Control Report Message. The second on is that the SMF can provide different bitrate changes percentage to RAN, only after the available mean bitrate is changed UP/Down above the change percentage, the RAN will generate the non-GBR QoS Notification Control Report message. One example of 3 level bitrate change percentage can be 50%, 100%, 200%.

Respond SA2 that RAN3 also thinks it is feasible to have a control plane based solution which is a similar mechanism as QNC for reporting the available bit rate for a GBR QoS flow.

3	Conclusion

This paper discusses SA2 LS on FS_XRM Ph2 (S2-2407351). And we propose:

- It is unclear why the periodicity and time interval between data bursts changes frequently and does not identify that the time to next burst is beneficial for RAN resource scheduling. RAN3 can leave the Q1&Q2 to RAN2.

- Respond SA2 that RAN3 does not see any technical issue that the NG-RAN indicates that " available data rate " for a GBR flow via GTP-U (e.g., in the "PDU Session Container" GTP-U Extension Header) to UPF.

- Respond SA2 that RAN3 also thinks it is feasible to have a control plane based solution which is a similar mechanism as QNC for reporting the available bit rate for a GBR QoS flow.

Reference

[1] S2-2407351. LS on FS_XRM Ph2

[2] R2-2304659	LS out on the N6 PDU Set Identification (S4-230739; contact: Intel)	SA4