3GPP TSG-RAN WG3 Meeting #125	R3-244333

Maastricht, Netherlands, 19th – 23rd August 2024

Agenda item:	  21.2

Source:		Lenovo

Title:		(TP for 38.340 BLR) Corrections on XR in NR-DC

Document for:		Discussion

- Introduction

In this contribution, we propose the corrections to 37.340 BLCR.

- Discussion

For XR awareness, in case of NR-DC, the PDU Set Information should be forwards from PDCP hosting node to the corresponding node. It is beneficial to make clarifications in stage 2 BL CR that if the data forwarding is performed and the PDCP hosting node receives SDAP SDUs with PDU Set Information Container in the GTP-U extension header on NG-U from the UPF and the corresponding node supports PDU Set based handling, the PDCP hosting node forwards the PDCP PDUs with the corresponding PDU Set Information Container in the GTP-U extension header to the corresponding node NG-RAN node.

-------------------------------------------------------Start of TP for 37.340 BL CR-----------------------------------------------------

## 13.x	eXtended Reality Services

### 13.x.1	Overview

The eXtended Reality Services as described in TS 38.300 [3] is extended to address the NR-DC operation.

### 13.x.2	Awareness

During the SN Addition Preparation procedure and the MN initiated SN modification procedure, the MN may send the PDU Set QoS Parameters as part of the QoS profile to the SN to enable PDU Set based QoS handling.

If the data forwarding is performed and the PDCP hosting node receives SDAP SDUs with PDU Set Information Container in the GTP-U extension header on NG-U from the UPF and the corresponding node supports PDU Set based handling, the PDCP hosting node forwards the PDCP PDUs with the corresponding PDU Set Information Container in the GTP-U extension header to the corresponding node NG-RAN node.

The UE may report uplink assistance information (jitter range, burst arrival time, UL data burst periodicity) per QoS flow via UE Assistance Information to the MN or the SN as configured. If the SN receives the burst arrival time, the SN may use it by considering the SFN offset of the MN.

When the PDCP hosting gNB receives the End of Data Burst Indication of a QoS flow from the UPF, it always provides the End of Data Burst Indication of the QoS flow to the corresponding node when data transmission from the PDCP hosting node to the corresponding node is needed for the QoS flow.

-------------------------------------------------------End of TP for 37.340 BL CR-------------------------------------------------------

For UL PDCP discard, in the stage 3 TP, it has captured that for split bearer the if the PSI based SDU Discard UL IE is also sent from PDCP hosting node to the corresponding node. But there is still FFS on split bearer in Stage 2.

For DL PDCP discard, RAN3 has not identified there is any issues to be solved for DL PDCP discard coordination between MN and SN. Then the FFS on DL PDCP discard coordination should be removed.

-------------------------------------------------------Start of TP for 37.340 BL CR-----------------------------------------------------

### 13.x.3	Discard

For MN terminated SCG bearer/split bearer, the MN notifies SN whether the UL PSI based SDU discarding is (re)configured/released for uplink discarding via XnAP signaling.

For SN terminated MCG bearer/split bearer, the SN notifies MN whether the UL PSI based SDU discarding is (re)configured/released for uplink discarding via XnAP signaling.

-------------------------------------------------------End of TP for 37.340 BL CR-------------------------------------------------------

- Conclusion

In this contribution, we are proposing the corrections to 37.340 BL CR. We propose to agree above TPs.

Reference

[1] R3-243973 (BL CR to 37.340) Support of XR enhancements. ZTE, Nokia, Nokia Shanghai Bell, Qualcomm Inc, Lenovo, Huawei, Ericsson, CATT, CMCC, Samsung, Xiaomi